3GPP TSG-WG SA2 Meeting #144E	S2-2102340

Elbonia, April 12 – 16, 2021

Source:	Qualcomm Incorporated

Title:	Discussion of UE data reporting solution

Document for:	Approval

Agenda Item:	8.1

Work Item / Release:	FS_eNA_Ph2 / Rel-17

Abstract: This contribution proposes the discussion and update of UE data reporting solution.

# 1. Introduction

The Data Collection procedure from UE was approved in last meeting in clause 6.2.8, but there are still some issues need to be solved. This paper focus on the UE IP address mapping and AF selection. This paper also consider how to support the data collection for the group of UE and any UE.

# 2. Discussion

## 2.1	UE IP address mapping

There were two potential directions about UE IP address mapping that were argued in last meeting. The comments that received about the BSF based solution was that it depended on the PCF deployment while PCF was optional in the network, and also the UE IP address and GPSI mapping for AF in untrusted domain depended on SA3 feedback.

The main concern about the UDM/SMF based solution as concluded in the TR is that all the UE IP address that allocated for the UE will exposed to the AF for data collection and it requires the AF for data collection to support the interface to different SMFs.

Although we still think the BSF based solution is simpler than the other one, in order to make progress, the following proposes to focus on the UDM/SMF based solution with some enhancement.

### 2.1.1	How to unique identify the mapping?

Figure 2.1.1-1 is the call flow for AF in trusted domain to collect the mapping from UDM/SMF based on the conclusion in TR.

In step 2, UDM provides all the SMFs related to the SUPI if multiple SMFs that served for the UE. Step 3 and step 4 are performed repeatedly to collect all the UE IP addresses for different PDU sessions. AF compares the received UE IP addresses for the SUPI to the UE IP addresses for the user plane connection and link the data collection request signalling received from NWDAF to the user plane signalling to UE application.

Figure 2.1.1-1	AF requests UE IP address mapping procedure

If the S-NSSAI, DNN of the PDU session that UE establishes the user plane connection to AF can be provided to the UDM, then the UDM can uniquely provide the SMF and SMF can provides the specific single UE IP address to AF. But the PDU session which is selected for the user plane connection between UE and AF maybe different to the PDU session for the UE and ASP, the network does not know which S-NSSAI, DNN is related to the user plane connection to AF, and there is no mechanism to ask UE to identify which PDU session is used for the user plane connection to AF. One possible solution is that AF is configured with the supported S-NSSAI, DNN by SLA, and compares the UE established PDU session that the S-NSSAI and DNN are supported by the AF. AF is only provided with the related UE IP address for the PDU session.

Proposal 1:	AF is configured with the supported S-NSSAI and DNN by SLA, AF is only provided the UE IP address for the PDU session that the S-NSSAI and DNN are supported by the AF.

### 2.1.2	Which entity is responsible for collecting mapping information?

According to the TR conclusion as described in figure 2.1.1-1, it requires AF to contact UDM and further contact the SMFs. It requires the AF to support the interfaces to multiple SMFs. If the analytics subscription is for the group of UEs or any UE, the AF has to collect the mapping for multiple UEs which is also a big burden to AF. Consider that the NWDAF has already support for  all the interfaces to UDM and SMF, it is more appropriate for the NWDAF to collect the mapping information.

Figure 2.1.2-1 is the call flow about NWDAF collects the mapping information between UE IP address and SUPI.

Figure 2.1.2-1 UE IP address mapping for AF in trusted domain

When the AF receives the Naf_Event_Exposure_Subscribe from NWDAF with the request to collect data from UE, AF will check if there is a corresponding UE IP address that identifies the user plane connection. If there is a mapping relationship stored in AF, AF can collect data from UE directly.

If AF can not find the related user plane connection to the SUPI, the AF will further request NWDAF to provide the mapping information. NWDAF is enhanced to support a new Nnwdaf_EventExposure service with the event ID as UE IP address collection, AF includes the SUPI, supported S-NSSAI and DNN information to the NWDAF in step 2. The AF may provide multiple pairs of (S-NSSAI, DNN) if the AF supports multiple pairs. NWDAF collects the served SMFs and the corresponding S-NSSAI and DNN for the SUPI from UDM. NWDAF will determine the SMF that the corresponding S-NSSAI and DNN is supported by the AF. NWDAF collects the UE IP address from SMF that serves the PDU session with the specific S-NSSAI and DNN. NWDAF provides the UE IP address to AF in step 7 and AF stores the UE IP address allocated to SUPI.

Proposal 2:	AF is configured with the supported S-NSSAI and DNN information. AF requests the supported S-NSSA and DNN to NWDAF and asks NWDAF to provide the corresponding UE IP address.

### 2.1.3	Target of analytics reporting for group of UE and any UE

The target of analytics reporting sent from the consumer can be group of UE or any UE. For group of UEs, UDM collects the list of SUPIs related to the group of UE from UDM. NWDAF sends the Naf_Event_Exposure_Subscribe to AF as described in step 1 in figure 2.1.2-1. The call flow described in Figure 2.1.2-1 is performed for each SUPI in the group of UE.

If the target of analytics reporting is any UE, NWDAF decides the list of SUPIs based on the mechanism in Rel-16, e.g. collects the SUPIs in the area of interest from AMF / SMF, or use sampling ratio. The procedure described in figure 2.3-1 is performed respectively for each UEs that decided as the target of analytics reporting.

Proposal 3:	if the target of analytics reporting is the group of UE or any UE, NWDAF decides the list of SUPI and sends the Naf_Event_Exposure_Subscribe to AF to collect data for each UE.

### 2.1.4	Mapping procedure for AF in untrusted domain

Figure 2.1.4-1 UE IP address mapping procedure for AF in untrusted domain

As the same principle for AF in trusted domain in clause 2.1.2-1, if NWDAF requires to collect data from AF in untrusted domain, NWDAF sends step 1 and 2 to AF. AF will check if there is a corresponding UE IP address that identify the user plane connection. If there is a mapping relationship stores in AF, AF can collect data from UE directly.

If AF can not find the related user plane connection to the SUPI, AF will further request NWDAF to provide the mapping information. An enhancement of the Service for NEF and NWDAF as proposed in step 3 and step 4. AF provides the GPSI with supported S-NSSAI and DNN to the NEF and NEF forwards the information to NWDAF. NWDAF collects the related UE IP address as described in clause 2.1.2. NWDAF provides the UE IP address to AF via NEF in step 6 and step 7.

If the target of analytics reporting is group of UE, NWDAF collects the list of SUPIs related to the group of UE from UDM and initiates the procedure as described in figure 2.1.4-1 for each UE separately. If the target of analytics reporting is for any UE, NWDAF decides the list of SUPIs based on the mechanism in Rel-16, e.g. collects the SUPIs in the area of interest from AMF / SMF, or use sampling ratio. The procedure described in figure 2.1.4-1 are performed respectively for each UEs that decided as the target of analytics reporting.

## 2.2	AF registration and discovery

According to the description in clause 6.2.8.2.2 in TS 23.288, it is clear that AF registers the application ID to NRF, but if there are other NF profiles for AF registered to NRF is not clear. The NWDAF discovers the AF by using the application ID (or any relevant Event ID) from NRF, but AF does not know the Event ID and can not register the supported Event ID to NRF, NWDAF can only use application ID to select the AF for data collection.

If we consdier the consumer that sends the analytics subscribes to NWDAF, only the AF, PCF can provide an application ID to NWDAF in the analytics subscribe message. If other 5GC NF, for example, AMF, SMF and OAM requests analytics which requires to collect data from UE, NWDAF can not discover the specific AF because the AMF can not provide the application ID. There is a use case that the UE attribute is required for AMF load analytics that we approved in SA2#143E meeting and captured in clause 6.5.2 in TS 23.288, but it is not described where the NWDAF gets the application ID and it can not make sure NWDAF can always get the application ID from the consumer. Considering the possible future enhancement to support more use cases to collect data from UE, it is more appropriate to consider the AF discovery without application ID.

For each use case that requests to collect data from UE, we should define the UE attribute as the table 6.5.2-4 in TS 23.288 for each analytics ID, it is reasonable to define a UE attribute ID for each UE attribute. AF is configured with the supported UE attribute ID(s), registers the supported attribute ID(s) as AF profile to NRF. If NWDAF receives an analytics subscription from a consumer, NWDAF can discover an AF by using either application ID if application ID provided or UE attribute ID if application ID is not provided.

Proposal 4: UE attribute ID is defined for each UE attribute. AF is configured with the supported UE attribute ID(s) and registers the supported UE attribute ID(s) to NRF as AF profile. NWDAF can discover an AF by using either application ID if application ID provided or UE attribute ID if application ID is not provided.

# 2. Proposal

According to the above discussion and proposals, it is proposed to approve the CR S2-21xxxx.