SA WG2 Meeting #144e	S2-2102751

- April 12 – 16, 2021; Elbonia                   				(revision of S2-2100227)

Title:	draft LS on E-RABs that cannot be handed over to 2G/3G or 5G

Response to:	R3-211273 / S2-2102093

Release:	Rel-16

Work Item:	5GS_Ph1, TEI16

Source:	Will be SA2

To:	RAN3

Cc:	CT4, CT1

Contact Person:

#### Name:	 Laurent Thiebaut

E-mail Address:	 Laurent.thiebaut@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	23.502 CR 2477

1. Overall Description:

SA2 thank RAN3 for their LS that they have discussed.

For 5G SRVCC to 3G it is described in 23.501 § 5.17.2.4 “Mobility between 5GS and GERAN/UTRAN” that “After the 5G SRVCC to UTRAN, all the PDU sessions of the UE are released”;

The same behaviour may take place at the indirect mobility 5G  4G  (SRVCC) 3G

Thus SA2 assumes that in networks where 5GS has been rolled out, , the ENB may be configured to disable PS Hand-over to UTRAN. when a SRVCC procedure to UTRAN is taking place.

For a PS Hand-over to UTRAN (without SRVCC), the MME may issue a Forward Relocation Request message to the SGSN but this Forward Relocation Request does not transfer to the SGSN PDN connections for which no TI has been negotiated with the UE (these PDN connections correspond to PDU Sessions that were established over 5GS). If there would remain no PDP context to hand-over to SGSN in a Forward Relocation Request message, the MME is expected to fail the Hand-over request from the ENB;

The UE is assumed to locally release any PDN connection without a TI negotiated with the network when it moves to GERAN/UTRAN;

SA2 agreed to the corresponding attached CR;

2. Actions:

To RAN3:

ACTION: 	SA2 kindly asks RAN3 to take the above information into account.

3. Date of Next TSG-SA WG2 Meetings:

|  | DATES (2021) | LOCATION | CTRY |
| --- | --- | --- | --- |
| SA2#145e | May17th – 28th | E-meeting | any |