3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102132

# Elbonia, April 12th - 16th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0258 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#8 – Resolving Editor´s note on mapping UE IP address and SUPI or GPSI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The following Editor´s note is included in clause 6.2.8.2.4: Editor's note: The procedure about how to map the UE IP address to a SUPI or GPSI is FFS. This Editor´s note aims to resolve how the AF can correlate the NWDAF request for input data that includes a SUPI or a GPSI and the user plane connection established between the UE application and the AF for data collection. The proposal to resolve is as follows: At the time the user plane connection is established over a PDU session for a SUPI, DNN, S-NSSAI, the AF stores the UE IP address as described clause 6.2.8.2. Later, when the AF receives a request to collect UE data, this trigger both the data collection from the UE and the correlation of the NWDAF request for a SUPI and the user plane connection identified by a UE IP address. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The AF finds the SMF serving the PDU session for the user plane connection to the AF (for direct reporting) then next the AF requests the SMF to provide the allocated IPv4address or IPv6 prefix or both. Then AF can correlate the NWDAF request to collect UE data and the user plane connection that transport this data. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification of the conclusions in KI#8. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 6.2.8.2.4.x (new), 6.2.8.2.4.y (new), 6.2.8.2.4.z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---------------------------------First Change---------------------------------------

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[3]	3GPP TS 23.502: "Procedures for the 5G System; Stage 2".

[4]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System; Stage 2".

[5]	Void.

[6]	3GPP TS 28.532: "Management and orchestration; Generic management services".

[7]	3GPP TS 28.550: "Management and orchestration; Performance Assurance".

[8]	3GPP TS 28.552: "Management and orchestration; 5G performance measurements".

[9]	3GPP TS 28.545: "Management and orchestration; Fault Supervision (FS)".

[10]	3GPP TS 28.554: "Management and orchestration; 5G end to end Key Performance Indicators (KPI)".

[11]	ITU-T Recommendation P.1203.3: "Parametric bitstream-based quality assessment of progressive download and adaptive audiovisual streaming services over reliable transport - Quality integration module".

[12]	3GPP TS 38.215: "NR; Physical layer measurements".

[13]	Void.

[14]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[15]	3GPP TS 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC); Protocol specification".

[16]	3GPP TS 38.413: "NG-RAN; NG Application Protocol (NGAP)".

[17]	3GPP TS 29.244: "Interface between the Control Plane and the User Plane Nodes".

[18]	3GPP TS 29.510: "5G System; Network function repository services; Stage 3".

[19]	3GPP TS 28.533: "Management and orchestration; Architecture framework".

[20]	3GPP TS 37.320: "Radio measurement collection for Minimization of Drive Tests (MDT); Overall description; stage 2".

[21]	3GPP TS 28.201: "Charging management; Network slice performance and analytics charging in the 5G System (5GS); stage 2".

[22]	3GPP TS 28.541: "Management and orchestration; 5G Network Resource Model (NRM); Stage 2 and stage 3".

[23]	3GPP TS 24.501: "Non-Access-Stratum (NAS) protocol for 5G System (5GS); Stage 3".

[24]	3GPP TS 28.310: "Management and orchestration; Energy efficiency of 5G".

[25]	3GPP TS 29.518: "5G System; Access and Mobility Management Services; Stage 3".

[x]	3GPP TS 29.503: " Unified Data Management Services; Stage 3".

-----------------------------Next Change-------------------------------------------

##### 6.2.8.2.4	Correlation between UE data collection and the NWDAF data request

6.2.8.2.4.x	General

The UE IP address is used to identify the user plane connection established between the UE application and the AF for data collection, while the AF receives the Naf_Event_Exposure_Subscribe to request for the specific UE data collection by using SUPI (for AF in trusted domain) or external UE ID (i.e. GPSI) (for AF in untrusted domain).

AF may indicate in NF profile if it supports to do the mapping itself (i.e. AF interacts directly to UDM/SMF) or ask NWDAF to do it (i.e. AF requests NWDAF to interact with UDM/SMF).

Accordingly, if AF supports the mapping, it is required to correlate the UE IP address to the SUPI or to GPSI.

6.2.8.2.4.y	NWDAF data request using SUPI or GPSI (for AF in trusted domain)

If the AF receives the Naf_EventExposure_Subscribe/Request including Target for Event Reporting set to SUPI or GPSI, the AF finds the PDU session(s) serving the SUPI or GPSI, DNN, S-NSSAI from UDM and the allocated IPv4 address or IPv6 prefix or both from SMF as described in Figure 6.2.8.2.4.y-1.

Figure 6.2.8.2.4.y-1.: AF in trusted domain correlates UE data collection and NWDAF request

- At the establishment of the user plane connection between the UE Application and the AF, the AF stores the UE IP address (for both direct and indirect reporting) as described clause 6.2.8.2.1. Also, as part of the Connection establishment between UE Application and AF described in clause 6.2.8.2.1, the ASP provides the DNN, S-NSSAI to the AF (e.g. as part of SLA configuration).

1.	The AF receives a request to retrieve input data as described in clause 6.2.8.2.3 including a SUPI or a GPSI. The AF finds the SMF serving the PDU session(s) for this SUPI or GPSI, DNN, S-NSSAI using Nudm_UECM_Get_Request including SUPI, type of requested information set to SMF Registration Info and the S-NSSAI and DNN, as defined in clause 5.3.2.5.7 in TS 29.503 [x].

2. The UDM provides the SMF id and the PDU Session id using Nudm_UECM_Get_Response to the AF.

3. The AF sends Nsmf_EvenExposure_Subscribe to the SMF identified in step 2, including the Target for Event Reporting set to the PDU Session id(s) provided in step 2 and the Event ID set to IP address/prefix allocation/change.

4.	The SMF provides the allocate IPv4 address or IPv6 prefix or both per PDU Session id to the AF.

5.	The AF correlates the UE data that includes the UE IP address and the NWDAF request for a SUPI using the retrieved IPv4 address or IP v6 prefix.

If the target of analytics reporting is any UE, AF subscribes to NRF Discovery e.g. based on the event filter provided on the area of interest (as list of TAIs) to find matching AMF instances. The SUPIs are retrieved from relevant AMF(s) based on the event filter and then the procedure to interact with SMF as above is followed per individual SUPI or list of SUPIs.

Editor’s Note: For the case of “any UE” support, whether AF or NWDAF interacts with AMF is FFS depending on the analytics types.

6.2.8.2.4.z	NWDAF data request using GPSI (for AF in untrusted domain)

Editor’s Note: The procedure for the AF in untrusted domain is FFS.

-----------------------------End of Changes---------------------------------------