3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102134

# Elbonia, April 12th - 16th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.503 | CR | 0543 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | KI#5 – Total bandwidth per S-NSSAI |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, KPN |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Include conclusions on KI#5 in TR 23.700-40 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Define new policy control data on slice level that includes the Maximum data rate per S-NSSAI that . is provisioned by OAM Define how PCF monitors the aggregate data rate in the slice that includes to aggregate the MBR for each PDU session in that slice. Define how to enforce the Maximum Data rate in the slice, that implies that no new PDU session or QoS flows are established or changes in the existing QoS flows may happen depending on a policy decision in the PCF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete specification of the conclusions in KI#5. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.1.x (new), 6.2.1.y (new), 6.2.1.y.1 (new), 6.2.1.y.2 (new), |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

---------------------------------------First Change------------------------------------------------

#### 6.2.1.x	Slice specific policy control information

The slice specific policy control information used for policy control includes:

- Remaining Data rate per S-NSSAI stored in the UDR as Data Set "Policy Data" and Data Subset “Remaining  Data Rate”. It contains either Remaining Uplink Maximum UL data rate per S-NSSAI or Remaining Maximum DL data rate per S-NSSAI or both,

If the S-NSSAI is subject of slice specific bandwidth restrictions the Maximum Data rate per S-NSSAI and the Remaining Data rate  per S-NSSAI is stored in the UDR.

The monitoring and enforcement of the Maximum Data rate per S-NSSAI by the PCF is defined in clause 6.2.1.y.

---------------------------------------Next Change------------------------------------------------

#### 6.2.1.y	Monitoring the Total Bandwidth per S-NSSAI

##### 6.2.1.y.1	General

The PCF supports monitoring and enforcement of the Total Bandwidth per S-NSSAI

The PCF may receive slice specific policy control information per S-NSSAI from the UDR, i.e. the  remaining data rate per S-NSSAI.

##### 6.2.1.y.2	Monitoring the Total Bandwidth per slice using the MBR per PDU session in that slice

For the purpose of slice specific bandwidth monitoring control, the PCF shall at the establishment of a Session Management Policy association, for a S-NSSAI subject of slice bandwidth control, deduct the value of the Authorized Session-AMBR allocated to the PDU session to the remaining data rate for that S-NSSAI and store it in the UDR.

At the time one or more PCC Rules are installed or activated in the SMF, the PCF requests, for PCC Rules including a GBR value. reporting “Successful resource allocation” with the corresponding Policy Control Request Trigger, and when the SMF reports that the resources are successfully allocated the PCF shall deduct the value of the MBR allocated to each SDF template to the remaining data rate for that S-NSSAI and store it in the UDR.

At the time a change of Authorized Session-AMBR occurs, a PCC Rule is removed or modified, the PCF shall update the remaining data rate for that S-NSSAI in the UDR.

If the remaining data rate for that S-NSSAI reaches a threshold that indicates that is closer to zero, , the PCF may apply a policy decision (e.g. terminate the Session Management Policy association) and interact with the SMF accordingly.

At the time the Session Management Policy Association, for a S-NSSAI subject of slice bandwidth control, is terminated, the PCF shall update the remaining data rate for that S-NSSAI in the UDR.

---------------------------------------End of Changes---------------------------------------------