3GPP TSG-SA2 Meeting # 144E (e-meeting)	S2-2102169

# Elbonia, April 12 – 16, 2021										  				   (revision of S2-2100284)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3622 | rev | 2 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Function Description for Multi-USIM devices |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, LG Electronics, Huawei, China Telecom, Charter, MediaTek, Intel, Vodafone, Spreadtrum, Sony, Apple, Samsung, Nokia, Nokia Shanghai Bell, Vivo. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MUSIM |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introducing overall function description for Multi-USIM device as agreed in TR 23.761 |  |  |  |  |  |  |  |  |
| Summary of change: |  | General description for Multi-USIM device function. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing function description |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 4.3.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR …CR … |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START CHANGE ***************

## 3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

MME Pool Area: An MME Pool Area is defined as an area within which a UE may be served without need to change the serving MME. An MME Pool Area is served by one or more MMEs ("pool of MMEs") in parallel. MME Pool Areas are a collection of complete Tracking Areas. MME Pool Areas may overlap each other.

Serving GW Service Area: A Serving GW Service Area is defined as an area within which a UE may be served without need to change the Serving GW. A Serving GW Service Area is served by one or more Serving GWs in parallel. Serving GW Service Areas are a collection of complete Tracking Areas. Serving GW Service Areas may overlap each other.

PDN Connection: The association between a PDN represented by an APN and a UE, represented by one IPv4 address and/or one IPv6 prefix (for IP PDN Type) or by the UE Identity (for Non-IP and Ethernet PDN Types).

Default Bearer: The EPS bearer which is first established for a new PDN connection and remains established throughout the lifetime of the PDN connection.

Default APN: A Default APN is defined as the APN which is marked as default in the subscription data and used during the Attach procedure and the UE requested PDN connectivity procedure when no APN is provided by the UE.

eCall Only Mode: A UE configuration option that allows the UE to attach at EPS and register in IMS to perform only eCall Over IMS, and an IMS call to a non-emergency MSISDN or URI for test and/or terminal reconfiguration services. For a short period following either such call, an incoming call (e.g. callback from a PSAP or HPLMN operator) or other incoming session (e.g. for USIM reconfiguration) is possible. At other times when the UE is configured in this mode, the UE is required to refrain from any signalling to a network. Use of eCall Only Mode is configured in the USIM for the UE.

PDN Connection to the SCEF: The association between a UE, represented by the UE Identity, and a PDN represented by an APN to external packet data network via SCEF to allow transfer of Non-IP data. It includes establishment and persistence of T6 connection between MME and SCEF (see TS 29.128 [79]).

Emergency attached UE: A UE which only has bearer(s) related to emergency bearer service.

NOTE 1:	The above term is equivalent to the term "attached for emergency bearer services" as specified in TS 24.301 [46].

LIPA PDN connection: a PDN Connection for local access (e.g. for IP or Ethernet access) for a UE connected to a HeNB.

en-gNB: As defined in TS 37.340 [85].

SIPTO at local network PDN connection: a PDN connection for SIPTO at local network for a UE connected to a (H)eNB.

Correlation ID: For a LIPA PDN connection, Correlation ID is a parameter that enables direct user plane path between the HeNB and L-GW.

SIPTO Correlation ID: For a SIPTO at local network PDN connection, SIPTO Correlation ID is a parameter that enables direct user plane path between the (H)eNB and L-GW when they are collocated.

Local Home Network: A set of (H)eNBs and L-GWs in the standalone GW architecture, where the (H)eNBs have IP connectivity for SIPTO at the Local Network via all the L-GWs.

Local Home Network ID: An identifier that uniquely identifies a Local Home Network within a PLMN.

Presence Reporting Area: An area defined within 3GPP Packet Domain for the purposes of reporting of UE presence within that area due to policy control and/or charging reasons. In the case of E-UTRAN, a Presence Reporting Area may consist in a set of neighbor or non-neighbor Tracking Areas, or eNodeBs and/or cells. There are two types of Presence Reporting Areas: "UE-dedicated Presence Reporting Areas" and "Core Network pre-configured Presence Reporting Areas" that apply to an MME pool.

RAN user plane congestion: RAN user plane congestion occurs when the demand for RAN resources exceeds the available RAN capacity to deliver the user data for a prolonged period of time.

NOTE 2:	Short-duration traffic bursts is a normal condition at any traffic load level, and is not considered to be RAN user plane congestion. Likewise, a high-level of utilization of RAN resources (based on operator configuration) is considered a normal mode of operation and might not be RAN user plane congestion.

IOPS-capable eNodeB: an eNodeB that has the capability of IOPS mode operation, which provides local connectivity (e.g. for IP or Ethernet) and public safety services to IOPS-enabled UEs via a Local EPC when the eNodeB has lost backhaul to the Macro EPC or it has no backhaul to the Macro EPC.

IOPS network: an IOPS network consists of one or more eNodeBs operating in IOPS mode and connected to a Local EPC.

Local EPC: a Local EPC is an entity which provides functionality that eNodeBs in IOPS mode of operation use, instead of the Macro EPC, in order to support public safety services.

Macro EPC: the EPC which serves an eNodeB when it is not in IOPS mode of operation.

Nomadic EPS: a deployable system which has the capability to provide radio access (via deployable IOPS-capable eNodeB(s)), local connectivity (e.g. for IP or Ethernet) and public safety services to IOPS-enabled UEs in the absence of normal EPS

Multi-USIM UE: a UE with multiple USIMs, capable of maintaining a separate registration state with a PLMN for each USIM at least over 3GPP Access and supporting one or more of the enhancements described in clause 4.3.x.

IOPS-enabled UE: is an UE that is configured to use networks operating in IOPS mode.

Cellular IoT: Cellular network supporting low complexity and low throughput devices for a network of Things. Cellular IoT supports IP, Ethernet and Non-IP traffic.

Narrowband-IoT: a 3GPP Radio Access Technology that forms part of Cellular IoT. It allows access to network services via E-UTRA with a channel bandwidth limited to 180 kHz (corresponding to one PRB). Unless otherwise indicated in a clause, Narrowband-IoT is a subset of E-UTRAN.

LTE-M: a 3GPP RAT type Identifier used in the Core Network only, which is a sub-type E-UTRAN RAT type, and defined to identify in the Core Network the E-UTRAN when used by a UE indicating Category M in its UE radio capability.

WB-E-UTRAN: in the RAN, WB-E-UTRAN is the part of E-UTRAN that excludes NB-IoT. In the Core Network, the WB-E-UTRAN also excludes LTE-M.

DCN-ID: DCN identity identifies a specific dedicated core network (DCN).

For the purposes of the present document, the following terms and definitions given in TS 23.167 [81] apply:

eCall Over IMS: See TS 23.167 [81].

RLOS attached UE: A UE is attached only for accessing Restricted Local Operator Services (see TS 23.221 [27]).

IAB-donor: For the purposes of this specification, this is a NR Secondary RAN node is further described in TS 37.340 [85] that supports Integrated access and backhaul (IAB) feature and provides connection to the core network to IAB-nodes. It supports the CU function of the CU/DU architecture for IAB defined in TS 38.401 [90].

IAB-node: A relay node that supports wireless in-band and out-of-band relaying of NR access traffic via NR Uu backhaul links. It supports the UE function and the DU function of the CU/DU architecture for IAB defined in TS 38.401 [90].

****************** NEXT CHANGES ***************

### 4.3.x	Support for Multi-USIM UE

#### 4.3.x.1	General

A network and a UE may support one or more of the following enhancements for Multi-USIM UE operation:

-	Connection release, as described in clause 4.3.x.2

-	Paging cause indication for voice service, as described in clause 4.3.x.3

-	Reject paging request, as described in clause 4.3.x.4

-	Paging collision control, as described in clause 4.3.x.5.

-	Paging Restriction, as described in clause 4.3.x.6

In the Attach procedure (as specified in clause 5.3.2.1), or in the Tracking Area Update procedure (as specified clause 5.3.3), a Multi-USIM UE may include the Multi-USIM Mode Indication to the MME if it has more than one USIM active and intends to use Multi-USIM specific features. Based on the received Multi-USIM Mode Indication from the UE, the MME shall indicate to the UE the Multi-USIM features supported by network.

#### 4.3.x.2	Connection release

A Multi-USIM UE may request to be released to CM-IDLE state for a USIM due to activity on another USIM.

The UE indicates that it requests to be released to CM-IDLE state for the USIM by initiating the Service Request procedure (using Extended Service Request message) or a Tracking Area Update procedure if the UE needs to perform Track Area Update at the same time with this network, including a Release Indication.  If supported by the UE and network, the UE may also provide, together with the Release Indication, a Paging Restriction Information, as specified in clause 4.3.x.6, which requests the network to restrict paging. The Paging Restriction Information from the UE is stored in the UE context in the MME. If no Paging Restriction Information is provided in the Extended Service Request or the Tracking Area Update Request, any stored Paging Restriction Information in the UE context is removed.

When the UE initiates for the USIM a Service Request procedure or Tracking Area Update procedure without providing a Release Indication, the network removes any stored Paging Restriction Information.

#### 4.3.x.3	Paging cause indication for voice service

In additional to the CN Domain indication in the S1AP Paging message, the MME may provide an andditional indication of voice service in the S1AP Paging message for the MMTel voice service,  if  UE Multi-USIM Mode Indication is provided to the MME. The MME determines whether the paging related to a voice service based on the Paging Policy Indicator as specified in clause 4.9 for MMTel voice.

Upon reception of the voice service indication in S1AP Paging Message, the eNodeB supporting paging cause indication should include the voice service indication as the paging message to the UE.

#### 4.3.x.4	Rejection of paging request

Based on activity ongoing on another USIM. a Multi-USIM UE may respond to a page for a USIM with an indication to  the MME that the UE does not accept the paging and requests to be released to CM-IDLE state,

The Multi-USIM UE attempts to send a Service Request message to the paging network including the above indication Reject Paging Indication, the UE may include Paging Restriction Information as specified in 4.3.x.6 in the Service Request, if supported by UE and network.

#### 4.3.x.5	Paging collision control

To avoid possible paging occasion collision and to guarantee the paging reception for differnt USIMs, the UE may request to  provide, for at least one USIM,  an additional offset value that is used for determination of paging occasions . This is achieved by the UE and network negotiating an offset value during the Attach procedure or Tracking Area Update procedure.  Upon reception of an offset value from UE in Attach Request or Tracking Area Update Request, a supporting MME provides an accepted offset value to the UE in the Attach Accept or Tracking Area Update Accept message to acknowledge the use of offset value. The accepted offset value is stored in the UE context in the MME, which may differ from the one requested by the UE.  If the UE does not provide any offset value in Attach Request or Tracking Area Request, the MME removes any stored offset value in the UE context. The MME provides the additional offset to RAN in S1 AP message to derive the occasion for paging.

NOTE:	 The accepted offset is used to derive the paging occasions as specified in TS 36.304 [34].

#### 4.3.x.6	Paging Restriction

The UE and the network may support Paging Restriction. The UE, if the MME indicates that the network supports Paging Restriction, may indicate Paging Restriction Information in the Service Request, or the Extended Service Request or the Tracking Area Update Request as specified in clause 4.3.x.2 and 4.3.x.4. The Paging Restriction Information may indicate any of the following:

a) 	all paging is restricted, or

b)	all paging is restricted, except paging for voice service (MMTel voice or CS domain voice), or

c)	all paging is restricted, except for certain PDN Connection(s), or

d)	all paging is restricted, except for certain PDN Connection(s) and voice service (MMTel voice or CS domain voice).

NOTE:	The UE expects not to be paged for any purpose in case a). The UE expects to be paged only for voice service in case b). The UE expects to be paged only for certain PDN Connection(s) in case c). The UE expects be paged for voice service and certain PDN Connection(s) in case d).

****************** END CHANGES ***************