3GPP TSG-SA WG2 Meeting #144E	S2-2102179r02

# e-meeting, April 12-16, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0259 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NWDAF registering into UDM |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In a multiple NWDAF deployement (incuding selection and re-selection of NWDAFs with overlapping serving areas), make it possible for collecting UE related data from NWDAFs, ensure the same NWDAF is used for multiple NWDAF consumers and for an NWDAF consumer to be able to request statistics (Analytics in the past). |  |  |  |  |  |  |  |  |
| Summary of change: |  | When an NWDAF starts serving a UE, NWDAF registers into UDM the UE it serves. When NWDAF purges the UE analytics context, it deregisters from UDM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In a multi NWDAF deployment, not possible to discover NWDAF that is serving a UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.X (new), 6.1C (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 CR 2604 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * * *

## 5.X	NWDAF Registration in UDM

In order to make NWDAF discoverable in some network deployments, NWDAF may be configured to register in UDM for the UEs it is serving. Registration in UDM should take place at the time the NWDAF starts serving the UE or collecting data for the UE. Deregistration in UDM takes place when NWDAF deletes the analytics UE context for the UE.

* * * Next Change * * * *

## 6.1C	NWDAF Registration/Deregistration in UDM

### 6.1C.1	General

The procedures in this clause are applicable to UE-related analytics (e.g., UE mobility analytics), in deployments, e.g. such with a co-located NWDAF, where the NWDAF is configured to register in UDM for the UEs that it is serving or collecting data for. This enables NWDAF service consumers to discover the NWDAF that is already serving the UE.

### 6.1C.2	NWDAF Registration in UDM

Figure 6.1C.2-1 shows the procedures for registration of the NWDAF in UDM for UE-related analytics.

Figure 6.1C.2-1: NWDAF registration in UDM

1.	NWDAF triggers a registration in UDM. This trigger may be, e.g., the reception of a new Analytics subscription request, an internal trigger  such as a local configuration in the NWDAF, or an OAM configuration action, or initiated by the the reception of data from producer.

2.	The NWDAF registers into UDM for the served UE, by sending Nudm_UECM_Registration request (UE ID, NWDAF ID, Analytics ID).

3.	UDM sends a response to NWDAF.

### 6.1C.3	NWDAF De-registration from UDM

Figure 6.1C.3-1 shows the procedures for deregistration of the NWDAF in UDM.

Figure 6.1C.3-1: NWDAF de-registration from UDM

1.	NWDAF triggers a deregistration from a previous registration in UDM. This trigger may be that, e.g., the NWDAF has purged the Analytics context for the UE, the NWDAF is no longer collecting data for the UE, or an administrative action.

2.	NWDAF sends an Nudm_UECM_Deregistration request (UE ID, NWDAF ID, Analytics ID).

3. 	UDM sends a response to NWDAF.

* * * End of Changes * * * *