3GPP TSG-SA WG2 Meeting #143E	S2-2102184

# e-meeting, April 12 – 16, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0260 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Different deployment options for NWDAFs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is a need to describe the different Analytics Deployment options. |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Introduced a clause 4.x on Analytics deployment options -Moved text to 4.X Different deployment options for Analytics from clause 5.1 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No examples of deployments are provided. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.X (new), 5.1 and Annex A (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * * *

## 4.X	Different deployment options

The NWDAF architecture enables different deployment options.

Multiple NWDAF instances are allowed to be arranged in a hierarchy/tree with a flexible number of layers/branches. The number and organisation of the hierarchy layers, as well as the capabilities of each NWDAF instance remain deployment choices.

In a hierarchical deployment, NWDAFs may provide data collection exposure capability for generating analytics based on the data collected by other NWDAFs, when DCCF, MFAF are not present in the network.

Deployment options for Analytics are available in the informative Annex A.

* * * Next Change * * * *

# 5	Network Data Analytics Functional Description

## 5.1	General

The NWDAF provides analytics to 5GC NFs, and OAM as defined in clause 7. An NWDAF can be decomposed into:

-	Analytics logical function (AnLF): An NWDAF containing the Analytics logical function, denoted as NWDAF(AnLF), can perform inference, derive analytics information (i.e. derives statistics and/or predictions based on Analytics Consumer request) and expose analytics service i.e. Nnwdaf_AnalyticsSubscription or Nnwdaf_AnalyticsInfo.

-	Model Training logical function (MTLF): An NWDAF containing the Model Training logical function, denoted as NWDAF(MTLF), trains ML models and exposes new training services (e.g. providing trained model) as defined in clause 7.

NOTE 1:	NWDAF can contain Model Training logical function, Analytics logical function, or both. An NWDAF contains both logical functions and is denoted as NWDAF, unless explicitly mentioned as NWDAF(AnLF) and NWDAF(MTLF).

NOTE 2:	Pre-trained ML model storage and provisioning to NWDAF is out of the scope of 3GPP.

NOTE 3:	Sharing of models or model meta data is limited to single vendor environments in this Release of the specification.

Analytics information are either statistical information of the past events, or predictive information.

Different NWDAF instances may be present in the 5GC, with possible specializations per type of analytics. The capabilities of a NWDAF instance are described in the NWDAF profile stored in the NRF.

In order to support NFs to discover and select an NWDAF instance containing MLTF, AnLF, or both, that is able to provide the required service (e.g. analytics exposure or ML model provisioning) for the required type of analytics, each NWDAF instance should provide the list of supported Analytics ID(s) (possibly per supported service) when registering to the NRF, in addition to other NRF registration elements of the NF profile. NFs requiring the discovery of an NWDAF instance that provides support for some specific service(s) for a specific type of analytics may query the NRF for NWDAFs supporting the required service(s) and the required Analytics ID(s).

The consumers i.e. 5GC NFs and OAM decide how to use the data analytics provided by NWDAF.

The interactions between 5GC NF(s) and the NWDAF take place within a PLMN.

The NWDAF has no knowledge about NF application logic. The NWDAF may use subscription data but only for statistical purpose.

* * * Next Change * * * *

Annex A (informative):
Considerations for NWDAF in different deployment options

# A.1	General

This is an informative Annex and describes some of the feasible NWDAF architecture deployments options.

## A.2	Different deployment options

NWDAF architecture enables different deployment options.

The following deployment options are feasible.

1)	A flat deployment. It is characterized for one or multiple NWDAFs that can supply analytics for one or more analytic IDs, for UEs located in the serving area of the NWDAF, and in a given S-NSSAI. Any NWDAF can get data from any data source deployed in the same S-NSSAI and serving area of the NWDAF. NWDAFs do not cooperate among them, nor they are synchronized in data collection or analytic reports. A characteristic of this type of deployment is the lack of coordination among NWDAFs.

Figure A.2-1: Flat deployment

2)	A mixed deployment, with two types of NWDAFs:

-	distributed NWDAFs, that are deployed close to the data sources, e.g., in the same data center as data sources. The distributed NWDAFs are restricted in the sense that they collect data from a small set of data sources and produce analytics for a subset of UEs. A distributed NWDAF may be co-located to an NF.

-	central NWDAFs, that might cover large serving areas within a network slice, even the entire PLMN. They collect data from a wider set of data sources or NWDAFs deployed in the network slice and serving area and produce analytics. Central NWDAFs may be able to retrieve analytics delivered by distributed NWDAFs and may have aggregation capabilities.

A mixed deployment may be designed with layers of NWDAFs in a hierarchy.

Figure A.2-2: Mixed deployment

3)	A strict hierarchical deployment, composed of NWDAFs, which can be deployed in layers. This deployment option is characterized for each data source NF sending data to a designed associated lower-layer NWDAF. Higher-layer NWDAFs do not collect data directly from the data sources. Instead, higher-layer NWDAFs collect data or retrieve analytic reports from the lower-layer NWDAFs. Analytic consumers communicate with NWDAFs in any layer. These deployments may make use of DCCF functionality hosted by NWDAFs.

Figure A.2-3: Strict Hierarchical deployment

Any of the previously mentioned deployment options may also include standalone ADRF and DCCF.

Deployments with DCCF, resemble a mixed deployment, but with a standalone associated DCCF performing the coordination of data and Analytics requests. 

There is also a deployment option where DCCF performs both coordination and data retrieval. This is a replica of a hierarchical deployment, but without the possibility to perform Analytics of the retrieved data at the aggregation point (DCCF).

-

* * * End of Changes * * * *