3GPP TSG-SA WG2 Meeting #143E	S2-2102185r1

# e-meeting, April 12 – 16, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0261 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | NWDAF Selection and re-selection for Multiple NWDAF deployments - procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Re-selection procedure has been introduced in last meeting SA#143. This contribution introduces Selection procedure. Also updates to the re-selection procedure is done to encompass preparation phase. It is also included how the two procedures can live together in a network. |  |  |  |  |  |  |  |  |
| Summary of change: |  | -Introducing the selection procedure including a preparation phase -Resolving the EN whether to combine the selection and re-selection procedure or not, by introducing Selection procedure and making clear the difference between the two procedures. Solving following ENs Editor's note: It is preferably to have one procedure to cover the re-selection of NWDAF covering both when consumer change and when not. But it is FFS how. Editor's note: It FFS how this aligns with the procedure when Analytics consumer changes -Adding the use case to the re-selection procedure to also encompass re-selection in conjunction with discovery and selecton of NWDAF (was earlier part of clause 5.2 NWDAF discover and selection. -Putting the prepared Analytics Subscription procedure into separate clause |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | -Procedure for selection is not available. -Procedure for re-selection is inconsistent with selection procedure. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1B, 6.1B.1, 6.1B.2 (new), 6.1B.3, 6.1B.4, 6.1B.5, 6.1B.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * Start of Changes * * * *

## 6.1B	Procedures for NWDAF Selection and Re-selection

### 6.1B.1	General

In multiple NWDAFs deployment scenario, selection of NWDAF is performed by the Analytics consumer. The selection procedure may or may not include a preparation phase, depending on whether, e.g., the UE has been served before and, therefore, an Analytics Context may exist in the network. Selection of NWDAF may be done as part of a change of Analytics consumer e.g at UE mobility events. Analytics Context may be interchanged between NWDAFs to support the target NWDAF.

It is possible that, within the time period of data collection and/or analytics computation, an NWDAF instance is required to transfer one or more of its analytics subscriptions to another NWDAF instance due to internal (e.g. load balancing, graceful shutdown) external triggers (e.g. UE mobility). For external triggers, NWDAF subscribes with NF(s) to be notified about corresponding events. As for UE mobility, upon on the UE location change event notified by AMF subscribed by NWDAF, NWDAF determines whether it can continue to provide the analytics service, or should trigger reselection of NWDAF to make analytics subscription transfer, when the serving area of the target NWDAF is within the serving area of the analytics consumer NF.

Procedures for analytics subscription transfer allow one NWDAF instance to transfer or handover its ongoing analytics subscriptions to another NWDAF instance. The transfer can be done for all subscriptions or just a selected subset of subscriptions related to specific area(s), specific Analytics ID(s), and/or specific UE(s).

The analytics subscription transfer can also be prepared if the source NWDAF instance anticipates that it will soon not be able to continue its current analytics tasks.

### 6.1B.2 NWDAF selection

The procedure in figure 6.1B.2-1 is used to perform NWDAF selection. The procedure is used when an Analytics Consumer NF discovers and selects an NWDAF. An Analytics consumer NF or another NWDAF can use this selecton procedure for retrieving an existing analytics context. In case of a change in Analytics Consumer, the procedure also includes the ID of the source Analytics Consumer sending analytics information to the target Analytics Consumer. The procedure also includes a target NWDAF optionally retrieving the analytics context from another NWDAF.

Figure 6.1B.2-1: NWDAF selection and collection of Analytics Context

1.	[Optional] Source Analytics Consumer subscribes to Analytics from source NWDAF. Optionally the source NWDAF subscribes to UE mobility events.

2.	[Optional] Source NWDAF triggers to prepare an analytics subscription transfer to Higher Layer NWDAF to store Analytics Context, as specified in the procedure illustrated in figure 6.1B.4-1.

NOTE 2:	The Source NWDAF may decide to store Analytics context in ADRF (if deployed) instead of in a higher layer NWDAF.

NOTE 3: The triggers to store Analytics Context may be internal such as the source NWDAF is running out of available resources, it is configured to repetadly store or that an OAM action is started such as load sharing or graceful shutdown is imminent.

3.	[Optional] Higher Layer NWDAF starts collecting data for the specific UE

4.	[Optional] Source NWDAF triggers, e.g. caused by a UE mobility event notification, to prepare an analytics subscription transfer to associated NWDAF(s), as specified in the procedure illustrated in figure 6.1B.4-1.

5.	[Optional] If NF type is AMF and handover between AMFs is being performed, Source AMF may send information on the Source NWDAF to target AMF according to TS 23.502 [3] clause 4.2.2.2.

6.	The target NF starts serving the UE.

7.	The target NF discovers and selects the target NWDAF.

8.	The target NF sends Nnwdaf_AnalyticsSubscription_Subscribe to the target NWDAF. If available, this message may contain information on the source NWDAF. 

If optional step 5 was performed, target NF should send this message immediately after step 7 , to allow the target NWDAF to trigger step 11 to retrieve UE related Analytics Context, immediately from the source NWDAF.

9.	[Conditional] If step 5 was performed, the target AMF acknowledges to the source AMF, according to TS 23.502 [3], clause 4.2.2.2.

10.	The target NWDAF discovers if and where Analytic Context resides.

11.	[Optional] The target NWDAF retrieves Analytics Context from the source NWDAF.

12.	[Optional] If not yet done, the source NF unsubscribes from the source NWDAF.

13.	[Optional] If not yet done Source NWDAF unsubscribes from the data sources.

14.	The target NWDAF subscribes to the data sources.

15.	The target NWDAF receives data and derives Analytics resports.

16.	The target NWDAF sends notifications to the target Analytics Consumer.

NOTE 4:	If DCCF is deployed messages between Analytics Consumers and NWDAF as well as messages between NWDAFs or ADRF may be sent via DCCF.

### 6.1B.3	NWDAF re-selection with Analytics Subscription Transfer

The procedure in Figure 6.1B.2-1 is used by an NWDAF instance to request the transfer of analytics subscription(s) to another NWDAF instance, using the Nnwdaf_AnalyticsSubscription_Transfer service operation defined in clause 7.2.5. In this procedure, the analytics consumer can also change, thereby triggering the NWDAF to check if analytics subscriptions from that analytics consumer should be transferred to another NWDAF; if yes, the NWDAF will initiate the analytics subscription transfer for those subscriptions.

NOTE 1:	This procedure is relevant for UE related data, e.g. when the Analytics consumer is not associated to any service area (e.g. OAM) or when the analytics consumer serves a larger serving area than the NWDAF.

Editor's note:	It is FFS whether this procedure is useful for any other analytics than UE related.

NOTE 2:	Handling of overload situation or preparation for a graceful shutdown are preferably executed inside an NWDAF Set, when available, therefore, not requiring an analytics subscription transfer as described in this clause. The procedure in Figure 6.1B.3-1 is applicable for analytics subscription transfer across NF Sets (when available) or if the NWDAFs are not deployed in a Set.

Figure 6.1B.3-1: NWDAF re-selection with Analytics subscription transfer

0.	Source analytics consumer subscribes to Analytics from source NWDAF. Optionally the source NWDAF subscribes to UE mobility events.

1a. [Optional] Source NWDAF triggers to prepare an analytics subscription transfer to Higher Layer NWDAF to store Analytics Context, as specified in the procedure illustrated in figure 6.1B.4-1.

NOTE 4:	The Source NWDAF may decide to store Analytics context in ADRF (if deployed) instead of in a higher layer NWDAF.

1b.	[Optional] Higher Layer NWDAF starts collecting data for the specific UE

1c.	[Optional] Source NWDAF triggers, e.g. caused by a UE mobility event notification, to prepare an analytics subscription transfer to target NWDAF(s), as specified in the procedure illustrated in Figure 6.1B.4-1.

2a.	Source NWDAF determines, e.g. based on the UE location information received, the analytics subscription(s) to be transferred to a target NWDAF. NWDAF discovery may be skipped if the target NWDAF had already been discovered as part of a prepared analytics subscription transfer in step 4. NWDAF discovery and selection is specified in clause 6.3.13, TS 23.501 [2]

Step 2 – 3 shall not to be performed when Analytics consumer is changed. A change of consumer may be performed because of a mobility event. If the consumer is serving a specific service area, the Source NWDAF need to know the serving area of the Analytics consumer, which may be done by querying NRF.

2b.	Source NWDAF performs an NWDAF discovery and selects the target NWDAF.

3a.	Source NWDAF requests, using Nnwdaf_AnalyticsSubscription_Transfer Request service operation, a transfer of the analytics subscription(s) determined in step 2 to the target NWDAF. The request contains a callback URI of the analytics consumer. The request also contains Id(s) of active data sources related to the analytics subscriptions requested to be transferred, if not already provided as part of the prepared analytics subscription transfer in the preparation procedure (see step 1). The request message may also include "analytics context identifier(s)" indicating the availability of analytics context for particular Analytic ID(s).

If no information is to be sent (e.g. source NWDAF has not yet served the consumer), the Source NWDAF may redirect the request or subscription or perform an Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_analyticsInfo_Request service operation on behalf of the consumer.

NOTE 5:	If multiple notifications/requests are received in steps 3 and 7, indicating the necessity to transfer analytics subscriptions, the source NWDAF will only trigger a single analytics subscription transfer and ignore additional notifications/requests that would result in the same analytics subscription transfer.

3b.	Target NWDAF accepts the analytics subscription transfer and takes over the analytics generation based on the information received from the source NWDAF.

NOTE 6:	If not yet done during a prepared analytics subscription transfer, the target NWDAF allocates a new subscription Id to the received analytics subscriptions, following the behaviour of AmfEventSubscription as specified in TS 29.518 [25] clause 6.2.6.2.2 and UEContextTransfer as specified in TS 29.518 [25] clause 5.2.2.2.1.1.

NOTE 7:	The target NWDAF might already have received information on some/all of the analytics subscriptions as part of the prepared analytics subscription transfer request received in step 1 and, thus, might already have started to prepare for the analytics generation, e.g. by having already subscribed to relevant event notifications.

3c.	Target NWDAF informs the target analytics consumer about the successful analytics subscription transfer using a Nnwdaf_AnalyticsSubscription_Notify message and providing the new subscription Id, that had been assigned by the target NWDAF, in the Change Subscription Notification Correlation Id parameter of this message as specified in clause 7.2.4.

NOTE 8:	The existing Analytics context in the source NWDAF is not deleted directly but will be purged first when it was collected by the target NWDAF.

4.	[Conditional] If "analytics context identifier(s)" had been included in the Nnwdaf_AnalyticsSubscription_Transfer Request received in step 3a, the target NWDAF, based on the provided "analytics context identifier(s)", may request the "analytics context". The analytics information transfer procedure is specified in clause 6.1B.3.

Editor's note:	It is FFS, whether procedure in clause 6.1B.3 is specified, or other new/existing service operations are used instead.

3d.	[Optional] Target NWDAF subscribes to relevant data source(s), if it is not yet subscribed to the data source(s) for the data required for the Analytics.

3e.	Target NWDAF confirms the analytics subscription transfer to the source NWDAF.

3f.	[Optional] Source NWDAF unsubscribes with the data source(s) that are no longer needed for the remaining analytics subscriptions.

5a,b.	Target NWDAF at some point derives new output analytics based on new input data and notifies the analytics consumer about the new analytics using a Nnwdaf_AnalyticsSubscription_Notify message as specified in clause 6.1.1.

NOTE 9:	Notification correlation information allows the analytics consumer to correlate the notifications to the initial subscription request made with the source NWDAF in step 0.

NOTE 10:	If the NWDAF having the data is a Higher Layer NWDAF the Target NWDAF must not request the “transfer” immediately, but rather only when it is needed. If the NWDAF having the data is a distributed NWDAF (e.g. same as the one requesting the transfer) it is more urgent to retrieve the data immediately before the data is purged in the Source NWDAF.

### 6.1B.4 Prepared Analytics subscription transfer

The procedure in Figure 6.1B.4-1 is used by an NWDAF instance to request another NWDAF instance for a prepared analytics subscription(s) transfer from the source NWDAF instance, using the Nnwdaf_AnalyticsSubscription_Transfer service operation defined in clause 7.2.5.

NOTE 1:	The source NWDAF might determine that it needs to prepare to transfer analytics subscriptions to another NWDAF instance, as the source NWDAF estimates that the UE might enter an area which is not covered by the source NWDAF, e.g. by subscribing to AMF event exposure service for UE mobility event notifications, by performing UE mobility analytics, or by subscribing to another NWDAF providing UE mobility analytics.

NOTE 2	Another example for source NWDAF to determine to prepare for an analytics transfer is that the source NWDAF wants to resolve an overload situation or prepare for a graceful shutdown.

NOTE 3:	Handling of overload situation or preparation for a graceful shutdown are preferably executed inside an NWDAF Set, when available.

NOTE 4:	For simplification, this procedure visualizes when the analytics consumer does not change.

Figure 6.1B.4-1: Prepared analytics subscription transfer

The procedure of analytics subscription transfer comprises the following steps:

1a.	Analytics consumer subscribes to the source NWDAF for certain analytics as specified in clause 6.1.1.

1b.	[Optional] Source NWDAF starts data collection from relevant data source(s) (e.g. NFs or OAM) as specified in clause 6.2. Source NWDAF starts generating requested analytics.

2.	[Conditional] (Only if the source NWDAF does not serve the whole PLMN and the requested analytics involves UE related data) The source NWDAF subscribes, using Namf_EventExposure_Subscribe and/or Nsmf_EventExposure_Subscribe service operation, to receive notifications on UE mobility events AMF and/or SMF, respectively.

If the source NWDAF determines that it needs to prepare to transfer analytics subscriptions to another NWDAF instance, the following steps 3 to 8 shall be executed:

3.	The source NWDAF discovers candidate target NWDAF instances (e.g. NWDAFy and NWDAFz) supporting the requested analytics information for the predicted target area(s). NWDAF discovery and selection is specified in clause 6.3.13, TS 23.501 [2].

NOTE 5:	In this procedure, NWDAFy and NWDAFz are examples for target NWDAF instances that are candidates to take over those analytic subscriptions.

4-5.	In the case of a prepared analytics subscription transfer, the source NWDAF requests, using Nnwdaf_AnalyticsSubscription_Transfer Request, to the candidate target NWDAFs (e.g. NWDAFy and NWDAFz) to prepare for an analytics subscription transfer by including a "prepared analytics subscription transfer indication" in the request message. The request message may include information on the analytics subscriptions to be transferred. The request message may include "analytics context identifier(s)" indicating the availability of analytics context for particular Analytics ID(s).

The candidate target NWDAFs (e.g. NWDAFy and NWDAFz) respond to the request from the source NWDAF using a Nnwdaf_AnalyticsSubscription_Transfer Response message.

6-7.	[Conditional] If "analytics context identifier(s)" had been included in the Nnwdaf_AnalyticsSubscription_Transfer Request received in step 4, the determined target NWDAFs (e.g. NWDAFy and NWDAFz), based on the provided "analytics context identifier(s)", may request the "analytics context" from the source NWDAF. The analytics information transfer procedure is specified in clause 6.1B.3.

Editor's note:	It is FFS, whether procedure in clause 6.1B.3 is specified, or other new/existing service operations are used instead.

NOTE 6:	The target NWDAFs (e.g. NWDAFy and NWDAFz) allocate a new subscription Id to the received analytics subscriptions, following the behaviour of AmfEventSubscription as specified in clause 6.2.6.2.2, TS 29.518 [25] and UEContextTransfer as specified in clause 5.2.2.2.1.1, TS 29.518 [25].

8.	[Optional] Based on the information received from the source NWDAF, the target NWDAFs (e.g. NWDAFy and NWDAFz) start data collection from NFs or OAM (as specified in clause 6.2) and analytics generation for the indicated analytics subscriptions.

### 6.1B.5	Analytics Context Information Transfer

The procedure depicted in Figure 6.1B.3-1 is used by an NWDAF instance to request analytics context information from another NWDAF instance, using the Nnwdaf_AnalyticsInfo_Transfer service operation as defined in clause 7.3.3. This procedure, for example, can be called during an analytics subscription transfer procedure (as specified in clause 6.1B.3) to request the transfer of relevant analytics information.

Figure 6.1B.5-1: Analytics Context Information Transfer

The procedure of analytics information transfer comprises the following steps:

1.	The consumer NWDAF requests analytics context information by invoking Nnwdaf_AnalyticsInfo_Request service operation. The parameters that can be provided in the request message are listed in clause 6.1B.6.

2.	The provider NWDAF responds with analytics context information to the consumer NWDAF.

### 6.1B.6	Contents of Analytics Context Information

The consumers of the Nnwdaf_AnalyticsInfo_Transfer service operation (as specified in clause 7.3.3) provide the following input parameters listed below:

-	A list of analytics context identifier(s): identify a set of analytics context information that are available at the NWDAF instance providing this service and that are requested to be transferred to the consumer NWDAF instance.

NOTE:	A list of "analytics context identifier(s)" is received during an analytics subscriptions transfer request as described in clause 6.1B.3.

Editor's note:	It is FFS whether an enhanced Analytics request is to be used instead of transfer service for Analytics. And it is FFS whether bulk data is to be used for Data instead of transfer service.

The NWDAF provides to the consumer of the Nnwdaf_AnalyticsInfo_Transfer service operation (as specified in clause 7.3.3), the output information listed below:

-	(Set of) Analytics context information for each of the "analytics context identifier(s)" provided as input parameter. "Analytics context information" includes the following information parts:

-	Analytics related:

-	[OPTIONAL] Pending output analytics (i.e. not yet notified to the consumer) or historical output analytics information, if available at the source NWDAF. The content of the output analytics is specified in clause 6.1.3 as output information of the Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request service operations.

-	Timestamp(s) of the last output analytics provided to the analytics consumer(s). Value is set to 0 if no output analytics had been sent yet.

-	Data collection related:

-	[OPTIONAL] Historical data that is available at the source NWDAF and that is related to the Analytics to be handed over to the target NWDAF. This includes not only the raw data collected from the data source(s), but also "analytics metadata" such as the time period of the collected data, information about the data source(s), etc. If historical data is available at the source NWDAF, this data shall be provided.

-	Information about subscriptions with data sources that are related to the analytics.

* * * Next Change * * * *

### 7.2.5	Nnwdaf_AnalyticsSubscription_Transfer service operation

Service operation name: Nnwdaf_AnalyticsSubscription_Transfer.

Description: Requests to NWDAF to transfer analytics subscription(s) from the consumer NWDAF.

Inputs, Required:

-	Transfer type: indicates the type of the transfer request. The following values are supported:

-	Analytics subscription transfer preparation: requests the NWDAF to prepare for taking over the analytics subscription(s) and/or prepare for collecting other Analytics Context(s).

-	Analytics subscription transfer: requests the NWDAF to take over the analytics subscription(s).

-	Analytics subscription transfer cancel: cancels a prepared analytics subscription request.

Inputs, Optional:

-	If this service operation is for "analytics subscription transfer preparation", the following parameters may be provided:

-	(Set of) analytics subscription(s) with the following parameters:

-		All input parameters for the analytics exposure as specified in clause 6.1.3.

-	[OPTIONAL] Active data source ID(s): Instance Id or Set Id of the active data source(s) the consumer NWDAF is currently using for the analytics of this analytics subscription.

-	Subscription Correlation Id.

-	Subscription callback URI: indicates the callback URI of the analytics consumer of this subscription, i.e. the URI the analytics notifications are to be sent to.

-	Id of analytics consumer that is subscribed to receive analytics.

-	(Set of) Analytics context identifier(s) of existing Analytics Context(s) in the NWDAF according to clause 6.1B.6.

-	If this service operation is to request analytics subscriptions transfer and if analytics subscription transfer preparation is not performed, the same parameters as those for Transfer type of "Analytics subscription transfer preparation" shall be provided.

-	If this service operation is to request analytics subscriptions transfer and if analytics subscription transfer preparation is performed, the same parameters as those for Transfer type of "Analytics subscription transfer preparation" can be provided with updated parameter values.

-	If this service operation is to request analytics subscriptions transfer cancel, the following parameter shall be provided:

-	Subscription Correlation Id.

-	(Set of) analytics context identifier(s): identifies analytics context information available at the consumer NWDAF.

Outputs Required: Operation execution result indication.

Outputs, Optional: None.

* * * End of Changes * * * *