SA WG2 Meeting #144e	S2-2102195

April 12th – April 16th, 2021 ; Elbonia

Source: 	Ericsson

Title: 	Alternative EAS discovery with Distributed anchor (KI#1, solution#12)

Document for: 	Agreement

Agenda Item: 	8.3

Work Item / Release:	eEDGE_5GC / Rel-17

Abstract of the contribution: This contribution introduces an alternative for EAS discovery with Distributed anchor, where the PDU Session anchor is distributed dynamically after the DNS Query (aka Solution #12).

# 1 Discussion

The following text is captured in clause 9.1.2	“Conclusions for Key Issue #1: Distributed Anchors” of TR 23.748:

“The following aspects from Solution #12 are recommended for normative work:

-	For SSC mode 2/3 PDU Session with central PSA UPF, when LDNSR receive DNS response from DNS server, it may trigger the SMF to release old PDU Session and establish new PDU Session using SSC mode 2/3 PSA change procedures.“

This pCR updates TS 23.548 according to these conclusions.

Solution #12 is an alternative for EAS discovery with Distributed anchor, where the PDU Session anchor is distributed dynamically after the DNS Query. It also requires EASDF and procedure has been aligned with the procedure for EAS discovery with Session Breakout and refers to that procedure whenever possible to avoid duplications and prevent misalignments.

The proposal has been that deviations or additions specific for this EC solution on procedures in TS 23.502 are specified only in the new TS. This approach has also been selected for other functionalities, e.g. for LCS. This way a reader of TS 23.502 who is not interested in EC will not face the complexity related to this functionality.

# 2 Proposal

It is proposed to update TS 23.548 as follows:

*********************************Start Change ********

#### 6.2.2.4 Procedure for EAS Discovery with dynamic PSA Distribution

5GC supports an EAS Discovery procedure that allows that at PDU session establishment the SMF selects a central PSA, regardless if a local PSA is available to the SMF, and then, it allows to dynamically re-anchor the PDU Session and transition to a Distributed Anchor Point model when needed. This is applicable to PDU Sessions type SSC#2 and SSC#3.

This procedure relies on EASDF capability to influence the DNS Query of an Edge Application so that the EAS Discovery considers a candidate UE topological location of a PSA further out in the network than current PSA. The PDU Session re-anchoring to the edge is performed as part of the EAS Discovery procedure.

This procedure requires that the DNS settings provided to the UE for the PDU Session are respected.

Figure 6.2.2.4-1 Application Server Discovery with Dynamic PSA distribution using EASDF

The EAS Discovery procedure with Dynamic PSA distribution using EASDF is described in Figure 6.2.2.4.-1

The procedure is as follows:

1.  PDU session establishment, allocation of an EASDF and sending rules to the EASDF/. Steps 1-14 in the procedure 6.2.3.2.2-1 for EAS Discovery Procedure with EASDF for Session breakout Connectivity are applied. If Dynamic PSA distribution applies to the PDU Session, the SMF may have selected a central PSA at PDU session establishment, regardless of whether  a local PSA is available:

EN: whether step 14 is the correct step is FFS, depending on the progress for

The UE sends a DNS Query message for an FQDN to the EASDF via central PSA. The EASDF checks the DNS Query against the DNS handling Rules in the DNS Context and reports to SMF and/or forwards to DNS for resolution as instructed by these rules. For resolution, it applies Option A or option B in the procedure 6.2.3.2.2-1 or sends the DNS query to a pre-configured DNS server/resolver if none of them applies. When the DNS Response is received, EASDF checks it against the DNS context matching conditions for reporting. If applicable, it reports to SMF the selected EAS and (based on SMF DNS handling rules) discards the DNS Response, else it forwards the DNS Response to the UE.

When the EASDF discards the DNS response, the UE is expected to restart the DNS request  5in the new PDU Session)

For further details see clause 6.2.3.2.2.

2.	SMF determines that the central UPF (PSA) needs to be changed to an Edge UPF (L-PSA), and it triggers one of the procedures to change the PSA of the PDU Session to a distributed anchor:

-	Change of SSC mode 2 PDU Session Anchor with different PDU Sessions as in clause 4.3.5.1 of TS 23.502 [3]. The procedure applies with the following differences:

In step 2, the DNS context for the session is removed from EASDF as part of the PDU Session Release procedure (in step 12 of the PDU Session release procedure in TS 23.502 [3] in 4.3.4.2).

In step 3, SMF selects and provisions the DNS settings for the new PDU session as required by the procedure for EAS Discovery on Distributed anchor as described in clause 6.2.2.2.

-	Change of SSC mode 3 PDU Session Anchor with multiple PDU Sessions as in clause 4.3.5.2 of TS 23.502 [3]. The procedure applies with the following differences:

In step 4, SMF selects and provisions the DNS settings for the new PDU session as required by the procedure for EAS Discovery on Distributed anchor as described in clause 6.2.2.2. After step 4, SMF can already perform from step 3 in 6.2.2.4-1.

In step 6, the DNS context for the session is removed from EASDF as part of the PDU Session Release procedure (in step 12 of the PDU Session release procedure in TS 23.502 [3] in 4.3.4.2).

-    Change of SSC mode 3 PDU Session Anchor with IPv6 Multi-homed PDU Session as in clause 4.3.5.3 of TS 23.502 [3]. The procedure applies with the following differences:

In step 12, SMF sends a PDU Session Modification Command to provide in PCO new DNS Settings for the PDU and as required by the procedure for EAS Discovery on Distributed anchor as described in clause 6.2.2.2. After step 12, SMF can already perform from step 3 in 6.2.2.4-1.

To remove the Session context in EASDF, SMF invokes Neasdf_DNSContext_Delete Request/Response.

NOTE 1:	Dynamic re-anchoring to an edge PSA implies that the UE IP address is changed from a UE IP address corresponding to the old (central) PSA to a UE IP address corresponding to the new (edge) PSA for all applications on the PDU session. If SSC#3, former PSA and IP address will still be available for some time.

NOTE 2:	Further re-anchoring (to a central UPF) can be triggered if activity is monitored e.g. if EC application traffic ceases. In that case, EASDF is provided again in the DNS settings for the PDU Session. New EAS Discovery will go to EASDF and be handled as described in Step 1.

Steps 3 to 5 only apply to SSC#3 PDU Sessions (before the DNS Context is removed from the EASDF):

3.	The SMF invokes Neasdf_DNSContext_Update Request (forward DNS response indication) and the EASDF responds with Neasdf_DNSContext_Update Response.

The forward DNS response indication is used to indicate the EASDF to forward to the UE the DNS Response received and cached in Step 1.

5.	The application may start sending traffic over UPF1 (Central PSA) to the selected EAS until an EAS rediscovery takes place or it may as well skip this step.

EN: step 5 is FFS,

6.	A new (re)discovery procedure is triggered for the application over the new PSA. (Re)discovery follows the EAS (re)Discovery procedure for distributed anchor connectivity model as in clauses 6.2.2.2. and 6.2.2.3.

7.  Application traffic starts via the PDU Session Edge PSA to the EAS selected in Step 6.

*********************************Stop Change********