3GPP TSG-SA WG2 Meeting #144-e	S2-2102202

# Electronic meeting, April 12-16 2021

Source:	Ericsson

Title:	Updates for clause 3.1 in TS 23.304

Spec:	3GPP TS 23.304 v0.1.0

Agenda item:	8.8

Document for:	Approval

Work Item / Release:	5G_ProSe/Rel.17

Abstract of the contribution: This contribution proposes to update the definition clause 3.1 by including references to definitions in TS 23.303 which are relevant for TS 23.304. Also, definitions related to UE-to-UE relay are removed since this use case is not within the scope of TS 23.304.

Discussion

This contribution proposes to update clause 3.1 by including references to definitions in TS 23.303 sinnce they are relevant for TS 23.304. Also, definitions related to UE-to-UE Relay are removed since this use case is not within the scope of TS 23.304.

Proposal

It is proposed to agree to the following changes to 3GPP TS 23.304 v0.1.0.

* * * * First Change * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Application Layer ID: An identity identifying a ProSe-enabled UE within the context of a specific application. The format of this identifier is outside the scope of 3GPP.

Direct Network Communication: One mode of network communication, where there is no ProSe UE-to-Network Relay between a UE and the 5G network.

Indirect Network Communication: One mode of network communication, where there is a ProSe UE-to-Network Relay between a UE and the 5G network.

Open ProSe Discovery: ProSe Direct Discovery without explicit permission from the ProSe-enabled UE being discovered, according to TS 22.278 [7].

ProSe Direct Discovery: A procedure employed by a ProSe-enabled UE to discover other ProSe-enabled UEs in its vicinity based on direct radio transmissions between the two UEs with NR technology.

ProSe Direct Communication: A communication between two or more UEs in proximity that are ProSe-enabled, by means of user plane transmission using NR technology via a path not traversing any network node.

ProSe UE-to-Network Relay: A ProSe-enabled UE that provides functionality to support connectivity to the network for Remote UE(s).

Remote UE: A ProSe-enabled UE that communicates with a DN via a ProSe UE-to-Network Relay.

Restricted ProSe Discovery: ProSe Direct Discovery that only takes place with explicit permission from the ProSe-enabled UE being discovered, according to TS 22.278 [7].

For the purposes of the present document, the following term and definition given in TS 23.303 [3] apply:

Application Layer Group ID

Destination Layer-2 ID

Discovery Entry ID

Discovery Filter

Discovery Group ID

Discovery Query Filter

Discovery Response Filter

Geographical Area

Local PLMN

Model A

Model B

Metadata Index

Metadata Index Mask

ProSe Application ID

ProSe Application Code

ProSe Application Mask

ProSe enabled UE

ProSe Query Code

ProSe Response Code

ProSe Restricted Code

ProSe Restricted Code Prefix

ProSe Restricted Code Suffix

ProSe Discovery UE ID

ProSe Layer-2 Group ID

ProSe-enabled UE

Restricted ProSe Application User ID

Source Layer-2 ID

User Info ID

* * * * End of Changes * * * *