3GPP TSG-WG SA2 Meeting #144E e-meeting	S2-2102212r11

# Elbonia, April 12 – 16, 2021							   			       			                                (was xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0262 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR to update MTLF services to resolve ENs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | This CR is to resolve 4 ENs with respect to MLModelProvisioning services as follows. In clause 6.2A.0, Editor's note: Whether/how ML model consumer NWDAF (i.e. AnLF) can negotiate the ML model update is FFS. In clause 6.2A.2, Editor's note: Whether other analytics exposure information is required is FFS. Editor's note: Whether other information is required is FFS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes to use Nnwdaf_MLModelProvisioning_Subscribe service operation in order for service consumer NWDAF to trigger negotiation on ML model with the service provider, i.e., MTLF with some minor editorial correction. Also, this CR proposes to have an expiry time in MLModelProvisioinig service to indicate an expiry time of the subscription regardless of the number of provided ML model. Since training and update of ML model solely depends on MTLF internal logic, it is natural that the consumer of ML model should indicate end time for the subscription rather than indicating several types of ML model reporting (e.g., periodicity of update as described in analytics reporting information) in subscription. Finally, this CR proposes to include spatial and temporal validity information on ML model, since the result of ML model training can be differ to the targeted location and time. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | ENs are still remained and the features of MTLF are not clear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.0, 6.2A.1, 6.2A.2, 7.5.2, 7.5.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * Start of the first change * * * *

6.2A.0	General

This clause presents the procedure for the ML Model provisioning.

The procedure for ML model provisioning is limited to a single vendor environment in this release of the specification.

Editor's note:	Whether/how support for hierarchy of MTLF is provided is FFS.

Editor's note:	It is FFS to define the ML model request procedure.

6.2A.1	ML Model Subscribe/Unsubscribe

The procedure in Figure 6.2A.1-1 is used by an NWDAF service consumer, i.e. an NWDAF (AnLF) to subscribe/unsubscribe at another NWDAF, i.e. an NWDAF containing MTLF, to be notified when ML model information on the related Analytics becomes available, using Nnwdaf_MLModelProvision Services as defined in clause 7.5. The ML model information is used by an NWDAF containing AnLF to derive analytics. The service is also used by an NWDAF to modify existing ML Model Subscription(s). An NWDAF (e.g. an NWDAF (MTLF+AnLF)) can be at the same time a consumer of this service provided by other NWDAF(s) and a provider of this service to other NWDAF(s).

Figure 6.2A.1-1: ML Model for analytics subscribe/unsubscribe

NOTE:	In Figure 6.2A.1-1 an NWDAF service consumer may host both an AnLF and an MTLF.

1.	The NWDAF service consumer (i.e. an NWDAF (AnLF)) subscribes to, modifies, or cancels subscription for a (set of) ML Model(s) associated with a (set of) Analytics ID(s) by invoking the Nnwdaf_MLModelProvision_Subscribe / Nnwdaf_MLModelProvision_Unsubscribe service operation. The parameters that can be provided by the NWDAF service consumer are listed in clause 7.5.

When a subscription to an ML model information for the Analytics is received, the NWDAF containing MTLF may:

-	determine whether an existing trained ML Model can be used for the subscription; or

-	determine whether triggering further training for an existing trained ML models is needed for the subscription; or

-	determine to generate an initial ML model for the subscription.

If the NWDAF containing MTLF generates an initial ML Model or determines that further training is needed, this NWDAF may initiate data collection from NFs, DCCF, or OAM as described in clause 6.2, required for the training of the ML model.

If the service invocation is for a subscription modification or subscription cancelation, the NWDAF service consumer includes an identifier (Subscription Correlation ID) to be modified in the invocation of Nnwdaf_MLModelProvision_Subscribe.

2.	If the NWDAF service consumer subscribes to a (set of) ML model(s) associated to a (set of) Analytics ID(s), the NWDAF containing MTLF notifies the NWDAF service consumer with the ML model information (containing the file address of the ML model) by invoking Nnwdaf_MLModelProvision_Notify service operation. The content of ML model information that can be provided by the NWDAF containing MTLF is specified in clause 7.5.

The NWDAF containing MTLF also invokes the Nnwdaf_MLModelProvision_Notify service operation to notify the available a re-trained ML model information when the NWDAF containing MTLF determines that the previously provided trained ML Model required re-training at step 1.

When receiving a request for ML model subscription modification, the NWDAF containing MTLF may provide either a new ML model different to the previously provided one, or re-trained ML model by invoking Nnwdaf_MLModelProvision_Notify service operation.

6.2A.2	Contents of ML Model Provisioning

The consumers of the ML model provisioning services (i.e. an AnLF of NWDAF) as described in clause 7 may provide the input parameters as listed below:

-	Information of the analytics for which the requested ML model is to be used, including:

-	A list of Analytics ID(s): identifies the analytics for which the ML model is used.

-	[OPTIONAL] Analytics Filter Information: indicates the conditions to be fulfilled for reporting Analytics Information. This set of optional parameter types and values enables to select which ML model for the analytics is requested. Analytics Filter Information are defined in each analytics procedures.

-	[OPTIONAL] Target of Analytics Reporting: indicates the object(s) for which ML model for the analytics is requested, entities such as specific UEs, a group of UE(s) or any UE (i.e. all UEs).

-	[OPTIONAL] ML Model target period: indicates time interval [start, end] for which ML model for the Analytics is requested. The time interval is expressed with actual start time and actual end time (e.g., via UTC time).

- 	[OPTIONAL] An expiry time: indicates time upto which the subscription is desired to be kept as active as described in clause 4.15.1 of TS 23.502 [3].

-	A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from the ML model provider NWDAF with this subscription.

The ML model provider NWDAF (i.e. an MTLF of NWDAF) provides to the consumer of the ML model provisioning service operations as described in clause 7, the output information as listed below:

-	The Notification Correlation Information.

-	ML model information, which includes the ML model file address (e.g. URL or FQDN) for the Analytics ID(s).

-	[OPTIONAL] Validity period: indicates time period when the provided ML model information applies.

-	[OPTIONAL] Spatial Validity: indicates Area where the provided ML model information applies.

NOTE: 	Validity period and Spatial Validity are determined by MTLF internal logic and it may not be exactly aligned with Analytics Filter Information and ML Model target period.

# * * * * Start of the second change * * * *

7.5.2	Nnwdaf_MLModelProvision_Subscribe service operation

Service operation name: Nnwdaf_MLModelProvision_Subscribe.

Description: Subscribes to NWDAF ML model provision with specific parameters.

Inputs, Required: (set of) Analytics ID(s) defined in Table 7.1-2, Notification Target Address (+ Notification Correlation ID).

Inputs, Optional: Subscription Correlation ID (in the case of modification of the ML model subscription), Analytics Filter information to indicate the conditions to be fulfilled for reporting Analytics Information, and Target of Analytics Reporting to indicate the object(s) for which ML model for the analytics is requested, entities such as specific UEs, a group of UE(s) or any UE (i.e. all UEs), ML Model target period, Expiry time.

Outputs Required: When the subscription is accepted: Subscription Correlation ID (required for management of this subscription), Expiry time (required if the subscription can be expired based on the operator’s policy).

Outputs, Optional: None.

# * * * * Start of the third change * * * *

7.5.4	Nnwdaf_MLModelProvision_Notify service operation

Service operation name: Nnwdaf_MLModelProvision_Notify.

Description: NWDAF notifies the ML model information to the consumer instance which has subscribed to the specific NWDAF service.

Inputs, Required: Set of the tuple (Analytics ID, address (e.g. URL or FQDN) of Model file), Notification Correlation Information.

Inputs, Optional: Validity period, Spatial validity.

Outputs, Required: Operation execution result indication.

Outputs, Optional: None.

# * * * * End of changes * * * *