SA WG2 Meeting #S2-144E	S2-2102230r02

# 12 - 16 April, 2021 ; Electronic meeting

# Title:	Reply to LS on UE location aspects in NTN

# Response to:	R2-2102055/S2-2102115

# Release:	Rel-17

# Work Item:	5GSAT_ARCH

Source:	SA2

To:	RAN2

Cc:	SA3-LI, RAN3, SA3, CT1

Contact Person:

Name:	Jean Yves Fine

Tel. Number:

E-mail Address:	jean-yves.fine@thalesgroup.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Attachments:	None

1. Overall Description:

SA2 would like to thank RAN2 for their LS on UE location aspects in NTN (R2-2102055/S2-2102115).

In this LS, RAN2 is asking the following questions to which SA2 would like to provide the indicated responses below.

- Question 1: RAN2 would like to ask RAN3, SA3-LI and SA2 to confirm whether the current functionality identified above is sufficient for use in Non-Terrestrial Networks including initial registration procedure.

Answer from SA2:

SA2 confirms the requirement on UE’s location information for NG-RAN to perform appropriate Core Network selection in some scenarios and to determine Cell ID for all NGAP messages containing User Location Information.

SA2 believes that according to SA3 requirement, the UE location that is obtained for regulatory services must be either network provided or at least network verified. If radio network planning allows cell size comparable to TN, then CGI can provide sufficiently accurate UE location. However, NR satellite access must be prepared also for deployment with super-national cell coverage area, in which case CGI alone does not give sufficiently accurate estimate of UE location. SA2 further notes that while accurate CGI determination for an initial registration procedure can be useful, it is not essential as long as a more accurate CGI can be provided soon after a UE has entered CONNECTED state.

- Question 2: RAN2 would like to ask SA3 and SA3-LI to confirm whether A-GNSS based UE location information, i.e. computed at network using A-GNSS based measurements provided by UE, or computed by UE, can be considered reliable e.g. for lawful interception.

Comment from SA2:

SA2 confirms that a reliable UE location is assumed mandatory for a CN supporting NTN. SA2 expects that SA3 or SA3-LI can provide a more informed response to this question.

2. Actions:

To RAN2 group.

ACTION: 	SA2 kindly asks RAN2 to align their work with these answers.

3. Date of Next SA2 Meetings:

3GPP TSG SA2 #145 (E)	            17th – 28th May 2021

3GPP TSG SA2 #146 (E)	23rd – 27th August 2021