SA WG2 Meeting #S2-144E	S2-2102230r05

# 12 - 16 April, 2021 ; Electronic meeting

Title:	Reply to LS on UE location aspects in NTN

Response to:	R2-2102055/S2-2102115

Release:	Rel-17

Work Item:	5GSAT_ARCH

Source:	SA2

To:	RAN2

Cc:	SA3-LI, RAN3, SA3, CT1

Contact Person:

Name:	Jean Yves Fine

Tel. Number:

E-mail Address:	jean-yves.fine@thalesgroup.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	None

1. Overall Description:

SA2 would like to thank RAN2 for their LS on UE location aspects in NTN (R2-2102055/S2-2102115).

In this LS, RAN2 is asking the following questions to which SA2 would like to provide the indicated responses below.

- Question 1: RAN2 would like to ask RAN3, SA3-LI and SA2 to confirm whether the current functionality identified above is sufficient for use in Non-Terrestrial Networks including initial registration procedure.

Answer from SA2:

SA2 confirms the requirement on UE’s location information for NG-RAN to perform appropriate Core Network selection in some scenarios and to determine Cell ID for all NGAP messages containing User Location Information.

SA2 believes that the methods indicated in the LS (mobility measurements, and/or UE position obtained from GNSS) may be sufficient to determine a CGI with sufficient accuracy to support services provided in 5GC such as support of emergency services calls. SA2 notes that the accuracy of a CGI may need to align with the accuracy of a CGI for TN in certain regions such as where an emergency services call needs to be routed to a specific PSAP associated with the current location of a UE.i.e. the CGI constructed by the NTN based NG-RAN should correspond to a geographical area which size shall be comparable with a cell for TN.

SA2 had ever agreed that AMF may initiate UE location procedure during and after registration if ULI (including Cell ID) received from the gNB is not accurate enough to determine the country where UE presents. SA2 would like to confirm if NG-RAN can always provide the accurate ULI to avoid invoking UE location procedure?

2. Actions:

To RAN2 group.

ACTION: 	SA2 kindly asks RAN2 to align their work with these answers.

3. Date of Next SA2 Meetings:

3GPP TSG SA2 #145 (E)	            17th – 28th May 2021

3GPP TSG SA2 #146 (E)	23rd – 27th August 2021