SA WG2 Meeting #144e (e-meeting)	S2-2102248

# 12-16 April, 2021 ; Elbonia              				     		 	 (revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2710 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Restricted PDU Session for remote provisioning of UE via UP in PLMN case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Implementing TR 23.700-07 conclusion on restricted PDU Session for remote provisioning of UE using User Plane for the PLMN case in TS 23.501: - A restricted PDU session is supported to be dedicated for the remote provisioning. Upon successful establishment of restricted access PDU session, if the UE still does not have a PS address via any of the methods listed below, the device uses a well-known FQDN to perform PS discovery. - When Onboarding network is a PLMN, the functionality to restrict usage is activated for the UE by SMF based on received existing subscription profile from the UDM and on existing policies from PCF, or local configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New subclause describing the functionality of restricted PDU Session for remote provisioning of UE via User Plane in PLMN case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Rel-17 eNPN feature not fully implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.X.4.3 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Impacts the same new clause as the technically endorsed CR 2568 from SA2#143e that is re-submited to SA2#144e in CR 2709. This text should be inserted after the text from CR 2709 or go into a new sub-clause. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

START CHANGE

#### 5.30.2.X.4.3 User Plane Remote Provisioning of UEs

When Onboarding network is a PLMN, UE network usage is restricted by the SMF based on a subscription profile used for onboarding received from UDM (e.g. containing S-NSSAI, DNN) , or based on policies from PCF as described in TS 23.503 [45], or local configuration in the SMF.

END CHANGE