3GPP TSG-SA2 # 144E (e-meeting)	S2-2102333

# April.12 – April 16, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2636 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Nsmf_EventExposure supports RAT Type Change Event |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Telecom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-3-31 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to DP S2-2102331, the SMF supports PCRT as RAT Type Change in Stage 2 TS23.503 and Stage 3 TS29.512. So, for Nsmf_EventExposure service, a NF should be able to subscribe to RAT Type Change event for RAT type inforamtion of sepecific UE. This CR is to update clause 5.2.8.3 of TS23.502, “Nsmf_eventExposure Service” with related description of “RAT type change event”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR: [Change -1] clause 5.2.8.3.1 - Add description of “RAT Type change event”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing RAT type support from Nsmf_EventExposure service. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.8.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * 1st Change * * *

5.2.8.3	Nsmf_EventExposure Service

5.2.8.3.1	General

Service description: This service provides events related to PDU Sessions towards consumer NF. The service operations exposed by this service allow other NFs to subscribe and get notified of events happening on PDU Sessions. The following are the key functionalities of this NF service.

-	Allow consumer NFs to Subscribe and unsubscribe for an Event ID on PDU Session(s);

-	Allow the NWDAF to collect data for network data analytics as specified in TS 23.288 [50];

-	Notifying events on the PDU Session to the subscribed NFs; and

-	Allow consumer NFs to acknowledge or respond to an event notification.

The following events can be subscribed by a NF consumer (Event ID is defined in clause 4.15.1):

-	UE IP address / Prefix allocation/change: The event notification may contain a new UE IP address / Prefix or an indication of which UE IP address / Prefix has been released.

-	PDU Session Establishment and/or PDU Session Release.

The event notification may contain following information:

-	PDU Session Type.

-	DNN.

-	UE IP address/Prefix.

-	UP path change: a notification corresponding to this event is sent when the UE IP address / Prefix and / or DNAI and /or the N6 traffic routing information has changed.

The event notification may contain following information:

-	the type of notification ("EARLY" or "LATE").

-	for both the source and target UP path between the UE and the DN, the corresponding information is provided when it has changed:

-	DNAI.

-	UE IP address / Prefix.

-	N6 traffic routing information.

NOTE 1:	UP path change notification, DNAI and N6 traffic routing information are further described in TS 23.501 [2] clause 5.6.7.

-	QoS Monitoring for URLLC: the event notification may contain the QoS Monitoring report as described in clause 5.33.3.2 of TS 23.501 [2].

-	Change of Access Type; The event notification contains the new Access Type for the PDU Session.

-    Change of RAT Type; the event notification contains the new RAT Type for the PDU Session.

-	PLMN change; The event notification contains the new PLMN Identifier for the PDU Session.

-	Downlink data delivery status. The event notification contains the status of downlink data buffering in the core network including:

-	First downlink packet per source of the downlink IP traffic in extended buffering and Estimated maximum wait time.

-	First downlink packet per source of the downlink IP traffic discarded.

-	First downlink packet per source of the downlink IP traffic transmitted after previous buffering and/or discarding of corresponding packet(s).

-	QFI allocation: The event notification is sent when a new QoS flow is established within a PDU session and contains:

-	If the Target of Event Reporting is a PDU session, both the allocated QFI and either one of the following (Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set). The DNN, S-NSSAI corresponding to the PDU session are also sent.

-	If the Target of Event Reporting is a SUPI, both the allocated QFI and either one of the following (Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set) for each PDU session ID established for this SUPI. The DNN, S-NSSAI corresponding to each PDU session are also sent.

-	If the Target of Event Reporting is an Internal-Group-Id or any UE, multiple instances of the tuple (allocated QFI and either one of the following (Application Identifier or IP Packet Filter Set or Ethernet Packet Filter Set). PDU session ID, SUPI). The DNN, S-NSSAI corresponding to each PDU session are also sent.

-	Total number of Session Management transactions:

-	The total number of Session Management transaction is used to collect the number of SM transactions of a SUPI or Internal Group ID, for example Dispersion Analytics as specified in TS 23.288 [50]. The transaction count is incremented when the NAS transactions from PDU Session Establishment, PDU Session Authentication, PDU Session Modification and PDU Session Release procedures is concluded. Only the periodic reporting mode applies.

-	Information on PDU Session for WLAN (i.e. Access Type is Non-3GPP and RAT Type is TRUSTED_WLAN).

NOTE 2:	When the consumer NF is the NWDAF, the event QFI allocation is used to collect data for Observed Service Experience analytics and UE communication analytics as specified in TS 23.288 [50].

-	User plane status information: The event notification contains:

-	PDU Session ID.

-	User Plane Inactivity Timer (as specified in TS 29.244 [65]).

-	PDU Session status (activated, deactivated).

NOTE 3:	When the consumer NF is the NWDAF, the event user plane status information is used to collect data for UE Communication analytics as specified in TS 23.288 [50].

-	Session Management Congestion Control Experience for PDU Session: The event notification contains the data related to Session Management Congestion Control experience per PDU Session as described in TS 23.288 [50].

-	UE behaviour trends (see clause 4.15.4.3); and

-	UE communications trends (see clause 4.15.4.3).

When the consumer NF is the NWDAF, the event Information on PDU Session for WLAN is used to collect data for WLAN performance analytics as specified in TS 23.288 [50].

When the consumer NF is the NWDAF, the event Session Management Congestion Control Experience for PDU Session is used to collect data for Session Management Congestion Control Experience analytics as specified in TS 23.288 [50].

Event Filters are used to specify the conditions to match for notifying the events (i.e. "List of Parameter values to match"). If there are no conditions to match for a specific Event ID, then the Event Filter is not provided. The following table provides as an example how the conditions to match for event reporting can be specified for various Event IDs for SMF exposure.

Table 5.2.8.3.1-1: Example of Event Filters for SMF exposure events

| Event ID for SMF exposure | Event Filter (List of Parameter Values to Match) |
| --- | --- |
| DNAI Change | None |
| PDU Session Release | None |
| PDU Session Establishment |  |
| QoS Monitoring for URLLC | None |
| QFI allocation | <Parameter Type = DNN, Value = DNN1> <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| QFI allocation | <Parameter Type = Application Identifier, Value = Application Identifier1> |
| Transaction Count | <Parameter Type = TAI, Value = TA1> (NOTE) <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| User plane status information | <Parameter Type = Application Identifier, Value = Application Identifier1> <Parameter Type = SUPI, Value = SUPI1> |
| Information on PDU Session for WLAN | <Parameter Type = Access Type, Value = Non-3GPP> && <Parameter Type = RAT Type, Value = TRUSTED_WLAN> |
| Session Management Congestion Control Experience for PDU Session | <Parameter Type = DNN, Value = DNN1> <Parameter Type = S-NSSAI, Value = S-NSSAI1> |
| NOTE: Optionally the SMF can fetch the location information from the AMF but transaction information correlation at the location can also be achieved without it and through transaction information associated with the requested time period, which corresponds to the UE's time span at the location of interest. |  |

The target of SMF event reporting may correspond to a PDU Session ID, an UE ID (SUPI), an Internal Group Identifier, an indication that any UE is targeted (e.g. on a specific DNN), or an indication that any PDU session is the target.

When acknowledgment is expected the SMF also provides Notification Correlation Information to the consumer NF in the event notification.

The consumer NF may provide the following event-specific information when acknowledging an event notification:

-	For UP path change event:

-	N6 traffic routing information related to the target DNAI.

NOTE 4:	Acknowledgement to a UP path change event notification is further described in TS 23.501 [2] clause 5.6.7.

* * *End of Changes * * *