3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102341

# Elbonia, April 12th - 16th, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0277 | rev | - | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of UE data reporting procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The UE data reporting procedure was agreed in the conclusions of the TR 23.700-91 and some normative work was performed in SA2#143E meeting, but there are still several issues that have not been added to the TS yey. There is a FFS about UE IP address and UE ID mapping in clause 6.2.8.2.4. it has not been added to the TS, although agreed in the TR how to support the data collection for a group of UE or any UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - AF or NEF is configured the the supported S-NSSAI(s) and DNN(s) and the supported UE attribute ID(s) in clause 6.2.8.1. - - Add UE IP address mapping for AF in trusted domain in clause 6.2.8.2.4.2. - Add UE IP address mapping for AF in untrusted domain in clause 6.2.8.2.4.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE data collection procedure is incompleted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.8.1, 6.2.8.2.2, 6.2.8.2.4.1 (new clause), 6.2.8.2.4.2 (new clause), 6.2.8.2.4.3 (new clause) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------------------------------------------------------First Change----------------------------------------------------------

#### 6.2.8.1	General

The NWDAF may interact with an AF to collect data from UE Application(s) as an input for analytics generation. The AF can be in the MNO domain or an AF external to MNO domain. The data collection request from NWDAF may trigger the AF to collect data from the UE Application.

The UE establishes a connection to the AF in the MNO domain or external to MNO domain over user plane via a PDU session. The AF communicates with the UE Application and collects data from UE Application.

For both an AF in trusted domain and an AF in untrusted domain (which supports to collect data from a UE application), the SLA between the operator and the application service provider (i.e. ASP) determines:

-	The AF for the UE Application to connect to (e.g. based on the FQDN).

-	The information that the UE Application shares with the AF, subject to user consent.

-	Possible Data Anonymization, Aggregation or Normalization, algorithms (if used).

-	The authentication information that enable the AF to verify the authenticity of the UE's Application that provides data.

NOTE 1:	The mutual authentication info that is used by the UE Application and the AF and how user consent is obtained is out of SA WG2's scope.

-	The supported S-NSSAIand DNN combination for the AF in trusted domain,

The AF (which supports the data collection) is configured based on the SLA above.

A UE Application (which supports to providing data to an AF) is configured/ provisioned with the below information from the ASP:

-	The address of the AF to contact.

-	The parameters that the UE Application is authorized to provide to the AF.

-	The authentication information to enable the UE Application to verify the authenticity of the AF that requests data.

NOTE 2:	The authentication and authorization info that is used by the UE Application and the AF for collection and how user consent is obtained is out of SA2's scope.

NOTE 3:	The configuration procedure for the above information from the ASP to the UE's Application is out of SA WG2 scope.

--------------------------------------------------------------------Next Change----------------------------------------------------------

##### 6.2.8.2.2	AF registration and discovery with NRF

The AF registers its available NF profile to the NRF. The AF in trusted domain registers to the NRF by using the Nnrf_NFManagement service that is defined in clause 5.2.7.2 of TS 23.502 [3]. The AF in untrusted domain registers the available NF profile to the NRF via the NEF as described in clause 6.2.2.3.

Since the UE Application provides the external application ID as described in clause 6.2.8.2.1 while the NWDAF request contains the SUPI and the internal application ID, the AF needs to correlate both the external application ID and the internal application ID. The AF (in trusted domain) or NEF (for AF in untrusted domain) registers at the NRF both the external application ID and internal application ID  as part of its available NF profile as also described in clause 6.2.2.3.

NOTE:	For the AF supporting data collection, it is only supported to register the external application ID and internal application ID (or any relevant Event ID) as AF profiles to NRF in this release.

The NWDAF discovers the AF serving the external application ID or the Internal Application ID from NRF by using the Nnrf_NFDiscovery service that defined in clause 5.2.7.3 of TS 23.502 [3].

--------------------------------------------------------------------Next Change----------------------------------------------------------

##### 6.2.8.2.4	Correlation between UE data collection and the NWDAF data request

###### 6.2.8.2.4.1	General

The UE IP address is used to identify the user plane connection established between the UE application and the AF for data collection, while the AF receives the Naf_Event_Exposure_Subscribe to request for the specific UE data collection by using SUPI (for AF in trusted domain) or external UE ID (i.e. GPSI) (for AF in untrusted domain). AF is required to correlate the UE IP address to the SUPI or to GPSI.

--------------------------------------------------------------------End of Changes --------------------------------------------------------