3GPP TSG-SA2 Meeting #144E	S2-2102364

# 12– 16 April 2021                   		 	 										(revision of S2-210)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2640 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support for TSC Capability Exposure – procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIoT, 5G_AIS |  |  |  |  |  | Date: |  |  | 2021-04-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Provide TS23.502 TSC Capability Exposure procedure to implement the Conclusion in TR23.700-20 clause 8.5 for Exposure of deterministic QoS. AF can use exposed information to formulate QoS requests to the 5GS. For example - the AF may: - Divide an overall delay budget between PDU sessions for UE-UE according to the reported 5GS delay for each PDU session - Allocate a portion of an overall delay budget to a PDU session for UE-DN according to the reported 5GS delay for the PDU session - Learn about the endpoints (identfied via MAC or IP@) are connected via the 5GS based on the AF query and hence warrant QoS setup - Determine whether GFBR/MFBR and MDBV offered by the 5GS is adequate for an Application The Capability Exposure of the Delay Critical QoS capability can be used to the AIS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new sub-section describing Exposure for TSC QoS is proposed. Services definitions are also updated/proposed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - Agreed approach for exposure of deterministic QoS for TSC is incomplete and AF will not know what QoS it can request. - AIS can not be supported in 5G. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.x, 5.2.5.1, 5.2.5.x, 5.2.5.1, 5.2.6.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

#### 4.15.6.x	TSC Capability Exposure

This procedure exposes 5GS information to aid the AF in formulating a request to establish or modify an AF session with required QoS. The AF may use 5GS delay information reported by the NEF to allocate a portion of an end-to-end delay budget to one or more PDU sessions and the MDBV and GFBR/MFBR to determine whether transmission of specific TSC streams is possible.

Figure 6.5.3-2: TSC Capability exposure towards external AF

1.	The AF requests the 5GS Capability to support QoS parameters along with traffic pattern. The AF includes an Application Identifier and one or more MAC addresses of devices, GPSIs or IP addresses in the Nnef_TSCCapability_Request.

2.	The NEF authorizes the AF and finds for each UE ID, the PCF (e.g: by using the BSF) that serves the PDU session. If the authorization is granted, the NEF sends Npcf_TSCCapability_Requests to each PCF. If the authorization is not granted, steps 3 and 4 are skipped and the NEF replies to the AF with a result indication value “authorization failed”.

NOTE 1:	The NEF sends the Npcf_ TSCCapability_Request for devices where UPF MAC learning has detected the AF requested MAC address as a Source Address in an uplink frame (and hence the device is connected via the associated PDU session), or where the IP address or GPSI is associated with a UE. The BSF may be used to finds a PCF for the UE address/GPSI.

3.	The PCF provides a Npcf_ TSCCapability_Request response to the NEF containing the Delay Critical QoS Capability for each UE ID:

- 	If the UE-DS-TT has reported a residence time to the PCF (see clause 4.16.5.1: SMF initiated SM Policy Association Modification), then the residence time is included in the response

- 	If the PCF is configured with one or more 5QI that supports delay critical QoS and the QoS is allowed for the Application Identifier, then for each allowed 5QI, the PDB, GFBR, MFBR and MDBV QoS Characteristics are included in the response.

4.	For PDU Sessions for which a Residence Time and one or more PDB is received from the PCF, the NEF aggregates UE-DS-TT residence time and each PDB to determine the 5GS delays for TSC.

The NEF responds to the AF with Delay Critical QoS Capability containing for each UE-ID:

- One or more sets of 5GS Delay, PDB, GFBR, MFBR and MDBV and a “success” result indication

- The result indication “no PDU session found” if the requested MAC address GPSI, or IP address is not found in step 2

Next Change

### 5.2.6	NEF Services

#### 5.2.6.1	General

The following table shows the NEF Services and Service Operations:

Table 5.2.6.1-1: NF Services provided by the NEF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nnef_EventExposure | Subscribe | Subscribe/Notify | AF, NWDAF |
|  | Unsubscribe |  | AF, NWDAF |
|  | Notify |  | AF, NWDAF |
| Nnef_PFDManagement | Fetch | Request/Response | SMF |
|  | Subscribe | Subscribe/Notify | SMF |
|  | Notify |  | SMF |
|  | Unsubscribe |  | SMF |
|  | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_ParameterProvision | Update | Request/Response | AF |
|  | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_Trigger | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
| Nnef_BDTPNegotiation | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify |  | AF |
| Nnef_TrafficInfluence | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
|  | Notify | Subscribe/Notify | AF |
|  | AppRelocationInfo | Subscribe/Notify | AF |
| Nnef_ChargeableParty | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Notify | Request/Response | AF |
| Nnef_AFsessionWithQoS | Create | Request/Response | AF |
|  | Notify | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Revoke | Request/Response | AF |
| Nnef_MSISDN-less_MO_SMS | Notify | Notify | AF |
| Nnef_ServiceParameter | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Get | Request/Response | AF |
| Nnef_APISupportCapability | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe | Subscribe/Notify | AF |
|  | Notify | Subscribe/Notify | AF |
| Nnef_NIDDConfiguration | Create | Request/Response | AF |
|  | TriggerNotify | Subscribe/Notify | AF |
|  | UpdateNotify | Subscribe/Notify | AF |
|  | Delete | Request/Response | AF |
| Nnef_NIDD | Delivery | Request/Response | AF |
|  | DeliveryNotify | Subscribe/Notify | AF |
|  | GroupDeliveryNotify | Notify | AF |
| Nnef_SMContext | Create | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
|  | DeleteNotify | Subscribe/Notify | SMF |
|  | Delivery | Request/Response | SMF |
| Nnef_AnalyticsExposure | Subscribe | Subscribe/Notify | AF |
|  | Unsubscribe |  | AF |
|  | Notify |  | AF |
|  | Fetch | Request/Response | AF |
| Nnef_UCMFProvisioning | Create | Request/Response | AF |
|  | Delete | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ECRestriction | Get | Request/Response | AF |
|  | Update | Request/Response | AF |
| Nnef_ApplyPolicy | Create | Request/Response | AF |
|  | Update | Request/Response | AF |
|  | Delete | Request/Response | AF |
| Nnef_Location | LocationUpdateNotify | Notify | AF |
| Nnef_TSCCapability | Get | Request/Response | AF |

Next Change (all new text)

#### 5.2.6.x	Nnef_TSC_Capability service

##### 5.2.6.x.1	General

This service is used to learn about support for TSC QoS for an application and UE. See clause 4.15.6.x.

##### 5.2.6.x.2	Nnef_TSCCapability_Request service operation

Service operation name: Nnef_TSCCapability_Request

Description: The consumer requests the network to provide information on support for Delay Critical QoS for a UE(s) and an AF.

Inputs, Required: AF Identifier, UE identifier(s) (i.e. IP address, MAC address or GPSI), Transaction Reference ID.

Inputs, Optional: None

Outputs, Required:, Result Indication(s)

Output (optional): Delay Critical QoS Capability

Next Change

### 5.2.5	PCF Services

#### 5.2.5.1	General

The following table illustrates the PCF Services.

Table 5.2.5.1-1: NF services provided by PCF

| Service Name | Service Operations | Operation Semantics | Example Consumer (s) |
| --- | --- | --- | --- |
| Npcf_AMPolicyControl | Create | Request/Response | AMF |
|  | Update | Request/Response | AMF |
|  | UpdateNotify | Subscribe/Notify | AMF |
|  | Delete | Request/Response | AMF |
| Npcf_Policy Authorization | Create | Request/Response | AF, NEF |
|  | Update | Request/Response | AF, NEF |
|  | Delete | Request/Response | AF, NEF |
|  | Notify | Subscribe/Notify | AF, NEF, NWDAF |
|  | Subscribe |  | AF, NEF, NWDAF |
|  | Unsubscribe |  | AF, NEF, NWDAF |
| Npcf_SMPolicyControl | Create | Request/Response | SMF |
|  | UpdateNotify | Subscribe/Notify | SMF |
|  | Update | Request/Response | SMF |
|  | Delete | Request/Response | SMF |
| Npcf_BDTPolicyControl | Create | Request/Response | NEF |
|  | Update | Request/Response | NEF |
|  | Notify |  | NEF |
| Npcf_UEPolicyControl | Create | Request/Response | AMF, V-PCF |
|  | Update | Request/Response | AMF, V-PCF |
|  | UpdateNotify | Subscribe/Notify | AMF, V-PCF |
|  | Delete | Request/Response | AMF, V-PCF |
| Npcf_EventExposure | Subscribe | Subscribe/Notify | NEF, NWDAF |
|  | Unsubscribe |  |  |
|  | Notify |  |  |
| Npcf_TSCCapability | Get | Request/Response | NEF |

Next Change

#### 5.2.6.x	Npcf_TSC_Capability service

##### 5.2.6.x.1	General

This service is used to learn whether TSC QoS can be supported for an application and UEs. See clause 4.15.6.x.

##### 5.2.6.x	Npcf_TSCCapability_Request service operation

Service operation name: Npcf_TSCCapability_Request

Description: The consumer requests the PCF to provide information on support for Delay Critical QoS for a UE(s) and an AF.

Inputs, Required: AF Identifier, UE identifier(s) (i.e. IP address, MAC address or GPSI)

Inputs, Optional: None

Outputs, Required: Result Indication(s)

Output (optional): Delay Critical QoS Capability

End of CHANGEs