SA WG2 Meeting #144e	S2-2102366r02

# Apr 12th – Apr 16th, 2021 ; Elbonia                   		 	 				(revision of S2-210)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2737 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | SNPN with separate entity hosting subscription |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell, Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-03-31 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Chapter 5.30.2.9 describing UE authenticationg using a AAA server owned by a Credential Holder has beem introduced in version 16.8.0 of 3GPP TS 23.501. The quesiton whether AUSF of the SNPN and AAA of the CH should have a direct interface or if a new NF should be introduced has been identified as a study item leading to a Editor's Note. No agreement during the FS_eNPN study phase to include a new NF. Discussion: Arguments supporting direct interface between AUSF and AAA: - Any new NF between AUSF and AAA would be part of the Control Plane as well. That is, any argument that Control Plane should consist only of SBA Interfaces would be equally valid for the new NF as well. - The interface between AUSF and AAA is used to provide secret key material (i.e,, MSK or EMSK used to generate the 5G key hierarchy), which should not be available to any intermediary function. If a new NF was acting as protocol tranlator between AUSF (talking SBA) and AAA (talking e.g. Radius), this new NF would be inevitably be able to see the unciphered secret key material, making it an additional target for attack. Usage of an intermediate function between AUSF and AAA might be acceptable in certain deployment scenarios, but it should not be an architectural default specified normatively. SA3 has also concluded that AUSF should be enhanced to interface with the external entity (and not with a new NF) - From an architectural point of view the NSSAAF and AUSF have analogous functionality and identical architectural roles, with the difference that NSSAAF is handling network slice specific authentication and authorization (NSSAA) and AUSF primary authentication (aka UE authentication) . In both cases, slice specifc authentication and authorization (NSSAA)as well as primary authentication using external AAA; the AMF is granting resources to a UE based on the authentication and authorizaton of an external AAA server. In case of NSSAA a single NF (besides deployment specific proxies) is placed between AMF and the AAA. This NF, called NSSAAF, is offering a SBA authentication service (Nnssaaf_NSSAA_Authenticate) to the AMF and handles the AAA interworking without involvement of an intermediary NF. Likewise, for primary authentication the AUSF offers authentication service (Nausf_UEAuthentication_authenticate) to the AMF. In analogy to the NSSAA the AUSF can take care about the AAA interworking and does not need to involve a new intermediary NF for this purpose. If synergies between NFs were to be exploited, these synergies are between NSSAAF and AUSF, but not between NSSAAF and new NF. Arguments supporting introduction of new NF: - AUSF is part of control plane and should not use non SBA Interfaces - The new NF would over similar functionality as existing NSSAAF and therefore synergies (e.g. colocation) between new NF and NSSAF could be exploited. Conclusions: Based on security requirements, which prevail other architectural considerations, the integration of a SNPN and a CH hosting a AAA server, should use a direct interface between the SNPN'S AUSF and the CH's AAA server. Also from an architectural point of view a direct interface between AUSF and AAA is preferred. In this way maximum consistency between primary authentication using external AAA server and Network Slice Specific Authentication and Authorization can be achieved. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Implement the above conclusion as follows: - Editor's note related to interface between SNPN and AAA shall be removed - Direct line between AUSF and AAA in the architecture figure 5.30.2.9.1-1 to be added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work on eNPN Feature cannot be finalized, EN in case of SNPN with CH architecture remains. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.30.2.9	SNPN connectivity for UEs with credentials owned by Credentials Holder

5.30.2.9.1	General

SNPNs may support UE access using credentials owned by a Credentials Holder separate from the SNPN. In this case the Session Management procedures (i.e. PDU Sessions) terminate in an SMF in the SNPN.

5.30.2.9.2	Credentials Holder using AAA Server for primary authentication and authorization

The AUSF in SNPN may support primary authentication and authorization of UEs that use credentials from an AAA Server in a Credentials Holder (CH).

-	If the UDM instructs AUSF that primary authentication by a AAA server in a CH is required, the AUSF shall discover and select the AAA server, and then forward EAP messages to the AAA server which is acting as the EAP Server for the purpose of primary authentication.

-	The SUPI is used to identify the UE during primary authentication and authorization towards the AAA sever. SUPI privacy is achieved according to existing methods in Annex I.5 of TS 33.501 [29].

-	After completing the primary authentication with the AAA server successfully, the AMF and SMF shall retrieve the UE subscription data from UDM using SUPI.

Figure 5.30.2.9.1-1 depicts the 5G System architecture for SNPN with Credentials Holder using AAA Server for primary authentication and authorization.

Figure 5.30.2.9.1-1: 5G System architecture with access to SNPN using credentials from Credentials Holder using AAA Server

END CHANGE