3GPP TSG-WG SA2 Meeting #144E e-meeting  	S2-2102409r01

# Elbonia, April 12 – 16, 2021	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2751 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE Mobility support for SNPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Capture TR conclusion regarding mobility scenarios in 23700-07 clause 8.1.2: - In the case that there are common AMF and/or N14 interface between the source network and target network, mechanism defined in TS 23.502 [6] clause 4.9.1 is re-used to address UE mobility. - In the case of idle mode mobility, the UE performs initial or mobility registration as specified in clause 4.2.2.2.2 of TS 23.502 [6]. - Support of authentication using credentials from an external entity is homogenous throughout a SNPN i.e., the SIB information in clause 8.1.4 should be set uniformly and no changes in mobility handling are needed to address inhomogeneous support of the feature. NOTE: Needed updates to find the correct source or target AMF and what are the applicable UE identities in the registration message will be determined in normative phase. Besides that, During UE mobility from/to SNPN, it needs to clarify how to recover the PDU Session in the target network: - In the case of idle mode mobility, the UE may attempt to recover the previously established PDU Sessions in the following manner: - set the List Of PDU Sessions To Be Activated in registration request during mobility registration update procedure within the target network - initiate the Handover of a PDU Session between 3GPP and untrusted non-3GPP access procedure that follows a PDU Session Establishment procedure at the target network - initiate PDU Session Establishment procedure with Initial Request indication and same DNN, S-NSSAI as the previous PDU Session - In the case of connected mode mobility, the UE is handed over from source network to target network as specified in TS 23.502 clause 4.9.1 if there are common AMF and/or N14 interface between the source network and target network |  |  |  |  |  |  |  |  |
| Summary of change: |  | Capture TR conclusion regarding mobility scenarios in 23700-07 clause 8.1.2. And in the case of idle mode mobility, the UE may attempt to recover the previously established PDU Sessions in the following manner: - set the List Of PDU Sessions To Be Activated in registration request during mobility registration update procedure within the target network - initiate the Handover of a PDU Session between 3GPP and untrusted non-3GPP access procedure that follows a PDU Session Establishment procedure at the target network - initiate PDU Session Establishment procedure with Initial Request indication and same DNN, S-NSSAI as the previous PDU Session In the case of connected mode mobility, the UE is handed over from source network to target network as specified in TS 23.502 clause 4.9.1 if there are common AMF and/or N14 interface between the source network and target network |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mobility between SNPNs and PLMN is not supported and it is Not clear how to recover the previously established PDU Sessions during UE mobility from/to SNPN |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.30.2.X	UE Mobility support for SNPN

When there are common AMF and/or N14 interface between the SNPNs the UE can be handed over from source network to target network as specified in TS 23.502 [3] clause 4.9.1 for the case of connected mode mobility.

When a UE supports access to an SNPN using credentials from a Credential Holder, then the UE performs initial or mobility registration as specified in TS 23.502 [3] clause 4.2.2.2.2 for the case of idle mode mobility when the UE moves between SNPNs.

When a UE supports access to an SNPN using credentials from a Credential Holder, then the UE performs initial registration as specified in TS 23.502 [3] clause 4.2.2.2.2 for the case of idle mode mobility when the UE moves between SNPN and PLMN.

The UE may locally attempt to recover the previously established PDU Sessions as specified in TS 23.502 [3] clause 4.2.2.2.2, 4.3.2.2.1 and 4.9.2.

# * * * * End of changes * * * *