3GPP TSG-WG SA2 Meeting #144E e-meeting  	S2-2102415r02

# Elbonia, April 12 – 16, 2021	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2754 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of CH controlled prioritized list of preferred SNPNs and GINs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Miss details on support of dynamic update of CH controlled prioritized list of preferred SNPNs and GINs, it needs to clarify: - The UE subscription data shall contain CH controlled prioritized list of preferred SNPNs and CH controlled prioritized list of GINs. - When the CH offers AUSF and UDM, the UDM of the CH can dynamically update the CH controlled prioritized list of preferred SNPNs and/or GINs in the UE configuration. - When the CH offers AAA server, the UDM of the SNPN can dynamically update the CH controlled prioritized list of preferred SNPNs and/or GINs in the UE configuration. In addition, if the SNPN-enable UE that supports access to an SNPN with credentials from a CH offering AUSF and UDM is equipped with a PLMN subscription, it needs to clarify whether the UE can access to a PLMN using the PLMN subscription when the UE moves out of the SNPN coverage or the UE moves in the common coverage area of PLMN and SNPN. Therefore, when the SNPN-enable UE that supports access to an SNPN with credentials from a CH offering AUSF and UDM is equipped with a PLMN subscription, an “SNPN-only” indication that whether the UE is only allowed to access the SNPN needs to be contained in the UE subscription and UE configuration. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add details on how to support dynamic update of CH controlled prioritized list of preferred SNPNs and GINs |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Not clear how to support of dynamic update of prioritized list of preferred SNPNs and GINs to the UE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

5.30.2.3	UE configuration and subscription aspects

An SNPN-enabled UE is configured with the following information for each subscribed SNPN:

-	PLMN ID and NID of the SNPN;

-	Subscriber identifier (SUPI) and credentials;

-	Optionally, an N3IWF FQDN and an identifier of the country where the configured N3IWF is located;

-	Optionally, if the UE supports access to an SNPN using credentials from a Credentials Holder using AUSF and UDM or using AAA Server as specified in clause 5.30.2.9:

-	User controlled prioritized list of preferred SNPNs;

-	Credentials Holder controlled prioritized list of preferred SNPNs;

-	Credentials Holder controlled prioritized list of GINs.

A subscriber of an SNPN is either:

-	identified by a SUPI containing a network-specific identifier that takes the form of a Network Access Identifier (NAI) using the NAI RFC 7542 [20] based user identification as defined in TS 23.003 [19] clause 28.7.2. The realm part of the NAI may include the NID of the SNPN; or

-	identified by a SUPI containing an IMSI.

In the case of access to an SNPN using credentials owned by a Credentials Holder as specified in clause 5.30.2.9.3, the SUPI shall also contain identification for the Credentials Holder (i.e. the realm in the case of Network Specific Identifier based SUPI or the MCC and MNC in the case of an IMSI based SUPI).

NOTE 1:	When Credentials Holder is an SNPN, and the MCC and MNC of the SNPN is not unique, then IMSI based SUPI is not supported as the MCC and MNC need not be unique always; instead USIM credentials are supported using Network Specific Identifier based SUPI.

NOTE 2:	Network Specific Identifier are not supported for the case the Credentials Holder is provided by a PLMN.

An SNPN-enabled UE that supports access to an SNPN using credentials from a Credentials Holder and that is equipped with a PLMN subscription may additionally be configured with the following information for SNPN selection and registration using the PLMN subscription in SNPN access mode:

-	User controlled prioritized list of preferred SNPNs;

-	Credentials Holder controlled prioritized list of preferred SNPNs;

-	Credentials Holder controlled prioritized list of preferred GINs.

The UE subscription data shall contain the Credentials Holder controlled prioritized lists of preferred SNPNs and GINs. The Credentials Holder controlled prioritized lists of preferred SNPNs and GINs in the UE configuration can be pre-configured or dynamically updated in the following cases:

-	When the SNPN-enable UE supports access to an SNPN with credentials from a Credentials Holder offering AUSF and UDM, the Credentials Holder controlled prioritized list of preferred SNPNs and GINs in the UE configuration can be updated by UDM of the Credentials Holder.

Editor's note:	Whether the Credentials Holder controlled lists can be updated using the UE Parameters Update (UPU) via UDM Control Plane Procedure as defined in TS 23.502 [3] clause 4.20 or the Steering of Roaming (SoR) procedure as defined in TS 23.122 [17] Annex C will be determined based on feedback from CT WG1 and SA WG3.

Editor's note:	If the SNPN-enable UE that supports access to an SNPN with credentials from a CH offering AUSF and UDM is using a PLMN subscription, it is FFS whether the UE can access to a PLMN using the PLMN subscription when the UE moves out of the SNPN coverage or the UE moves in the common coverage area of PLMN and SNPN.

# * * * * End of changes * * * *