3GPP TSG-SA/WG2 Meeting #144e	S2-2102430r03

# Elbonia, Apr. 12 – 16 , 2020			   (revision of S2-210xxxx)

Source:	Intel, Samsung

Title:	Update Clause 5.1.3

Document for:	Agreement

Agenda Item: 	8.8

Work Item / Release:	5G_ProSe / Rel-17

Abstract of the contribution: This contribution proposes to update clause 5.1.3 for ProSe Direct Communication.

# 1	Proposal

It is proposed to agree the changes into TS 23.304:

- Resolve the EN w.r.t. the identifier for ProSe service. Align the identifier for ProSe service (Application ID) for ProSe parameters for direct communication. Since ProSe Application ID defined in TS 23.303 will be introduced into 23.304, thereby it’s further proposed to replace the Application ID with ProSe Application ID and remove the reference to 23.503.

- Replace the “not served by E-UTRA” and “not served by NR” with “not served by NG-RAN” in order to align the changes made to clause 5.1.2.1 and 5.1.3.1 at SA2#143e.

/************Start of Change**************/

### 5.1.3	Authorization and Provisioning for ProSe Direct Communication

#### 5.1.3.1	Policy/Parameter provisioning for ProSe Direct Communication

The following sets of information for ProSe Direct Communications over PC5 reference point is provisioned to the UE:

1)	Authorization policy:

-	When the UE is "served by NG-RAN":

-	PLMNs in which the UE is authorized to perform 5G ProSe Direct Communications over PC5 reference point when "served by NG-RAN".

-	When the UE is "not served by NG-RAN":

-	Indicates whether the UE is authorized to perform ProSe Direct Communications over PC5 reference point when "not served by NG-RAN".

NOTE 1:	In this specification, {When the UE is "served by NG-RAN"} and {When the UE is "not served by NG-RAN"} are relevant to ProSe Direct Communications over NR PC5 reference point.

2)	Groupcast mode ProSe Direct Communication policy/parameters:

-	For each application layer group supported include the parameters that enable the UE to perform Groupcast mode ProSe Direct Communication when provisioned from PCF or pre-configured in the ME or configured in the UICC:

-	Application Layer Group ID: Identifies an application layer group that the UE belongs to.

-	ProSe Layer-2 Group ID

-	ProSe Group IP multicast address

-	Indication whether the UE should use IPv4 or IPv6 for that group

-	For a specific Group configured to operate using IPv4, optionally an IPv4 address to be used by the UE as a source address. If none is provisioned, then the UE shall use Dynamic Configuration of IPv4 Link-Local Addresses RFC 3927 [18] to obtain a link local address for the Group.

-	Include group security related content for Groupcast mode ProSe Direct Communication.

NOTE 2:	More details on the necessary security aspect will be defined in SA3 specifications.

NOTE 3:	Groupcast mode ProSe Direct Communication parameters can be provided from ProSe Application Server to the UE.

3)	Radio parameters when the UE is "not served by NG-RAN":

-	Includes the radio parameters NR PC5 with Geographical Area(s) and an indication of whether they are "operator managed" or "non-operator managed". The UE uses the radio parameters to perform ProSe Direct Communications over PC5 reference point when "not served by NG-RAN" only if the UE can reliably locate itself in the corresponding Geographical Area. Otherwise, the UE is not authorized to transmit.

NOTE 4:	Whether a frequency band is "operator managed" or "non-operator managed" in a given Geographical Area is defined by local regulations.

4)	Policy/parameters related to privacy:

-	The list of ProSe services (i.e. Application IDs) with Geographical Area(s) that require privacy support.

5)	Policy/parameters when NR PC5 is selected:

-	The mapping of ProSe services (i.e. Application IDs) to radio frequencies with Geographical Area(s).

-	The mapping of ProSe services (i.e. Application IDs) to Destination Layer-2 ID(s) for broadcast.

-	The mapping of ProSe services (i.e. Application IDs) to Destination Layer-2 ID(s) for groupcast.

-	The mapping of ProSe services (i.e. Application IDs) to default Destination Layer-2 ID(s) for initial signalling to establish unicast connection.

NOTE 5:	The same default Destination Layer-2 ID for unicast initial signalling can be mapped to more than one ProSe service. In the case where different ProSe services are mapped to distinct default Destination Layer-2 IDs, when the UE intends to establish a single unicast link that can be used for more than one ProSe service, the UE can select any of the default Destination Layer-2 IDs to use for the initial signalling.

-	The mapping of ProSe services (i.e. Application IDs) to PC5 QoS parameters defined in clause 5.4.2 (i.e. PQI and conditionally other parameters such as MFBR/GFBR, etc.).

-	SLRB configurations, i.e. the mapping of PC5 QoS profile(s) to SLRB(s), when the UE is "not served by NG-RAN".

-	The PC5 QoS profile contains PC5 QoS parameters described in clause 5.4.2 of TS 23.287 [2], and value for the QoS characteristics regarding Priority Level, Averaging Window, Maximum Data Burst Volume if default value is not used as defined in Table 5.4.4-1 of TS 23.287 [2].

Editor's note:	It is FFS on how to identify ProSe services.

6)	Path selection policy:

-	The mapping of ProSe services  to path preference (i.e. PC5 preferred, Uu preferred, or no preference) as defined in clause 5.12.

Editor's note:	It is FFS on how to identify ProSe services for Path selection.

NOTE 6:	The path selection policy can be a one mapping for all ProSe services, i.e. same path preference for all ProSe services.

7)	Validity time indicating the expiration time of the Policy/Parameter for ProSe Direct Communication.

NOTE 7:	The Application ID is defined in TS 23.503 [9].

#### 5.1.3.2	Principles for applying parameters for ProSe Direct Communication

For ProSe Direct Communication over PC5 reference point, the operator may pre-configure the UEs with the required provisioning parameters for ProSe Direct Communication, without the need for the UEs to connect to the 5GC to get this initial configuration. The following apply:

-	The provisioning parameters for ProSe Direct Communication may be configured in the UICC, in the ME, or in both the UICC and the ME.

-	The ME provisioning parameters shall not be erased when a USIM is deselected or replaced.

-	If both the UICC and the ME contain the same set of overlapping provisioning parameters, the set of parameters from the UICC shall take precedence.

-	The provisioning parameters from the PCF or ProSe Application Server via PC1 reference point shall take precedence over the pre-configured parameters in the ME and UICC as defined in clause 5.1.1.

-	The UE shall use radio resources for ProSe Direct Communication as follows:

-	While a UE has a serving cell and is camped on a cell and the UE intends to use for ProSe the radio resources (i.e. carrier frequency) operated by this cell, then the UE shall use the radio resource description indicated by this cell the UE is camped on and ignore any radio resource description of the same radio resource provisioned in the ME or the UICC. If the cell does not provide radio resources for ProSe, the UE shall not perform ProSe message transmission and reception on radio resources operated by this cell.

-	If the UE intends to use "operator-managed" radio resources (i.e. carrier frequency) for ProSe that are not operated by the UE's serving cell, as specified in clause 5.1.3.1, or if the UE is out of coverage, the UE shall search for a cell in any PLMN that is operating the provisioned radio resources (i.e. carrier frequency) as defined in TS 38.300 [12] and TS 38.304 [13], and:

-	If the UE finds such a cell in the registered PLMN or a PLMN equivalent to the registered PLMN, and authorization for ProSe Direct Communication to this PLMN is confirmed, the UE shall use the radio resource description indicated by that cell. If that cell does not provide radio resources for ProSe, the UE shall not perform ProSe message transmission and reception on those radio resources.

-	If the UE finds such a cell but not in the registered PLMN or a PLMN equivalent to the registered PLMN, and that cell belongs to a PLMN authorized for ProSe Direct Communications and provides radio resources for ProSe then the UE shall perform PLMN selection triggered by ProSe Direct Communication as defined in TS 23.122 [14].

-	If the UE finds such cell but not in a PLMN authorized for ProSe Direct Communications the UE shall not use ProSe Direct Communication.

-	If the UE does not find any such cell in any PLMN, then the UE shall consider itself "not served by NG-RAN" and use radio resources provisioned in the ME or the UICC. If no such provision exists in the ME or the UICC or the provision does not authorize ProSe Direct Communication, then the UE is not authorized to transmit.

-	The UE is allowed to use "operator-managed" radio resources (i.e. carrier frequency) for ProSe Direct Communication when the UICC shall indicate it.

-	If the UE intends to use "non-operator-managed" radio resources (i.e. carrier frequency) for ProSe, according to TS 36.331 [15] or TS 38.331 [16] and as specified in clause 5.1.3.1, then the UE shall perform ProSe Direct Communication using resource provisioned in the ME or the UICC. If no such provision exists in the ME or the UICC or the provision does not authorize ProSe Direct Communications, then the UE is not authorized to transmit.

NOTE 1:	It is possible for operators to configure UEs (e.g. Public Safety UEs) to use only "operator-managed" radio resources (i.e. carrier frequency) for ProSe Direct Communication when the UE is "not served by NG-RAN".

-	The UE provisioning shall support setting Geographical Areas.

NOTE 2:	It is possible for a UE to use other radio resources for ProSe based on the Geographical Area instead of those operated by the serving NG-RAN cell, when provisioned in the UE, even if the UE's serving cell offers normal service and the SIBs for NR sidelink communication defined in TS 38.331 [16] indicates that the service (ProSe Direct Communication) is available. This is to cover the scenario when e.g. the radio resources used for ProSe Direct Communication are not owned by the serving network of the UE.

NOTE 3:	When cross-carrier operation is supported, according to TS 36.331 [15] or TS 38.331 [16], a UE can be instructed by its serving cell to perform ProSe Direct Communication over a different carrier frequency. The UE is still considered as "served by NR or E-UTRA" in this case.

NOTE 4:	The scenario that a cell is detected and the cell does not provide support for ProSe Direct Communication when the UE attempts to use a carrier frequency configured for ProSe Direct Communications, is considered a configuration error. Therefore, the UE does not transmit on that frequency to avoid interference to the network.

-	The ProSe Direct Communication is only specified for NR.

NOTE 5:	It is out of scope of the present specification to define how the UE can locate itself in a specific Geographical Area. When the UE is in coverage of a 3GPP RAT, it can for example, use information derived from the serving PLMN. When the UE is not in coverage of a 3GPP RAT, it can use other techniques, e.g. Global Navigation Satellite System (GNSS). User provided location is not a valid input.

/*******************Next Change****************/

### 4.3.3	PCF

In addition to the functions defined in TS 23.501 [4] and TS 23.503 [9], the PCF includes functions to provision the UE with necessary policies and parameters to use ProSe direct discovery and communication, as part of the UE Policy information as defined in TS 23.503 [9] clause 4.2.2:

-	Direct communication path selection policy, between PC5 and Uu, is provided to the UE to indicate which path(s) is/are preferred for all or specific ProSe services (i.e. PC5 preferred, Uu preferred or no preference indicated).

-	PC5 usage reporting configuration.

-	Authorization policy and parameters for ProSe Direct Discovery and Communication.

The PCF may update the 5G ProSe policy and parameters to the UE under certain conditions.

/*******************End of Change****************/