SA WG2 Meeting #144e	S2-2102433r01

12 – 16 April 2021, e-Meeting	(revision of S2-200xxxx)

Source:	Qualcomm Incorporated

Title:	Discussion of Mobility Registration Update to support NR Satellite Access

Document for:	Discussion / Approval

Agenda Item:	8.11

Work Item / Release:	5GSAT_ARCH / Rel-17

Abstract of the contribution: Discusses impacts of TAC update in a radio cell for NR Satellite Access.

# 1. Introduction

Broadcast of one TAC per PLMN in a radio cell (in SIB1) is referred to as “hard TAC update” or “hard switch” and aligns with support of TACs with TN. Broadcast of more than one TAC per PLMN in a radio cell is referred to as “soft TAC update” or “soft switch”. These were originally identified by RAN in TR 38.821 [1] (see clause 7.3.1.3.1). RAN2 has since agreed to support both of these as shown from an extract of the draft RAN2#113e meeting report [2] copied below.

Draft RAN2#113e Meeting Report [2]

Agreements: - In NTN, the UE determines the TA based on the broadcast information (the use of other information is not excluded). In any case RAN2 will not go in a different direction than other groups - In NTN, the network may broadcast more than one TACs per PLMN in a cell, which is to up to network implementation

Figure 1 – Illustration of Hard TAC Update

Figure 1 illustrates hard TAC update by reference to a satellite and radio cell moving from left to right (e.g. west to east) over two fixed geo area TAs, TA1 and TA2. In this example, at time T1, more of the radio cell coverage area is over TA1 than TA2. Accordingly, the radio cell indicates TAC 1 and not TAC 2 (e.g. in SIB1). At a later time T2, the satellite and the radio cell have moved such that more of the radio cell coverage area is now over TA 2 than TA 1. Accordingly, the radio cell can now indicate TAC2 and not TAC1. The switch from TAC1 to TAC2  will look like mobility to UEs accessing the radio cell (even for UEs which are stationary) and will trigger Registration Updates from those UEs whose registration areas do not include TAC2.

Observation 1	With hard TAC update, a radio cell indicates coverage of just one TAC. When the radio cell switches from one TAC indication to another, served UEs may need to perform a Mobility Registration Update, even if they are stationary. This procedure will not have any NAS impacts in the UE.

Figure 2 – Illustration of Soft TAC Update

Figure 2 illustrates the same scenario as in Figure 1 but with soft TAC update. At time T1 and at time T2, the radio cell indicates both TAC1 and TAC2, allowing access from UEs whose registration areas include at least one of TAC1 and TAC2 and avoiding a Registration update at time T2. At a later time T3 when the radio cell coverage area has moved completely out of TA1, the radio cell can just indicate TAC2 – and may also indicate other TACs (not shown in Figure 2) to the right of TA 2. For UEs whose registration areas do not include TAC2, this may trigger a mobility Registration update.

Observation 2	With soft TAC update, a radio cell can indicate more than one TAC. A radio cell can cease indicating a particular TAC once the radio cell coverage area no longer overlaps with the TA.

To support soft TAC update, UE NAS procedures may be changed to instigate a Mobility Registration Update when none of the TACs for an sPLMN belong to the UE Registration Area (RA). This will be a new impact to NAS procedures in a UE.

Observation 3	With soft TAC update, UE NAS procedures may be changed to instigate a Mobility Registration Update when none of the TACs for an sPLMN belong to the UE Registration Area (RA). This will be a new impact to NAS procedures in a UE.

# 2. Consequences of TAC Update to Paging and NAS Procedures

For both hard TAC update and soft TAC update, an AMF will continue to page a UE in the UE Registration Area (RA). The NG-RAN can determine which radio cells currently indicate a TAC (for hard TAC update) or at least one TAC (for soft TAC update) in the RA and can send paging messages only in those radio cells. (Note that this does not preclude more precise paging based on a last known CGI for a UE, but may be needed when the more precise paging fails to reach a UE.)  If a UE always performs a Mobility Registration update when accessing a radio cell which indicates a TAC outside the RA (for hard TAC update) or which indicates TACs that are all outside the RA (for soft TAC update), the 5GC (AMF) can update the UE’s RA to include the new TAC(s) and thereby allow access by the UE to the radio cell. Paging of the UE can then continue to function correctly. As an example, a UE, whose RA includes TAC1 but not TAC2, which accesses the radio cell in Figure 1 at time T2, would need to perform a Registration update to enable the AMF to change the UE’s RA to include TAC2. Similarly, the same UE accessing the radio cell in Figure 2 would not need to perform a Registration at time T2 as the radio cell continues to indicate TAC1, but would need to perform a Registration at or sometime before time T3 when the radio cell ceases to indicate TAC1.

Observation 4	Both hard TAC update and soft TAC update enable reliable Paging if NG-RAN directs paging messages to radio cells indicating a TAC (for hard TAC update) or at least one TAC (for soft TAC update) in the UE’s RA.

One consequence of soft TAC update is that a UE, from location tracking point of view, may move into the fixed geographic area of a TA whose TAC is not part of the RA and not be aware of this. For example, in Figure 2, a UE may be inside TA1 at time T1 and move into TA2 at time T2. The UE would not be aware of this because the radio cell will continue to indicate both TAC1 and TAC2 at time T1 and time T2. The same may happen for hard TAC update in Figure 1 if a UE whose RA includes TAC1 but not TAC2 were to move from TA1 into TA2 at time T1 as the radio cell indicates TAC1 at this time.

When a UE initiates a NAS procedure with a 5GC (e.g. a NAS Service Request), the  NG-RAN will need to decide what to indicate as part of NGAP User Location Information (ULI). If the NG-RAN indicates the TAC of the correct TA according to configured geographic definitions of TAs and the current geographic location of the UE, there may be problems for both hard TAC update and soft TAC update if the correct TA for the UE indicated in the ULI is not matching any of the TACs for which the UE is registered i.e. the RA of NAS. This may be seen as an anomaly (a “TA anomaly”) by the 5GC (e.g. AMF) because the UE is meant to perform a Registration update with the 5GC whenever it moves outside of the RA. As an example, if UE charging is based on the RA and the RA is not updated based on the current TA, there may be a charging error.

Observation 5	For both hard TAC update and soft TAC update, a UE can enter the fixed geographic area of a TA that is not part of the RA and remain unaware of this (e.g. not perform a Registration update). This may lead to a “TA anomaly” where the UE initiates a NAS procedure and the ULI delivered to the 5GC by NG-RAN indicates a TAC that is not one of the TACs in the RA of the UE in NAS.

The TA anomaly can be removed (at least superficially) for hard TAC update by “fudging” the TAC in the ULI. Instead of including the TAC corresponding to the geographically correct TA in the ULI in which the UE is geographically located, the NG-RAN can instead indicate the TAC which was indicated to the UE by the radio cell in SIB1.  This geographically incorrect TAC will at least match the TAC seen by the UE in the radio cell and will be part of the RA. Thus, the TA anomaly will not occur (or at least will not be observed), although the 5GC (AMF) will receive an indication of a geographically incorrect TAC. This geographically incorrect TAC indication will have other consequences, since the ULI will also need to include a CGI indicating a fixed cell in which the UE is located, in order to support services like routing of an emergency call to a PSAP, charging and possibly precise paging of a UE in a last known fixed cell. So the AMF will receive a CGI and TAC in the ULI where the CGI is not part of the TA indicated by the TAC, which can be considered as a “TAC-CGI mismatch anomaly”. In some implementations, this anomaly could trigger an error either in real time or later when the TAC and CGI are processed offline. Thus, even for hard TAC update, it may be preferable to deal with the “TA anomaly” rather than the “TAC-CGI mismatch anomaly”.

Observation 6	For hard TAC update but not soft TAC update, the TA anomaly can be removed if the NG-RAN indicates the same TAC to the 5GC as is indicated to the UE in the radio cell. However, this may lead to a different “TAC-CGI mismatch anomaly” which may be worse than the TA anomaly. Therefore, both hard and soft TAC update may benefit from a more robust resolution for the TA anomaly.

# 3. Proposal

The following observations have been made.

Observation 1	With hard TAC update, a radio cell indicates coverage of just one TAC. When the radio cell switches from one TAC indication to another, served UEs may need to perform a Mobility Registration Update, even if they are stationary. This procedure will not have any NAS impacts in the UE.

Observation 2	With soft TAC update, a radio cell can indicate more than one TAC. A radio cell can cease indicating a particular TAC once the radio cell coverage area no longer overlaps with the TA.

Observation 3	With soft TAC update, UE NAS procedures may be changed to instigate a Mobility Registration Update when none of the TACs for an sPLMN belong to the UE Registration Area (RA). This will be a new impact to NAS procedures in a UE.

Observation 4	Both hard TAC update and soft TAC update enable reliable Paging if NG-RAN directs paging messages to radio cells indicating a TAC (for hard TAC update) or at least one TAC (for soft TAC update) in the UE’s RA.

Observation 5	For both hard TAC update and soft TAC update, a UE can enter the fixed geographic area of a TA that is not part of the RA and remain unaware of this (e.g. not perform a Registration update). This may lead to a “TA anomaly” where the UE initiates a NAS procedure and the ULI delivered to the 5GC by NG-RAN indicates a TAC that is not one of the TACs in the RA of the UE in NAS.

Observation 6	For hard TAC update but not soft TAC update, the TA anomaly can be removed if the NG-RAN indicates the same TAC to the 5GC as is indicated to the UE in the radio cell. However, this may lead to a different “TAC-CGI mismatch anomaly” which may be worse than the TA anomaly. Therefore, both hard and soft TAC update may benefit from a more robust resolution for the TA anomaly.

Based on these, the following proposal is made.

Proposal 1	Consider whether the new NAS impact to support soft TAC update in Observation 3 is acceptable and whether the anomalies described in Observations 5 and 6 are in scope for SA2 to resolve.

# 4. References

[1]	3GPP TR 38.821: “Solutions for NR to support non-terrestrial networks (NTN)”.

[2]	Draft RAN2#113e Meeting Report: “Report of 3GPP TSG RAN WG2 meeting #113-e”,  ftp://ftp.3gpp.org/tsg_ran/WG2_RL2/TSGR2_113-e/Report/Draft_R2-113-e_Meeting_Report_v1.zip.

[3]	3GPP TR 38.821: “Solutions for NR to support non-terrestrial networks (NTN)”.

[4]	3GPP TR 23.737: “Study on architecture aspects for using satellite access in 5G”.

In these examples and for their evaluation in section 2, fixed tracking areas are asssumed similar to those used for TN and as agreed in both TR 38.821 (clause 7.3.1.3.3) [3] and TR 23.737 (clause 8.2) [4]. This also aligns with ongoing work in RAN2 and RAN3.