SA WG2 Meeting #144E (e-meeting)	S2-2102440r01

April 12 – 16, 2021, Elbonia

Source:	CATT

Title:	MB-SMF services

Document for:	Approval

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract of the contribution: This contribution proposes definitions of services provided by the MB-SMF to support MBS.

- Introduction

The MB-SMF provides services related to multicast and broadcast session management to support MBS. This contribution proposes definitions of those services.

# 2. Proposal

It is proposed to include the following changes in TS 23.247.

******************** 1st Change (all new text)********************

## 9.X	MB-SMF services

### 9.x.1	General

The following table shows the MB-SMF Services and MB-SMF Service Operations.

Table 9.x.1-1: NF services provided by the MB-SMF

| Service Name | Service Operations | Operation Semantics | Example Consumer(s) |
| --- | --- | --- | --- |
| Nmbsmf_MBSession | Create | Request/Response | MBSF, NEF, AF |
|  | Update | Request/Response | MBSF, NEF, AF |
|  | Release | Request/Response | MBSF, NEF, AF |

### 9.x.2	Nmbsmf_MBSession Service

#### 9.x.2.1	General

#### Editor’s note: What service operation is used for SMF creating shared tunnel between PSA UPF and MB-UPF is FFS.

Service description: This service operates on the multicast and broadcast sessions. The following are the key functionalities of this NF service:

-	(between MBSF/NEF and MB-SMF) Creation/Modification/Activation/Deactivation/Release of multicast sessions;

-	(between MBSF/NEF and MB-SMF) Creation/Modification/Start/Stop of broadcast sessions;

#### 9.x.2.2	Nmbsmf_MBSession_Create service operation

#### Editor’s note: What exact parameters are needed are FFS.

Service operation name: Nmbsmf_MBSession_Create.

Description: Create a new multicast session or broadcast session in the MB-SMF.

Input, Required: MBSF ID or NEF ID, DNN, Serving Network (PLMN ID, or PLMN ID and NID, see clause 5.18 of TS 23.501 [5]).

Input, Optional: S-NSSAI, MBS Session ID (source specific multicast address or TMGI), Subscription for MBS Session Status Notification, AN type, PCF ID, PCF Group ID.

Output, Required: Result Indication, and if successful MBS Session Context ID.

Output, Optional: MBS Session ID, S-NSSAI, Cause, MB-UPF tunnel info, QoS Profile(s), Session-AMBR, QoS Rule(s), QoS Flow level QoS parameters if any for the QoS Flow(s) associated with the QoS rule(s).

#### 9.x.2.3	Nmbsmf_MBSession_Update service operation

Editor’s note: What exact parameters are needed are FFS.

Service operation name: Nmbsmf_MBSession_Update.

Description: Update the established multicast session or broadcast session.

Input, Required: MBS Session ID.

Input, Optional: QoS Profile(s), Session-AMBR, QoS Rule(s), QoS Flow level QoS parameters if any for the QoS Flow(s) associated with the QoS rule(s).

Output, Required: Result Indication.

Output, Optional: Cause.

#### 9.x.2.4	Nmbsmf_MBSession_Release service operation

#### Editor’s note: What exact parameters are needed are FFS.

Service operation name: Nmbsmf_MBSession_Release.

Description: Release the multicast session or broadcast session.

Input, Required: MBS Session ID.

Input, Optional: None.

Output, Required: Result Indication.

Output, Optional: Cause.

******************** End of Change ********************