SA WG2 Meeting #S2-144E	S2-2102448r01

April 12 - 16, 2021, Electronic, Elbonia

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0284 | rev | 01 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ML Model Provisioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the ML Model Subscribe/Unsubscribe procedure, the "NWDAF service consumer" (and even "NWDAF 1") is used to represent "model consumer NWDAF", and "NWDAF containing MTLF" is used to represent "model provider NWDAF". However, NWDAF service consumer may also be understood as an NF other than NWDAF which consumes the services provided by the NWDAF. And an NWDAF (e.g. an NWDAF (MTLF+AnLF)) can also be a provider of this service to other NWDAF(s). To avoid obscurity, it is proposed to replace them by "model consumer NWDAF" and "model provider NWDAF". |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change the "NWDAF service consumer" to "model consumer NWDAF", and "NWDAF containing MTLF" to "model provider NWDAF" in the ML Model Subscribe/Unsubscribe procedure. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Obscurity in ML model provisioning service consumer and provider. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2A.1 |  |  |  |  |  |  |  |  |
| “ |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

******************** 1st Change ********************

### 6.2A.1	ML Model Subscribe/Unsubscribe

The procedure in Figure 6.2A.1-1 is used by an model consumer NWDAF, i.e. an NWDAF (AnLF), to subscribe/unsubscribe from an model provider NWDAF, i.e. an NWDAF containing MTLF, to be notified when ML model information on the related Analytics becomes available, using Nnwdaf_MLModelProvision Services as defined in clause 7.5. The ML model information is used by an NWDAF containing AnLF to derive analytics. The service is also used by an NWDAF to modify existing ML Model Subscription(s). An NWDAF (e.g. an NWDAF (MTLF+AnLF)) can be at the same time a consumer of this service provided by other NWDAF(s) and a provider of this service to other NWDAF(s).

Figure 6.2A.1-1: ML Model for analytics subscribe/unsubscribe

NOTE:	In Figure 6.2A.1-1 an NWDAF service consumer may host both an Analytics Logical Function and a Model Training Logical Function.

1.	The model consumer NWDAF (i.e. an NWDAF (AnLF)) subscribes to, modifies, or cancels subscription for a (set of) ML Model(s) associated with a (set of) Analytics ID(s) by invoking the Nnwdaf_MLModelProvision_Subscribe / Nnwdaf_MLModelProvision_Unsubscribe service operation. The parameters that can be provided by the model consumer NWDAF are listed in clause 6.2A.2.

When a subscription to an ML model information for the Analytics is received, the model provider NWDAF may:

-	determine whether an existing trained ML Model can be used for the subscription; or

-	determine whether triggering further training for an existing trained ML models is needed for the subscription; or

-	determine to generate an initial ML model for the subscription.

If the model provider NWDAF generates an initial ML Model or determines that further training is needed, this NWDAF may initiate data collection from NFs, DCCF, or OAM as described in clause 6.2, required for the training of the ML model.

If the service invocation is for a subscription modification or subscription cancelation, the model consumer NWDAF includes an identifier (Subscription Correlation ID) to be modified in the invocation of Nnwdaf_MLModelProvision_Subscribe.

2.	If the model consumer NWDAF subscribes to a (set of) ML model(s) associated to a (set of) Analytics ID(s), the model provider NWDAF notifies the model consumer NWDAF with the ML model information (containing the file address of the ML model) by invoking Nnwdaf_MLModelProvision_Notify service operation. The content of ML model information that can be provided by the model provider NWDAF is specified in clause 6.2A.2.

The model provider NWDAF also invokes the Nnwdaf_MLModelProvision_Notify service operation to notify the availability of a re-trained ML model, when the model provider NWDAF determines that the previously provided trained ML Model required re-training at step 1.

******************** End of Change ********************