3GPP TSG-WG SA2 Meeting #144E e-meeting  	S2-2102476r01

# Elbonia, April 12 – 16, 2021	(revision of S2-210xxxx)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2760 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarify the Supported Analytics Delay |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During the SA2#143E meeting, it is approved that an NWDAF could register the Supported Analytics Delay to help the Consumer NF discover a suitable NWDAF which fulfills the Consumer NF’s analytics delay requirement. However, on one hand, though it is stated that the determination of Supported Analytics Delay, and how the NWDAF avoid updating its Supported Analytics Delay in NRF frequently is NWDAF implementation specific, the 3GPP SA2 should at least give some clues that what what the Supported Analytics Delay includes. On the other hand, it is also unclear how the Consumer NF selects the target NWDAF from the candidate NWDAF list in which each candidate NWDAF has a Supported Analytics Delay. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Clarify the Supported Analytics Delay may include the data collection delay and/or inference delay; 2. Clarify the Consumer NF selects the target NWDAF by comparing the local Expected Analytics Delay and the Supported Analytics Delay per candidate NWDAF. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1. It is unclear what the Supported Analytics Delay includes; 2. It is unclear how the consumer NF selects the target NWDAF from the candidate NWDAF list in which each candidate NWDAF has a Supported Analytics Delay. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.11.3.1, 6.3.13 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

#### 6.2.6.2	NF profile

NF profile of NF instance maintained in an NRF includes the following information:

-	NF instance ID.

-	NF type.

-	PLMN ID.

-	Network Slice related Identifier(s) e.g. S-NSSAI, NSI ID.

-	FQDN or IP address of NF.

-	NF capacity information.

-	NF priority information.

NOTE 1:	This parameter is used for AMF selection, if applicable, as specified in clause 6.3.5. See clause 6.1.6.2.2 of TS 29.510 [58] for its detailed use.

-	NF Set ID.

-	NF Service Set ID of the NF service instance.

-	NF Specific Service authorization information.

-	if applicable, Names of supported services.

-	Endpoint Address(es) of instance(s) of each supported service.

-	Identification of stored data/information.

NOTE 2:	This is only applicable for a UDR profile. See applicable input parameters for Nnrf_NFManagement_NFRegister service operation in TS 23.502 [3] clause 5.2.7.2.2. This information applicability to other NF profiles is implementation specific.

-	Other service parameter, e.g. DNN or DNN list, notification endpoint for each type of notification that the NF service is interested in receiving.

-	Location information for the NF instance.

NOTE 3:	This information is operator specific. Examples of such information can be geographical location, data centre.

-	TAI(s).

-	NF load information.

-	Routing Indicator, for UDM and AUSF.

-	One or more GUAMI(s), in the case of AMF.

-	SMF area identity(ies) in the case of UPF.

-	UDM Group ID, range(s) of SUPIs, range(s) of GPSIs, range(s) of internal group identifiers, range(s) of external group identifiers for UDM.

-	UDR Group ID, range(s) of SUPIs, range(s) of GPSIs, range(s) of external group identifiers for UDR.

-	AUSF Group ID, range(s) of SUPIs for AUSF.

-	PCF Group ID, range(s) of SUPIs for PCF.

-	HSS Group ID, set(s) of IMPIs, set(s) of IMPU, set(s) of IMSIs, set(s) of PSIs, set(s) of MSISDN for HSS.

-	Supported Analytics ID(s), possibly per service, NWDAF Serving Area information (i.e. list of TAIs for which the NWDAF can provide services and/or data), Supported Analytics Delay per Analytics ID, NF types of the NF data sources, NF Set IDs of the NF data sources, if available, in the case of NWDAF.

NOTE 4:	The NWDAF's Serving Area information is common to all its supported Analytics IDs.

NOTE 5:	The Analytics IDs supported by the NWDAF may be associated with a Supported Analytics Delay. i.e. the Analytics report can be generated (including data collection delay and inference delay) in less than or equal to the Supported Analytics Delay.

NOTE 6:	The determination of Supported Analytics Delay, and how the NWDAF avoid updating its Supported Analytics Delay in NRF frequently is NWDAF implementation specific.

-	Event ID(s) supported by AFs, in the case of NEF.

-	Application Identifier(s) supported by AFs, in the case of NEF.

-	Range(s) of External Identifiers, or range(s) of External Group Identifiers, or the domain names served by the NEF, in the case of NEF.

NOTE 7:	This is applicable when NEF exposes AF information for analytics purpose as detailed in TS 23.288 [86].

NOTE 8:	It is expected service authorization information is usually provided by OA&M system, and it can also be included in the NF profile in the case that e.g. an NF instance has an exceptional service authorization information.

NOTE 9:	The NRF may store a mapping between UDM Group ID and SUPI(s), UDR Group ID and SUPI(s), AUSF Group ID and SUPI(s) and PCF Group ID and SUPI(s), to enable discovery of UDM, UDR, AUSF and PCF using SUPI, SUPI ranges as specified in clause 6.3 or interact with UDR to resolve the UDM Group ID/UDR Group ID/AUSF Group ID/PCF Group ID based on UE identity, e.g. SUPI (see clause 6.3.1 for details).

-	IP domain list as described in clause 6.1.6.2.21 of TS 29.510 [58], Range(s) of (UE) IPv4 addresses or Range(s) of (UE) IPv6 prefixes, in the case of BSF.

-	SCP Domain the NF belongs to.

-	DCCF Serving Area information, NF types of the data sources, NF Set IDs of the data sources, in the case of DCCF.

-	Supported DNAI list, in the case of SMF.

# * * * * End of changes * * * *