3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102489r05

Elbonia, 12 – 16 April, 2021	(revision of S2-2100115r17)

Source:	Huawei, HiSilicon, Nokia, Nokia Shanghai Bell, Tencent

Title:	General descriptions and reference architectures for Edge Computing

Document for:	Approval

Agenda Item:	8.3

Work Item / Release:	eEDGE_5GC / Rel-17

Abstract: This contribution introduces general descriptions and reference architectures for Edge Computing.

# 1. Introduction

This proposal describes reference architectures for Edge Computing.

This paper is based on the postponed S2-2100115r17 from SA2 #143E meeting.

An term "Local part of the DN" is added to resolve the EN:

Editor’s Note: Throughout the document “Local DN part” is improperly used to mean “Local part of the DN” . A suitable wording is FFS

# 2. Text Proposal

It is proposed to capture the following changes vs. TS 23.548.

# * * * * First change * * * *

## 3.1	Terms

For the purposes of the present document, the terms given in TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [1].

Edge Application Server: An Application Server resident in the Edge Hosting Environment.

Edge Hosting Environment: An environment providing support required for Edge Application Server's execution.

Local part of the DN: The set of network entities  of a DN that are deployed locally. The local access to the DN provides access to the local part of the DN.

## 3.2	Abbreviations

For the purposes of the present document, the abbreviations given in TR 21.905 [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in TR 21.905 [1].

C-DNS	Central DNS

C-NEF	Central NEF

C-PSA UPF	Central PSA UPF

EAS	Edge Application Server

EASDF	Edge Application Server Discovery Function

EHE	Edge Hosting Environment

L-DN	Local part of the DN

L-DNS	Local DNS

L-NEF	Local NEF

L-PSA UPF	Local PSA UPF

# * * * * First change, all new texts * * * *

## 4.2	Reference Architectures for Supporting Edge Computing

The reference architectures for supporting Edge Computing are based on the reference architectures specified in clause 4.2 of TS 23.501[2]. The following reference architectures are further depicting the relationship between the 5GS and EHE for non-roaming and LBO roaming scenarios.

Figure 4.2-1 depicts 5GS architecture for non-roaming scenario supporting Edge Computing with UL CL/BP.

NRF

PCF

AF

AMF

SMF

NEF

UE

AN

UPF

(UL CL/ BP)

UPF

(L-PSA)

UPF

(C-PSA)

Local part of the DN

Central DN

EAS

Nnrf

Npcf

Namf

Nsmf

N1

N2

N3

N4

N4

N4

N9

N6

N6

EASDF

Naf

Nnef

Neasdf

UDM

Nudm

Figure 4.2-1: 5GS providing access to EAS with UL CL/BP for non-roaming scenario

Figure 4.2-2 depicts 5GS architecture for non-roaming scenario supporting Edge Computing without UL CL/BP.

NRF

PCF

AF

AMF

SMF

NEF

UE

AN

UPF

(PSA)

Local part of the DN

EAS

Nnrf

Npcf

Namf

Nsmf

N1

N2

N3

N4

N6

EASDF

Naf

Nnef

Neasdf

UDM

Nudm

Figure 4.2-2: 5GS providing access to EAS without UL CL/BP for non-roaming scenario

Figure 4.2-3 depicts 5GS architecture for LBO roaming scenario supporting Edge Computing with UL CL/BP.

NRF

PCF

AF

AMF

SMF

NEF

UE

AN

UPF

(UL CL/ BP)

UPF

(L-PSA)

UPF

(C-PSA)

Local part of the DN

Central DN

EAS

Nnrf

Npcf

Namf

Nsmf

N1

N2

N3

N4

N4

N4

N9

N6

N6

EASDF

Naf

Nnef

Neasdf

PCF

Npcf

UDM

Nudm

VPLMN

HPLMN

Figure 4.2-3: 5GS providing access to EAS with UL CL/BP for LBO roaming scenario

Figure 4.2-4 depicts 5GS architecture for LBO roaming scenario supporting Edge Computing without UL CL/BP.

NRF

PCF

AF

AMF

SMF

NEF

UE

AN

UPF

(PSA)

Local part of the DN

EAS

Nnrf

Npcf

Namf

Nsmf

N1

N2

N3

N4

N6

EASDF

Naf

Nnef

Neasdf

PCF

Npcf

UDM

Nudm

VPLMN

HPLMN

Figure 4.2-4: 5GS providing access to EAS without UL CL/BP for LBO roaming scenario

NOTE 1:	Only some of the 5GS NFs are shown in the above reference architecture figures. In the above figures, the split between the UPF acting as UL CL/BP and the UPF acting as local PSA is illustrative.

NOTE 2:	Only the control plane of EASDF is depicted in the figure, the user plane between the EASDF and the UPF (i.e. over which the DNS messages are exchanged) is part of N6. Additionally, the EADSF may have direct connectivity with the local parts of one or more Data Networks.

# * * * * End of changes * * * *