3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102500r01

Elbonia, 12 – 16 April, 2021	(revision of S2-210xxxx)

Source:	Huawei, HiSilicon

Title:	Add General Description for Edge Relocation

Document for:	Approval

Agenda Item: 	8.3

Work Item / Release:	eEDGE_5GC/Rel-17

Abstract: This contribution proposes to add the general description for edge relocation.

# 1. Discussion

This contribution proposes to add the general description for edge relocation.

# 2. Text Proposal

It is proposed to capture the following changes into TS 23.548.

# * * * * First change * * * *

## 6.3	Edge Relocation

### 6.3.1	General

Edge Relocation refers to the procedures supporting EAS changes and/or PSA UPF relocation.

Edge Relocation may be triggered by the EHE (e.g. due to the load balance between EAS instances) or by the network (e.g. due to the UE mobility).

During Edge Relocation, the user plane path may be re-configured to keep optimization. This may be done by PDU Session re-establishment using SSC mode 2/3 mechanisms or Local PSA UPF relocation using UL CL and BP mechanisms. The corresponding procedures are defined in TS 23.501 [2] and TS 23.502 [3].

During Edge Relocation, the UE may need to re-discovery a new EAS and establish the connectivity to the new EAS to continue the service. The re-discovery of EAS is specified in clause 6.2.

Edge Relocation may result in AF relocation, for example, as part of initial PDU session establishment, a central AF may be involved, however, due to Edge relocation another AF serving the Edge Applications is selected.

The trigger of Edge relocation by the network is specified in clause 4.3.6.3 of TS 23.502 [3].

This clause further describes the following procedures:

-	Edge Relocation triggered by AF

-	Edge Relocation using EAS IP replacement

-	Simultaneous connectivity for Source and Target EASs

-	Packet buffering for low Packet Loss

-	Edge relocation considering User Plane Latency Requirements.

# * * * * End of changes * * * *