3GPP TSG-SA WG2 Meeting #144E 	S2-2102564r02

Elbonia, 12 - 16 April, 2021

Source:	Huawei, HiSilicon, Intel, Lenovo, Motorola Mobility, Samsung

Title:	Impacted EAS related to the EAS Rediscovery

Document for:	Agreement

Agenda Item: 	8.3

Work Item / Release:	eEDGE_5G /Rel-17

Abstract of the contribution: This contribution resolves editor’s notes in EAS rediscovery procedure.

- Discussion

There are three EN in the EAS rediscovery procedure. This paper try to resolve those issues.

Editor's note:	It is FFS whether the same procedure is needed for change of SSC mode 3 PSA with IPv6 multi-homing scenario. For this scenario, the UE's IPv6 prefix can be released with the existing procedure, therefore, the UE is aware when to release DNS cache if needed.

For SSC mode 1/ SSC mode 3 with IPv6 multi-homing scenario,

- For the case of the L-PSA removal, the SMF need reconfigure the IPV6 prefix@L-PSA information as described in TS23.502 clause 4.3.5. The SMF notifies the UE to stop using the old IPv6 prefix. Then the associated EAS is expected to be reallocated and need be rediscovered. It is possible for the UE to release DNS cache associated with the old L-PSA when an IPv6 prefix related to the L-PSA is not used anymore.

- For the case of the L-PSA insertion, the SMF notify the UE on which destination IP address a new IPV6 prefix is to be used. However from UE view, it may be not aware which EAS can be reallocated to that local DN and EAS rediscovery need be triggered. If the UE is not able to use the new destination IP address, the new IPV6 prefix can not be used.

Also in the UL-CL case using the routing information mechanism can not be used. Thus it is suggested in both case (UL-CL or BP) case one unify mechanism is adopted, i.e. the notification of impacted EAS via the NAS mechanism.

C1: it is proposed that the impacted EAS via NAS notification applies to BP case as well.

Editor's note:	Which information that is used by SMF to compose the optional impact field and how that is provisioned to SMF is FFS.

Editor's note:	It is FFS for the detail information provided by the AF.

As described in clause 6.2.3, the SMF is a support to do the UL-CL insertion per the received EAS IP address information. So the SMF has already the information of relation between IP range(s) and local DN. Hence, it is possible for the SMF to include in the impact field with the IP range(s) of the local DN to which the L-PSA has been released.

In case a new L-PSA (i.e. DNAI) is inserted, if the SMF is provisioned with FQDN(s) corresponding to the DNAI, the SMF can include the FQDN(s) corresponding to the DNAI of the new L-PSA in the EAS rediscovery indication to UE.

NOTE:  	Providing the FQDN(s) associated the DNAI is expected to be introduced for EAS discovery over Session Breakout Connectivity Model. This is used by the SMF to be aware on how to insert the ECS field.

In case the EAS rediscovery is triggered by AF, the AF indicates which EAS is impacted. Based on this, the SMF knows which FQDN need be updated.

C2: it proposes to add IP range(s) and FQDN(s) as parameters in Impact field.

Editor's note:	Whether interaction between the 5GS and application layer DNS cache for EAS rediscovery is required or not is FFS.

C3: It is proposed that the interaction between the 5GS and application layer DNS cache for EAS rediscovery is left for implementation.

- Proposal

It is proposed to update solution #43 as following in TS 23. 548:

**************************** 1st Change ***************************

6.2.3.3	EAS Re-discovery Procedure at Edge Relocation

For PDU Session with Session Breakout connectivity, the UE may need to re-discover the EAS after the insertion/change/removal of an L-PSA.

This procedure is used by the SMF to trigger the EAS rediscovery procedure when a new connection to EAS need to be established. It applies to both Session Breakout using ULCL and Session Breakout using BP.

Editor's note:	It is FFS whether the same procedure is needed for change of SSC mode 3 PSA with IPv6 multi-homing scenario. For this scenario, the UE's IPv6 prefix can be released with the existing procedure, therefore, the UE is aware when to release DNS cache if needed.

Figure 6.2.3.3-1: EAS re-discovery procedure at Edge relocation

During a previous EAS Discovery procedure on this PDU Session the UE may have cached EAS information (i.e. EAS IP address corresponding to an EAS FQDN) locally, e.g. during the previous connection with the EAS (for more information see Annex C UE considerations for EAS (re)discovery).

1a.	Due to the UE mobility, the SMF triggers L-PSA insertion, change or removal for the PDU Session. The insertion, change or removal of L-PSA triggers EAS rediscovery.

1b. The AF triggers EAS relocation e.g. due to EAS load balance or maintenance, etc, and informs the SMF the related information as described in clause 4.3.6 AF influence on traffic routing procedure in TS 23.502 [3].

2.	The SMF determines the impacted EAS(s) which need be rediscovered as the following:

- If an L-PSA is inserted/relocated/removed, the SMF determines the impacted EAS, which is associated with the local DN to be inserted, relocated or removed and identified by FQDN(s) or IP range(s), based on the association between FQDN(s)/IP range(s) and DNAI received from AF via AF influenced traffic steering enforcement control information in the PCC rules or local configuration on the local DN.

- For AF triggered EAS rediscovery, the AF may indicate the EAS rediscovery for the impacted applications, which are identified by FQDN(s), to the SMF via the AF influence on traffic routing procedure.

If the UE has indicated to SMF that it supports to do the EAS rediscovery per the indication from network, the SMF executes the network requested PDU Session Modification procedure from the step 3b-11b as defined in clause 4.3.3.2 TS 23.502 [3].

The SMF sends PDU Session Modification Command (EAS rediscovery indication, [impact field]) to UE. The impact field is used to identify which EAS(s) need to be rediscovered. The impact field includes the local DN information corresponding to the impacted EAS(s), identified by FQDN(s) or IP address range(s) of the local DN. If the impact field is not included, it means all EAS(s) associated with this PDU Session need to be rediscovered. The SMF may chooses a new local DNS server or EASDF, and configures it to the UE as new DNS server (Option C in clause 6.2.3.2.3). Otherwise the UE uses existing DNS server for EAS rediscovery.

For the following connection with the EAS(s) for which the EAS rediscovery need be executed per the received EAS rediscovery indication and impact field, the UE does not use the old EAS information stored locally. Instead it triggers EAS discovery procedure to get new EAS information as defined in clause 6.2.3.2.

NOTE 1:	The active connection(s) between the UE and the EAS(s) are not impacted.

Editor's note:	Whether interaction between the 5GS and application layer DNS cache for EAS rediscovery is required or not is FFS.

********************************* End Change ****************************