3GPP TSG-SA WG2 Meeting #144E 	S2-2102565r02

Elbonia, 12 - 16 April, 2021

Source:	Huawei, HiSilicon

Title:	ECS Address provision clarification

Document for:	Agreement

Agenda Item: 	8.3

Work Item / Release:	eEDGE_5GC /Rel-17

Abstract of the contribution: This contribution clarify the ECS address provision in case ECS is deployed by operator.

- Discussion

It is possible that the ECS is deployed by operator. For this case it is unclear how it is provided to UE. There are three possible options to provide ECS Address information to the SMF,

- Configured at the SMF, similar as the provision by the 3rd party.

- Provision as the UE subscription data, similar as the provision by the 3rd party.

- NRF query based mechanism. The ECS Address Configuration Information is registered in NRF by the operator. The OAM can do the registration. Using the NRF can simplify the network configuration at all SMF, i.e. the operator does not need to configure each ECS Address Configuration Information into each SMF. In addition, if the ECS Address Configuration Information is different for different locations, registration in NRF will make it very easy to get correct ECS Address Configuration Information based on UE’s location. Hence, it is proposed to clarify that the ECS Address Configuration information can be retrieved from NRF. This mechanism is also used for the PCSCF discovery before.

- Proposal

It is proposed to clarify how the ECS address configuration is provided to the UE in case ECS is deployed by operator.

**************************** 1st Change ***************************

### 6.5.2	ECS Address Provisioning

6.5.2.0	General

Editor's note:	This clause describes here the procedure agreed in conclusions in 9.1.3.

If the UE hosts an EEC and supports transferring the ECS address received from the 5GC to the EEC, the UE indicates in the PCO at PDU Session establishment or modification that it supports the ability to receive ECS address(es) via NAS and to transfer the ECS Address(es) to the EEC(s).

The ECS Address Configuration Information consists of one or more FQDN(s) and/or IP Address(es) of Edge Configuration Server(s). As described in clause 4.3.2 in TS 23.502 [3], if the UE supports the ability to receive ECS Address Configuration Information via NAS and to transfer the ECS Address(es) to the EEC(s), the UE may receive ECS Address Configuration Information from the SMF via PCO during PDU Session Establishment and/or during PDU Session modification procedures.

The SMF may derive the ECS Address Configuration Information based on local configuration, ECS profile retrieved from NRF, the UE's location, and/or UE subscription information. The SMF may decide to send updated ECS Address Configuration Information to the UE based on locally configured policy, updated UE subscription information, or a change of UE location. The PDU Session Modification procedure is used to send updated ECS Address Configuration Information to the UE as described in clause 4.3.3 in TS 23.502 [3].

NOTE:	In home routed sessions, the ECS Address Configuration Information comes from the H-SMF.

As described in TS 23.558 [5], the Edge Configuration Server can be deployed in a 3rd party domain by a service provider or can be deployed in the MNO domain.

6.5.2.1	ECS Address Provisioning by a 3rd Party AF

. If the ECS is deployed in a 3rd party domain by a service provider, a 3rd party AF can use Nnef_ParameterProvision to provide, update, or delete ECS Address Configuration Information for a UE (See TS 23.502 [3], clause 4.15.6.2).

When the AF uses Nnef_ParameterProvision to send a new ECS Address Configuration Information to the UDM for a UE (e.g. because on Application layer activity, change of UE location, etc.), the UDM may notify the SMF of the updated ECS Address Configuration Information and the new ECS Address Configuration Information will be sent to the UE in a PDU Session Modification procedure.

Editor's note:	It is FFS for multiple ECS address configuration information.

Editor's note:	It is FFS whether the 3rd party AF would know the GPSI; the possibility to configure the ECS address for all UE in one operation should be considered.

Editor's note:	Potential signalling overload is FFS When the AF uses Nnef_ParameterProvision to send a new ECS Address Information to the UDM for many UE.

6.5.2.2	ECS Address Provisioning in case of ECS deployed in MNO domain

. If the ECS is deployed in the MNO domain, per operator’s policy an ECS profile may be registered with the NRF and the ECS Profile may be used to derive the ECS Address Configuration Information to be sent to the UE.

.

********************************* End Change ****************************