3GPP TSG-WG SA2 Meeting #144E e-meeting  	S2-2102574r01

# Elbonia, Apr 12 – 16, 2021	(revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2669 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service operations of NSAC for Roaming UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The updates of Network Slice Admission Control function (NSACF) services to support Roaming UEs . |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is proposed add description related to NSAC for Roaming. |  |  |  |  |  |  |  |  |
| -- |  |  |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5GC cannot support NSAC in the roaming case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.11.1, 4.2.11.2, 4.2.11.z (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

4.2.11	Network Slice Admission Control Function (NSACF) procedures

4.2.11.1	General

The Network Slice Admission Control Function procedures are performed for an S-NSSAI which is subject to Network Slice Admission Control (NSAC) as described in TS 23.501 [2].

4.2.11.2	Number of UEs per network slice availability check and update procedure

The number of UEs per network slice availability check and update procedure is to update (i.e. increase or decrease) the number of UEs registered with a S-NSSAI which is subject to NSAC. The AMF is configured with the information indicating which network slice is subject to NSAC.

For roaming UEs, the AMF derives which network slice is subject to NSAC per UE subscription information.

Figure 4.2.11.2-1: Number of UEs per network slice availability check and update procedure

1.	The AMF triggers the Number of UEs per network slice availability check and update procedure to update the number of UEs registered with a network slice when a network slice subject to NSAC is included or removed from the Allowed NSSAI for a UE. The procedure is triggered in the following cases:

-	At UE Registration procedure, as per clause 4.2.2.2.2:

-	before the Registration Accept in step 21 if the EAC mode is active; or

-	after the Registration Accept message if the EAC mode is not active;

-	At UE Deregistration procedure, as per clause 4.2.2.3, after the Deregistration procedure is completed;

-	At UE Configuration Update procedure (which may result from NSSAA procedure):

-	before the UE Configuration Update message if the EAC mode is active; or

-	after the UE Configuration Update message if the EAC mode is not active;

NOTE:	Depending on the deployment, there may be different NSACF for different S-NSSAI subject to NSAC, and hence, during the registration, AMF triggers the Number of UEs per network slice availability check and update procedure to multiple NSACF.

Editor's note:	If one S-NSSAI the number of the UE/PDU session is managed by several different NSACF, it is FFS how to assure the AMF reaches the same NSACF and how to coordinate the number of UE among the NSACF(s)

2.	If the S-NSSAI is verified that it can be included in the Allowed NSSAI, and the S-NSSAI is subject to NSAC, the AMF sends Nnsacf_NumberOfUEsPerSliceAvailabilityCheckAndUpdate_Request message to the NSACF. The AMF includes in the message the UE ID, the S-NSSAI(s) for which the number of UEs registered per network slice update is required and the update flag which indicates whether the number of UEs registered with the S-NSSAI(s) is to be increased when the UE has gained registration to network slice(s) subject to NSAC or the number of UEs registered with the S-NSSAI(s) is to be decreased when the UE has deregistered from S-NSSAI(s) or could not renew its registration to an S-NSSAI subject to NSAC.

3.	The NSACF updates the current number of UEs registered for the S-NSSAI, i.e. increases or decrease the number of UEs registered per network slice based on the information provided by the AMF in the update flag parameter.

If the update flag parameter from the AMF indicates increase and the NSACF finds the UE ID is already in the list of UEs registered with the network slice, the number of the UEs registered with the network slice is not increased as the UE has already been counted as registered with the network slice. If the UE ID is not in the list of UE IDs registered with the network slice and the maximum number of UEs registered with the network slice has not been reached yet, the NSACF adds the UE ID in the list of UEs registered with the network slice and increases the current number of the UEs registered with the network slice. If the UE ID is not in the list of UEs registered with that S-NSSAI and the maximum number of UEs per network slice for that S-NSSAI has already been reached, then the NSACF returns a result parameter indicating that the maximum number of UEs registered with the network slice has been reached.

If the update flag parameter from the AMF indicates decrease, the NSACF removes the UE ID from the list of UEs registered with the network slice for each of the S-NSSAI(s) indicated in the request from the AMF and also the NSACF decreases the number of UEs per network slice that is maintained by the NSACF for each of these network slices.

4.	The NSACF returns the Nnsacf_NumberOfUEsPerSliceAvailabilityCheckAndUpdate_Response in which the NSACF includes the S-NSSAI(s) for which the maximum number of UEs per network slice has already been reached along with a result parameter indicating that the maximum number of UEs registered with the network slice has been reached.

If for all the S-NSSAI(s) the NSACF returned the maximum number of UEs per network slice has been reached, the AMF rejects the UE request for registration. In the Registration Reject message the AMF includes the rejected S-NSSAI(s) in the rejected NSSAI parameter, a reject cause set to 'maximum number of UEs per network slice reached' for each S-NSSAI and optionally a back-off timer.

Editor's note:	It is FFS whether the AMF shall send the Registration Reject message when for all the S-NSSAI(s) that the UE requested to register for and were subject to NSAC the NSACF returned the maximum number of UEs per network slice has been reached.

Otherwise, the AMF returns Registration Accept message in which the AMF includes the rejected S-NSSAI(s) in the rejected NSSAI list for which the NSACF has indicated that the maximum number of UEs per network slice has been reached, a reject cause set to 'maximum number of UEs per network slice reached' for each rejected S-NSSAI and optionally a back-off timer.

Editor's note:	It is FFS whether and how to restrict the signalling sent from the AMFs to the NSACF in case the maximum number of UEs has been reached for prolonged time.

# * * * * Second change * * * *

4.2.11.z	Network Slice Admission Control Support for Roaming

4.2.11.z.1	NSACF Support for Roaming by VPLMN

If NSAC for roaming UEs is managed and controlled by VPLMN, a local ‘maximum number of UEs per network slice per hPLMN' or ‘maximum number of PDU Sessions per network slice per HPLMN’ is allocated to the VPLMN per SLA agreement and stored in one responsible vNSACF of the S-NSSAI..

The maximum number of UEs registered with a network slice monitoring and enforcement is done in the vPLMN by the vNSACF as per the description in Figure 4.2.11.2-1 with the following differences:

- Step 2, In the Nnsacf_NumberOfUEsPerSliceAvailabilityCheckAndUpdate_Request service operation the subscribed S-NSSAI and its associated HPLMN ID are also sent to the vNSACF.

- Step 3, the local maximum number in the vNSACF is the one associated with the Subscribed S-NSSAI and HPLMN ID.

Editor’s note: It is FFS to define the interaction between vNSACF instances and the responsible vNSACF of the S-NSSAI per HPLMN to update the corresponding current number of UEs or PDU sessions.

Editor’s note: It is FFS whether and how NSAC can be supported for Roaming UEs by HPLMN, e.g., in case of HR scenario.

* * * * End of changes * * * *