3GPP TSG-WG SA2 Meeting #144E e-meeting	S2-2102587

Elbonia, April 12 – 16, 2021	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	Service-based interface for PC2a

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	5G_ProSe / Rel-17

Abstract: This contribution proposes to use a Service-based interface for PC2a and provides its service operations.

# 1. Introduction/Discussion

In clause 6.1.1.6 of TS 23.304, both PC2a interface and PC2 interface are supported for the interaction between 5G DDNMF and ProSe Application Server.

The 5G System architecture supports PC2a interface between 5G DDNMF and ProSe Application Server and optionally supports PC2 interface between 5G DDNMF and ProSe Application Server, to enable Proximity Services. See TS 23.501 [4] and TS 23.303 [3].

NOTE:	PC2 support between 5G DDNMF and ProSe Application Server is for backwards compatibility for early deployments using Diameter.

It is not that the PC2a interface is service based in 23.304 and this paper proposes to clarify that.

# 2. Text Proposal

It is proposed to capture the following changes vs. TS 23.304.

# * * * * First change * * * *

#### 6.1.1.6	5G DDNMF – ProSe Application Server

The 5G System architecture supports the service based PC2a interface between 5G DDNMF and ProSe Application Server and optionally supports PC2 interface between 5G DDNMF and ProSe Application Server, to enable Proximity Services. See TS 23.501 [4] and TS 23.303 [3].

NOTE:	PC2 support between 5G DDNMF and ProSe Application Server is for backwards compatibility for early deployments using Diameter.

# * * * * End of changes * * * *