SA WG2 Meeting #144E e-meeting	S2-2102613r04

Elbonia, April 12 – 16, 2021

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2796 | rev |  | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | AAA Server selection |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, Huawei, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  |  | Date: |  | 2021-03-29 |
| Category: | C |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Based on the following description in clause 5.30.2.9.2: - If the UDM instructs AUSF that primary authentication by a AAA server in a CH is required, the AUSF shall discover and select the AAA server, and then forward EAP messages to the AAA server which is acting as the EAP Server for the purpose of primary authentication. It is not clear how does UDM decide the primary authentication is performed by a AAA Server and how does AUSF discover and select AAA server. The CR is proposed to clarify that SUPI is used for UDM decision and AAA server selection based on TR conclusion. - The AMF discovers and selects the AUSF as described in TS 23.501 [4] clause 6.3.4 using the Home Network Identifier and Routing Indicator present in the SUCI provided by the UE. - The AUSF (e.g. enhanced AUSF or new NF) in SNPN selects the AAA hosted by the external entity. The discovery and selection functionality (within the AUSF) uses the domain name corresponds to the realm part (specified in the NAI format for SUPI in clauses 28.15.2 and 28.16.2) within Network specific identifier that identifies the external entity to perform discovery and selection of the AAA hosted by the external entity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | It is clarified that SUPI is used for UDM decision and AAA Server selection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear description. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.9.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

* * * * Start of Change * * * *

##### 5.30.2.9.2	Credentials Holder using AAA Server for primary authentication and authorization

The AUSF in SNPN may support primary authentication and authorization of UEs that use credentials from an AAA Server in a Credentials Holder (CH).

-	If the UDM decides that the primary authentication is performed by AAA Server in CH based on UE subscription data and UE’s SUPI, the UDM instructs AUSF that primary authentication by a AAA server in a CH is required, the AUSF shall discover and select the AAA server, and then forward EAP messages to the AAA server which is acting as the EAP Server for the purpose of primary authentication.  The AUSF uses the realm from SUPI to select AAA Server.

-	The SUPI is used to identify the UE during primary authentication and authorization towards the AAA sever. SUPI privacy is achieved according to existing methods in Annex I.5 of TS 33.501 [29].

-	After completing the primary authentication with the AAA server successfully, the AMF and SMF shall retrieve the UE subscription data from UDM using SUPI.

Figure 5.30.2.9.2-1 depicts the 5G System architecture for SNPN with Credentials Holder using AAA Server for primary authentication and authorization.

Figure 5.30.2.9.2-1: 5G System architecture with access to SNPN using credentials from Credentials Holder using AAA Server

Editor’s note:	 Whether an intermediate function is needed between the AUSF and the AAA-S is FFS.

Editor’s note:	 the Interface between AAA Server and SNPN is FFS.

* * * * End of Changes * * * *