3GPP TSG-SA WG2 Meeting #144E (e-meeting)	S2-2102624

# April 12 – April 16, 2021, Elbonia											 (revision of S2-210xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2798 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Network Slice Admission Control in EPC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | As discussed in S2-2102623, there is a need to specify NSAC mechanism in EPC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the network slice admission control in EPC - Proposal 1: The Network Slice Admission Control feature for maximum number of UEs and for maximum number of PDU Sessions per network slice is applied when the UE is connected to EPC. - Proposal 2: The Network Slice Admission Control for maximum number of UEs and for maximum number of PDU Sessions per network slice is performed during the PDN connectivity setup procedure when the UE is connected to EPC. - Proposal 3: To support Network Slice Admission Control in EPC, an interface between NSACF and SMF (i.e. SMF+PGW-C) is defined. No MME impact is expected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The network slice admission control in EPC is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.11.X (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** 1st change ***

5.15.11.X	Support of Network Slice Admission Control in EPC

Network Slice Admission Control for maximum number of UEs and for maximum number of PDU Sessions per network slice is performed at the time of PDN connection establishment in EPC. To support the NSAC for maximum number of UEs and for maximum number of PDU Sessions per network slice in EPC, the SMF+PGW-C is configured with the information indicating which network slice is subject to NSAC. During PDN connection establishment in the EPC, the SMF+PGW-C selects an S-NSSAI associated with the PDN connection as described in clause 5.15.7.1. If the selected S-NSSAI by the SMF+PGW-C is subject to the NSAC, the SMF+PGW-C triggers interaction with NSACF to check the availability of the network slice, before the SMF+PGW-C provides the selected S-NSSAI to the UE. If the NSACF returns that the network slice is available, the SMF+PGW-C continues to proceed the PDN connection establishment procedure.

If the slice availability request is sent from SMF+PGW-C, the NSACF checks the current number of UE registration and the current number of PDU sessions for the network slice.

If:

-	the UE identity is already included in the list of UE IDs registered with a network slice and the current number of PDU sessions is below the maximum number; or

-	the UE identity is not included in the list of UE IDs registered with a network slice, and neither the current number of UE registration nor the current number of PDU sessions reaches the maximum number;

the NSACF responds to the SMF+PGW-C with the information that the network slice is available. The NSACF increases the current number of UE registration (if applicable) and the current number of PDU sessions.

Editor’s Note:	It is FFS how to configure the NSACF with the maximum number, e.g. the operator may want to use a unified quota for 5GC and EPC, or may want to use separate quotas for 5GC and EPC.

Editor’s Note:	It is FFS whether one NSACF is in charge of registration and session admission control, or there are respective NSCAFs for registration and session admission control, depending on the deployment scenarios.

Editor’s Note:	It is FFS whether and how the SMF+PGW-C can accept the PDN connection establishment request if the current number of UE registration, the current number of PDU sessions, or both reaches the maximum number.

NOTE:	Network Slice Admission Control in EPC is not performed for the attachment without PDN connectivity.

When the UE having an ongoing PDN connection moves from EPC to 5GC, the session continuity is guaranteed as the admission was granted at the time of PDN connection establishment.

*** End of the changes ***