3GPP TSG-WG SA2 Meeting #144E e-meeting  	S2-21002662r02

# Elbonia, Apr 12 – Apr 16, 2021	(revision of S2-2100881)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 243 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Exposing UE mobility analytics for multiple NWDAFs case |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | With the assumption with analytics ID=mobility analytics as concluded in clause 8.2.3 in TS 23.288, the procedure for exposing UE mobility analytics for multiple NWDAFs case needs to be defined in TS 23.288. TR 23.700-91, KI#2 conclusions in 8.2.3 indicates that during normative phase we might consider to do an “extension to analytics filters”. But it may not easy to express the scenario that the UE(s) currently stays in an Area of Interest and once stayed in one of Target Area of Interest(s) by including two type of AOIs in the same analytics filter. After analysis, it is found that using the Analytics reporting information to restrict the reporting target UEs to the ones who once stayed in one of Target Area of Interest(s) may be a better choice, so it is suggested to extend the Analytics Reporting Information to include the target AOI(s), this target AOI is to figure out SUPI list who once stayed in the target area.. |
| Summary of change: |  | - For analytics ID= UE mobility, extend the Analytics Reporting Information to include the target AOI(s) to figure out SUPI list who once stayed in the target area. - Define the procedure for exposing UE mobility analytics for multiple NWDAFs case. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Lack the procedure for exposing UE mobility analytics for multiple NWDAFs case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.3, 6.7.2.1, 6.7.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR’s revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

### 6.1.3	Contents of Analytics Exposure

The consumers of the Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operations described in clause 7 provide the following input parameters listed below.

-	A list of Analytics ID(s): identifies the requested analytics.

-	Analytics Filter Information: indicates the conditions to be fulfilled for reporting Analytics Information. This set of optional parameter types and values enables to select which type of analytics information is requested. Analytics Filter Information are defined in the analytics related procedures.

-	Target of Analytics Reporting: indicates the object(s) for which Analytics information is requested, entities such as specific UEs, a group of UE(s) or any UE (i.e. all Ues).

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) A Notification Target Address (+ Notification Correlation ID) as defined in TS 23.502 [3] clause 4.15.1, allowing to correlate notifications received from NWDAF with this subscription.

-	Analytics Reporting Information with the following parameters:

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) Analytics Reporting Parameters as per Event Reporting parameters defined in Table 4.15.1-1, TS 23.502 [3].

-	(Only for Nnwdaf_AnalyticsSubscription_Subscribe) Reporting Thresholds, which indicate conditions on the level of each requested analytics that when reached shall be notified by the NWDAF. A matching direction may be provided such as below, above, or crossed. If no matching direction is provided, the default direction is crossed.

-	Analytics target period: time interval [start..end], either in the past (both start time and end time in the past) or in the future (both start time and end time in the future). An Analytics target period in the past is a request or subscription for statistics. An Analytics target period in the future is a request or subscription for predictions. The time interval is expressed with actual start time and actual end time (e.g. via UTC time). When the Analytics Reporting Parameters indicate a periodic reporting mode, the time interval can also be expressed as positive or negative offsets to the reporting time, which indicates a subscription for predictions or statistics respectively. By setting start time and end time to the same value, the consumer of the analytics can request analytics or subscribe to analytics for a specific time rather than for a time interval.

-	[OPTIONAL] Data time window: if specified, only events that have been created in the specified time interval are considered for the analytics generation.

-	Preferred level of accuracy of the analytics (“Low”, “Medium”, “High” or “Highest”).

-	[OPTIONAL] Accuracy level per analytics subset (“Low”, “Medium”, “High” or “Highest”). When an accuracy level is expressed for a given analytics subset, it takes precedence for this subset over the above preferred level of accuracy. Analytics subsets are defined in the “Output Analytics” clause of applicable analytics.

-	[OPTIONAL] Dataset Statistical Properties: NWDAF may use the dataset statistical properties information in order to influence the data selection mechanisms to be used for the generation of an analytics IDs, assuring that the generated analytics ID reflects the statistical characteristics of the data that are relevant for the NWDAF consumer in all different NWDAFs.

Editor’s note:	The specific parameters that are part of the “dataset statistical properties” are FFS.

-	Time when analytics information is needed (if applicable): indicates to the NWDAF the latest time the analytics consumer expects to receive analytics data provided by the NWDAF. If the time is reached the consumer does not need to wait for the analytics information any longer, yet the NWDAF may send an error response or error notification to the consumer. “Time when analytics information is needed” is a relative time interval as the gap with respect to analytics request /subscription (e.g. “in 10 minutes”).

-	[OPTIONAL] Maximum number of objects requested by the consumer (max) to limit the number of objects in a list of analytics per Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request response.

-	[OPTIONAL] Preferred order of results when a list of analytics is returned, possibly with a criterion for identifying the property of the results to which the preferred ordering is applied.

-	[OPTIONAL] Maximum number of SUPIs (SUPImax) requested by the consumer to limit the number of SUPIs in an object. When SUPImax is not provided, the NWDAF shall return all SUPIs concerned by the analytics object. When SUPImax is set to 0, the NWDAF shall not provide any SUPI.

-	[OPTIONAL, Only for the Analytics ID=UE Mobility] Target area requested by the consumer to figure out SUPI list and the corresponding UE mobility analytics who once stayed in the target area. If multiple target areas are provided by the consumer, that is to request the NWDAF to figure out SUPI list and the corresponding UE mobility analytics who once stayed in one of the target areas at least.

-	[OPTIONAL] Output strategy: indicates the output strategy for the analytics reporting. The following values are allowed:

-	Binary output strategy: if preferred level of accuracy is more important than providing an output, then the binary strategy is used such that all analytics outputs have equivalent confidence in the prediction.

-	Gradient output strategy: if having an analytics output is more important than reaching the preferred level of accuracy, then the gradient output strategy is used such that each NWDAF will timely provide the output indicating the level of accuracy at the moment of the output generation.

-	[OPTIONAL] Analytics Metadata Request: indicates a request from one NWDAF to another NWDAF to provide the “analytics metadata information” related to the produced output analytics. This input parameter is provided by the NWDAF if analytics metadata information is required to aggregate the output analytics for the requested Analytics ID(s).

Editor’s note:	Whether the “analytics metadata request” is a flag or it allows to request specific sub-parameters of the “analytics metadata information” is FFS.

The NWDAF provides to the consumer of the Nnwdaf_AnalyticsSubscription_Notify or Nnwdaf_AnalyticsInfo_Request service operations described in clause 7, the output information listed below:

-	(Only for Nnwdaf_AnalyticsSubscription_Notify) The Notification Correlation Information.

-	For each Analytics ID the analytics information in the requested Analytics target period.

-	In addition, the following additional information:

-	Timestamp of analytics generation, which allows consumers to decide until when the received information shall be used. For instance, an NF can deem a received notification from NWDAF for a given feedback as invalid based on this timestamp;

-	Validity period, which defines the time period for which the analytics information is valid.

-	Probability assertion: confidence in prediction.

-	(Only when analytics aggregation is required and Gradient Output Strategy is used) Level of accuracy reached for the specific notification for a cycle of the periodicity or for the request.

Editor’s note:	It is FFS whether the “level of accuracy” is also to be provided for other cases.

-	[OPTIONAL] Analytics Metadata Information: additional information required to aggregate the output analytics for the requested Analytics ID(s). This parameter shall be provided if the “Analytics Metadata Request” parameter was provided in the corresponding Nnwdaf_AnalyticsSubscription_Subscribe or Nnwdaf_AnalyticsInfo_Request service operation.

-	number of data samples used for the generation of the output analytics,

-	time window of the data samples,

-	Dataset Statistical Properties.

Editor’s note:	The specific parameters that are part of the “dataset statistical properties” are FFS.

Editor’s note:	The specific parameters that are part of the “analytics metadata information” are FFS. Whether “dataset statistical properties” and “time window” are part of the analytics metadata information is FFS.

-	(Only for error response or error notification) Revised waiting time: indicates to the consumer a revised waiting value for “Time when analytics information is needed”. Each NWDAF may include this as part of error response or error notification to “Time when analytics information is needed” as described in clause 6.2.5. Revised waiting time is the minimum time interval recommended by NWDAF to use as “Time when analytics information is needed” for similar future analytics requests/subscriptions.

Editor’s note:	The specification of “error response” and “error notification” is FFS, e.g. whether the other parameters of the output information are optional / not included in such error response/notification.

NEXT CHANGES

### 6.7.2	UE mobility analytics

#### 6.7.2.1	General

NWDAF supporting UE mobility statistics or predictions shall be able to collect UE mobility related information from NF, OAM, and to perform data analytics to provide UE mobility statistics or predictions.

The service consumer may be a NF (e.g. AMF, SMF or AF).

The consumer of these analytics may indicate in the request:

-	The Target of Analytics Reporting which is a single UE or a group of UEs.

-	Analytics Filter Information optionally containing:

-	Area of Interest;

NOTE:	For LADN service, the consumer (e.g. SMF) provides the LADN DNN to refer the LADN service area as the Area Of Interest.

-	An Analytics target period indicates the time period over which the statistics or predictions are requested.

-	Optionally, maximum number of objects.

-	Preferred level of accuracy of the analytics.

-	Preferred order of results for the time slot entries: ascending or descending time slot start;

-	In a subscription, the Notification Correlation Id and the Notification Target Address are included.

NEXT CHANGES

6.7.2.4	Procedures

The NWDAF can provide UE mobility related analytics, in the form of statistics or predictions or both, to another NF. If the NF is an AF, and when the AF is untrusted, the AF will request analytics via the NEF, and the NEF will then convey the request to NWDAF.

NOTE:	In the case of untrusted AF the Target of Analytics Reporting can be a GPSI or an External Group Identifier that is mapped in the 5GC to a SUPI or an Internal Group Identifier.

Figure 6.7.2.4-1: UE mobility analytics provided to an NF

1.	The NF sends a request (Analytics ID = UE mobility, Target of Analytics Reporting = UE id or Internal Group ID, Analytics Filter information =AOI, Analytics Reporting Information= Analytics target period) to the serving NWDAF for analytics information on a specific UE or a group of UEs, using either the Nnwdaf_AnalyticsInfo or Nnwdaf_AnalyticsSubscription service to derive UE mobility information. The NF can request statistics or predictions or both. For LADN service, the NF (i.e. SMF) provides LADN DNN as Area of Interest in the Analytics Filter Information.

If NF wants to obtain the UE ID list and their aggregated mobility analytics during an Analytics target period(s), who currently stays in an Area of Interest and once stayed in one of Target Area of Interest(s), the NF may send request for UE mobility analytics with Analytics ID = UE mobility, Target of Analytics Reporting = UE id(s), Analytics Filter information = current AOI, Analytics Reporting information = (Analytics target period, Target AOI (s)). In this case, the requested mobility analytics is a statistics.

2.	If the request is authorized, and in order to provide the requested analytics, the NWDAF may subscribe to events with all the serving AMFs for notification of location changes. This step may be skipped when e.g. the NWDAF already has the requested analytics available.

The NWDAF subscribes the service data from AF(s) in the Table 6.7.2.2-2 by invoking Naf_EventExposure_Subscribe service or Nnef_EventExposure_Subscribe (if via NEF) ) using event ID "UE Mobility information" as defined in TS 23.502 [3].

The NWDAF collects UE mobility information from OAM, following the procedure captured in clause 6.2.3.2.

NOTE:	The NWDAF determines the AMF serving the UE or the group of UEs as described in clause 6.2.2.1.

3.	The NWDAF derives requested analytics.

If in step 1 the NWDAF receives analytics subscription/request from NF to obtain the UE ID list and their aggregated mobility analytics during an Analytics target period(s), who currently stays in an Area of Interest and once stayed in one of Target Area of Interest(s), and if target AOI(s) and current AOI are covered by different NWDAFs, in addition to the data collected in step 2, the NWDAF can also obtain UE mobility analytics in one of the Target Area of Interest(s) during the Analytics target period from other NWDAF instance(s). Then the NWDAF supporting analytics aggregation capability derives the UE ID list and the requested aggregated analytics based on the data collected in step 2 and UE mobility analytics obtained from the other NWDAF instance(s). UE visited locations in target AOI(s) and current AOI will be included in the aggregated UE mobility analytics.

NOTE: If the target AOI(s) and current AOI are covered by different NWDAFs, then consumer firstly discovers a NWDAF supporting analytics aggregation capability in the current AOI from the NRF, as defined in clause 6.3.13, TS 23.501[2].

4.	The NWDAF provide requested UE mobility analytics to the NF, using either the Nnwdaf_AnalyticsInfo_Request response or Nnwdaf_AnalyticsSubscription_Notify, depending on the service used in step 1. The details for UE mobility analytics provided by NWDAF are defined in clause 6.7.2.3.

5-6. If at step 1, the NF has subscribed to receive notifications for UE mobility analytics, after receiving event notification from the AMFs, AFs and OAM subscribed by NWDAF in step 2, the NWDAF may generate new analytics and provide them to the NF.

END OF CHANGES