3GPP TSG-SA WG2 Meeting #144E	S2-2102815

# E-Meeting, 12th  April – 16th April 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2826 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Mobility support between SNPNs and between SNPN and PLMN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TR23.700-07 clause 8.1.2, it concludes: - In the case that there are common AMF and/or N14 interface between the source network and target network, mechanism defined in TS 23.502 [6] clause 4.9.1 is re-used to address UE mobility. - In the case of idle mode mobility, the UE performs initial or mobility registration as specified in clause 4.2.2.2.2 of TS 23.502 [6]. The mobility will be supported between PLMN and SNPN, and between SNPNs. The following sepcification will be updated based on the above conclusion. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - The handover is supported by using the existing procedure - The corresponding sepcification is updated accordingly to support the idle mode mobility between SNPNs and between SNPN and PLMN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The mobility is not supported between SNPNs and between PLMN and SNPN |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.2.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS 23.502 |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*******Start of changes*******

### 5.30.2	Stand-alone non-public networks

#### 5.30.2.0	General

SNPN 5GS deployments are based on the architecture depicted in clause 4.2.3, the architecture for 5GC with untrusted non-3GPP access (Figure 4.2.8.2.1-1) for access to SNPN services via a PLMN (and vice versa) and the additional functionality covered in clause 5.30.2.

Alternatively, a Credentials Holder (CH) may authenticate and authorize access to an SNPN separate from the Credentials Holder based on the architecture specified in clause 5.30.2.9.

In this Release, direct access to SNPN is specified for 3GPP access only.

Interworking with EPS is not supported for SNPN. Also, emergency services are not supported for SNPN. Furthermore, roaming is not supported for SNPN, e.g. roaming between SNPNs. Connected mode mobility between SNPNs, between SNPN and PLMN or PNI NPN is not supported. Idle mode mobility is supported as defined in clause 5.30.2.X. CIoT 5GS optimizations are not supported in SNPNs.

*******End of changes*******