3GPP TSG-WG SA2 Meeting #144E e-meeting 	S2-2102828

Elbonia, April 12 – April 16, 2021	(revision of S2-210xxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0325 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | User consent revocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to the conclusion of KI#15: User consent for UE data collection/analytics, the UDR (via UDM services) holds the user consent for user related data which is provisioned by MNO as a user subscription information. If user consent is revoked, currently, there is no way to inform the data consumers storing/processing the data subject to the revoked user consent, requesting those entities to delete the affected data. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A functionality is provided to support tracking the distribution and usage of any user related information that may be subject to user consent. This functionality, in case of revocation of user consent, will contact data consumers requesting them to delete any data subject to the revoked user consent. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In case of user consent revocation, the 5GS does not know about entities storing/processing data subject to the revoked user consent. Not being able to delete such data can violate GDPR in certain jurisdictions. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.X.2 (new), 6.2.X.3 (new), 6.2.X.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

*** Start of the changes (all new text) ***

#### 6.2.X.2	Data request for UE data subject to user consent

Figure 6.2.X.2-1 depicts the procedure for checking user consent for UE data collection. The user consent for UE data collection is managed within the 5GC via subscription management procedures.

Figure 6.2.X.2-1: User consent for UE data collection

1.	Data consumer (e.g. NWDAF) needs to collect UE related data.

2.	Data consumer determines an NF instance providing the required data, e.g. it may query UDM to discover the NF serving the UE.

3.	Based on local policy or regulations, data consumer may request to UDM using Nudm_SDM_Get (UE_Id, user consent for UE data collection) service operation whether user consent is available and valid for the given UE Id, and purpose of data collection, which indicates the purpose for data collection is to do data analysis related to specific analytic ID when the data consumer is the NWDAF. This requires that UDM may retrieve this information from UDR by Nudr_DM_Query.

Editor's Note:	Whether to also include information about the requested data is FFS.

Editor's Note:	Whether the user consent information is managed by the UDM/UDR should be aligned with SA3 work, and whether to use Nudm_SDM_Get service operation or another existing or new service operation is FFS.

4.	UDM responds to the Data consumer with the user consent data for the requested UE data collection if available.

5.	If requested UE data is subject to user consent and user consent is available, the data consumer subscribes to changes in the user consent for UE data collection information via Nudm_SDM_Subscribe (UE_Id, user consent for UE data collection) service operation.  UDM may subscribe to UDR by Nudr_DM_Subscribe.

NOTE 1:	If the subscription data is updated (e.g. to remove the user consent of a particular UE), the UDM notifies Data Consumers subscribed to changes in the user consent for UE data collection for that UE. Based on operator policies and legal requirements, the Data consumer receiving such notification might need to delete any affected data and stop any active data collection no longer allowed by the user consent. See clause 6.2.X.4.

5.	UCDT service in UDM responds to the Data consumer whether user consent for the requested UE data collection is available or not, or not needed.

6a.	If user consent is available or no user consent is needed Data consumer requests the data from the data source, e.g. as a data request or event subscription.

6b.	Data source provides the data, e.g. as Data response or as notifications related to the requested subscription.

NOTE 2:	The response message in step 4 6b may also include a validity time (based on operator policy, e.g. 1 week) indicating the duration over which the Data may be stored and/or processed. At the expiry of this validity time, any Data Consumer (e.g. NWDAF) needs to either delete corresponding Data and/or Analytics or request from the Data Source an extension of the validity time.

Editor's Note:	Whether to specify a dedicated service operation for requesting an extension of the validity time or to extend an existing service operation is FFS.

#### 6.2.X.3	Analytics request subject to user consent

Figure 6.2.X.3-1 depicts the procedure for checking user consent for analytics requests/subscriptions.

Editor's Note:	Below procedure might need to be aligned based on SA3 feedback.

Figure 6.2.X.3-1: User consent for analytics requests/subscriptions

1.	Analytics consumer requests/subscribes to analytics ID(s), prior to that the Analytics consumer get the user consent to provide analytics, and contact UDM

2.	NWDAF determines required Data and data source(s). If required Data is UE related, NWDAF checks the user consent for UE data collection according to the procedure specified in clause 6.2.X.2. NWDAF collects the required Data from the data source(s).

3.	NWDAF generates analytics and responds/notifies analytics report to analytics consumer.

4.	(If

#### 6.2.X.4	User consent revocation procedure

Figure 6.2.X.4-1 depicts the procedure related to user consent revocation.

Figure 6.2.X.4-1: User consent revocation procedure

1.	Subscription data for a user is updated removing user consent for UE data collection.

2.	UDM determines the subscribed Data Consumers (e.g. NWDAF) required to be notified.

NOTE 1:	Data Consumers can also include consumers having received analytics data from NWDAF.

3.	UDM may notify all Data Consumers determined in step 2 using Nudm_SDM_Notify to delete any UE Data and stop any active subscriptions related to the user that has revoked his consent.

4.	Based on local policy or regulations, Data Consumers (e.g. NWDAF) confirm UDM that the UE related Data has been deleted, is no longer collected and is no longer being processed/used, Nudm_SDM_Info.

NOTE 2:	Based on local policy or regulations, it might not be required to delete UE related Data collected by the data consumer.

Editor's Note:	Whether data should not be deleted in case of Lawful Interception and how to indicate this to the UDM and/or Data Consumer is FFS and should be aligned with SA3-LI.

### *** End of the changes ***