SA WG2 Meeting #144E (e-meeting)	S2-2102839

# April 12 - 16, 2021 									                                (revision of S2-210XXXX)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2832 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE configuration for remote provisioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-04-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It is to refect the UE configuration in the conclusions for “Remote provisioning for SNPN credentials (Component 2 of KI#4)” in clause 8.4.1 of TR23.700-07, Onboarding configuration is introduced to User Plane Remote Provisioning for SNPN credentials. Additionally, as discuessed in S2-2102838: - An ON may be configured with multiple PVSs’ information. But different UEs may have different requirement for credential. E.g. UE A wants SNPN K’ credential, UE B wants SNPN Q’s credential, UE C wants SNPN R’s credential. Different credentials may be provisioned by different PVSs. - Provisioning all the PVSs’ information to the UE make the UE to blind attempt and it is inefficient since at most only one PVS is valid for the UE but the UE may need to try all the PVS and receiving failure time by time. E.g. the UE A may happen to try PVS_2 and PVS_3 before PVS_1. - In order to provide correct PVS information the UE, the ON needs the index from the UE, i.e. the ID of the SNPN of which the credential is required by the UE. The SNPN ID is preconfigured on the UE or provide by the user. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the UE configuration for UP Remote Provisioning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE configuration for Onboarding configuration is not introduced. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.X.2.7.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** Start of changes *****

5.30.2.X.4	Remote Provisioning of UEs in Onboarding Network

5.30.2.X.4.2	Onboarding configuration for the UE

Editor’s Note:	This clause should include the configuration data in the UE for UP provisioning.

In order to enable UP Remote Provisioning of SNPN credentials for a UE, Configuration data for UP Remote Provisioning are either pre-configured on the UE or provided by the ONN. Configuration data for UP Remote Provisioning provided by the ONN take precedence over corresponding configuration data stored in the UE.

Configuration data for UP Remote Provisioning (i.e., PVS address information including PVS IP address or PVS FQDN) may be stored in the ME or on the UICC.

Additionally, the UE may be preconfigured with the PLMN ID and the NID of the SNPN (i.e., SO SNPN) of which the credential is required by the UE. The PLMN ID and the NID of the SNPN may also be by provided to the UE by the User.

The UE may obtain the Configuration data for UP Remote Provisioning from the ONN as the following: the SMF in the ONN may store the Configuration data for UP Remote Provisioning (i.e. PVS IP address and/or PVS FQDN), which is associated with a list of SNPN ID (i.e. PLMN ID and the NID). If the UE wants to download a SNPN’s credential, during the PDU session setup procedure, it provides the SNPN ID to the SMF. The SMF provides PVS IP address and/or PVS FQDN to the UE. If there are multiple PVSs’ IP addresses and FQDNs, the SMF use the SNPN ID provided by the UE to determine the PVS IP address and/or PVS FQDN for the UE.

Editor’s Note X1: It is FFS whether a domain name is also required in the Configuration data for UP Remote Provisioning.

Editor’s Note X2:  It is FFS whether request the UE to provide its preconfigured Configuration data for UP Remote Provisioning (e.g. the PVS IP address) to the ONN, in order to support the ON-SNPN to restrict the PDU session used for the PVS IP address only, when there is not PVS IP address in the ONN for the UE.

The ONN may obtain the Configuration data for UP Remote Provisioning as the following:

- Locally configured in the ON-SNPN; or

- Provided by the DCS to the AMF in ON-SNPN as part of authentication procedure as specified in TS 33.501 [29]. AMF stores the UE onboarding configuration locally and forwards it to SMF as part of the PDU Session Establishment Request.

- Dynamic configuration information driven by AF via NEF at the Onboarding Network (e.g. Service specific parameter provisioning procedure as specified in TS 23.502 clause 4.15.6.7).

***** End of changes *****