SA WG2 Meeting #144E (e-meeting)	S2-2102839r07

# April 12 - 16, 2021 									                                (revision of S2-210XXXX)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2832 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | UE configuration for remote provisioning |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vivo, Nokia, Nokia Shanghai Bell, ETRI |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNPN |  |  |  |  |  | Date: |  |  | 2021-04-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | It is to refect the UE configuration in the conclusions for “Remote provisioning for SNPN credentials (Component 2 of KI#4)” in clause 8.4.1 of TR23.700-07, Onboarding configuration is introduced to User Plane Remote Provisioning for SNPN credentials. Additionally, as discuessed in S2-2102838: - An ON may be configured with multiple PVSs’ information. But different UEs may have different requirement for credential. E.g. UE A wants SNPN K’ credential, UE B wants SNPN Q’s credential, UE C wants SNPN R’s credential. Different credentials may be provisioned by different PVSs. - Provisioning all the PVSs’ information to the UE make the UE to blind attempt and it is inefficient since at most only one PVS is valid for the UE but the UE may need to try all the PVS and receiving failure time by time. E.g. the UE A may happen to try PVS_2 and PVS_3 before PVS_1. - In order to provide correct PVS information the UE, the ON needs the index from the UE, i.e. the ID of the SNPN of which the credential is required by the UE. The SNPN ID is preconfigured on the UE or provide by the user. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the UE configuration for UP Remote Provisioning. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE configuration for Onboarding configuration is not introduced. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.30.X.2.7.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  | X |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | The UE constructed FQDN for PVS discovery according to 3GPP rules are expected to be defined in TS 23.003. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

***** Start of changes *****

##### 5.30.2.X.4	Remote Provisioning of UEs in Onboarding Network

##### 5.30.2.X.4.2	Onboarding configuration for the UE

In order to enable UP Remote Provisioning of SNPN credentials for a UE, UE Configuration Data for UP Remote Provisioning are either pre-configured on the UE or provided by the ON-SNPN. UE Configuration Data for UP Remote Provisioning provided by the ON-SNPN take precedence over corresponding configuration data stored in the UE.

UE Configuration Data for UP Remote Provisioning consist of PVS address information including PVS IP address or PVS FQDN.

After the establishment of the restricted PDU session used for onboarding, if the UE does not have any PVS address or PVS FQDN, the UE may construct an FQDN for PVS discovery as defined in TS 23.003 [19].

The UE Configuration Data for UP Remote Provisioning may be stored in the ME.

UE Configuration Data for UP Remote Provisioning (i.e. PVS IP address or PVS FQDN) may be locally configured in the SMF of ON-SNPN and may be provided to the UE during the establishment of the restricted PDU Session as part of extended Protocol Configuration Options (PCO) in the PDU Session Establishment Response.

Editor’s Note: How the ON-SNPN obtains the PVS IP address or PVS FQDN for UP Remote Provisioning is FFS.

***** End of changes *****