SA WG2 Meeting #144E (e-meeting)	S2-2102844

# Feb 24 – Mar 09, 2021 									                         (revision of S2-2100XXXX)

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2834 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct the Time Synchronization service |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | vivo |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | IIOT |  |  |  |  |  | Date: |  |  | 2021-04-01 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The time synchronization may ternimated at the UE (e.g. the UE is the End Station). For this case, if DS-TT is present, it is collocated with the UE and the DS-TT doesn’t need to opertate and distribute the received (g)PTP messages. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify when the time synchronization is ternimated at the UE. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The operation is incorrect when the time synchronization is ternimated at the UE. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.27.1.1, 5.27.1.2.1, 5.27.1.2.2.1, 5.27.1.2.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

FIRST CHANGE

5.27.1	Time Synchronization

5.27.1.1	General

For supporting time synchronization service, the 5GS is configured to operate in one of the following modes (if supported):

1)	as time-aware system as described in IEEE Std 802.1AS [104],

2)	as Boundary Clock as described in another IEEE Std 1588-2019 [126] profile,

3)	as peer-to-peer Transparent Clock as described in another IEEE Std 1588-2019 [126] profile, or

4)	as end-to-end Transparent Clock as described in another IEEE Std 1588-2019 [126] profile.

The configuration of the time synchronization service in 5GS for option 1 by TSN AF and CNC is described in clause 5.28.3, and for options 1-4 by AF and NEF in clause 5.27.1.z.

The 5GS shall be modelled as an IEEE Std 802.1AS [104] or IEEE Std 1588-2019 [126] compliant entity based on the above configuration.

The TSN Translators (TTs) at the edges of the 5G system may support the IEEE Std 802.1AS [104] or other IEEE Std 1588-2019 [126] profiles' operations respective to the configured mode of operation. The UE, gNB, UPF, NW-TT and DS- TTs are synchronized with the 5G GM (i.e. the 5G internal system clock) which shall serve to keep these network elements synchronized. The TTs located at the edge of 5G system fulfil some functions related to IEEE Std 802.1AS [104] and may fulfill some functions related to other IEEE Std 1588-2019 [126] profiles, e.g. (g)PTP support and timestamping. Figure 5.27.1-1 illustrates the 5G and PTP grandmaster (GM) clock distribution model via 5GS.

Figure 5.27.1-1: 5G system is modelled as PTP instance for supporting (g)PTP time synchronization

Figure 5.27.1-1 depicts the two synchronizations systems considered: the 5GS synchronization and the (g)PTP domain synchronization, as well as the Master (M) and Slave (S) ports considered when the PTP GM is located at (g)PTP network outside of 5GS.

-	5GS synchronization: Used for NG RAN synchronization. 5G RAN synchronization is specified in TS 38.331 [28].

-	(g)PTP domain synchronization: Provides synchronization service to (g)PTP network. This process follows IEEE Std 802.1AS [104] or IEEE Std 1588-2019 [126].

For the case of 5G system modelled as a Transparent clock or as an IEEE 802.1AS Boundary Clock, the two synchronization processes can be considered independent from each other and the gNB only needs to be synchronized to the 5G GM clock.

The 5GS supports two methods for determining the grandmaster PTP Instance and the time-synchronization spanning tree.

-	Method a), BMCA procedure.

-	Method b), local configuration.

This is further described in clause 5.27.1.6.

END OF CHANGES