3GPP TSG-WG SA2 Meeting #143E e-meeting 	S2-2102861

# Elbonia, April 12 – 16, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0330 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Extension of Naf_EventExposure for observed service experience data collection from UEs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | InterDigital |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA_Ph2 |  |  |  |  |  | Date: |  |  | 2021-03-26 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The conclusions for KI#8: “UE data as an input for analytics generation” in TR 23.700-91, propose that the observed service experience data collection, defined for Naf_EventExposure in Rel-16, be extended in TS 23.288. |  |  |  |  |  |  |  |  |
| Summary of change: |  | UE data input for observed service experience are added in clause 6.4.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE data input collected through AF for observed service expeirence is not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--------------------------------------------------------------------FIRST CHANGE-------------------------------------------------------

### 6.4.2	Input Data

The service data and performance data collected from the AF(including the service data collected from the UE through the AF),, the network data from other 5GC NFs and the network data from OAM for observed service experience are defined in Table 6.4.2-1, 6.4.2-1a, Table 6.4.2-2, Table 6.4.2-3 and Table 6.4.2-4 respectively.

Table 6.4.2-1: Service Data from AF related to the observed service experience

| Information | Source | Description |
| --- | --- | --- |
| Application ID | AF | To identify the service and support analytics per type of service (the desired level of service) |
| IP filter information | AF | Identify a service flow of the UE for the application |
| Locations of Application | AF/NEF | Locations of application represented by a list of DNAI(s). The NEF may map the AF-Service-Identifier information to a list of DNAI(s) when the DNAI(s) being used by the application are statically defined. |
| Service Experience | AF/UE(via AF) | Refers to the QoE per service flow as established in the SLA and during on boarding. It can be either e.g. MOS or video MOS as specified in ITU-T P.1203.3 [11] or a customized MOS for any kind of service including those not related to video or voice. |
| QoE metrics | UE(via AF) | QoE metrics observed at the UE(s). QoE metrics and measurement as described in Clause 16 of TS 26.114 [x], or ASP specific QoE metrics, as agreed in the SLA with the MNO. may be used. |
| Timestamp | AF | A time stamp associated to the Service Experience provided by the AF, mandatory if the Service Experience is provided by the ASP. |
| Application Server Instance | AF | The IP address or FQDN of the Application Server that the UE had a communication session when the measurement was made. |

NWDAF subscribes to the service data from AF in the Table 6.4.2-1 either directly for trusted AFs by invoking Naf_EventExposure_Subscribe service (Event ID = Service Experience information, Event Filter information = Area of Interest, Application ID) as defined in TS 23.502 [3], or indirectly for untrusted AFs via NEF by invoking Nnef_EventExposure_Subscribe service (Event ID = Service Experience information, Event Filter information = Area of Interest, Application ID) where NEF translates the Area of Interest into geographic zone identifier(s). For the information whose source is UE(via AF), the AF collects data from the UE as defined in 6.2.8.

NOTE:	When the Service Experience is expressed as a customized MOS, the customized MOS may be defined by the content provider or by the MNO and may be based on the nature of the targeted service type (e.g. web browsing, gaming, augmented reality, V2X, SMS).

Table 6.4.2-1a: Performance Data from AF

| Information | Source | Description |
| --- | --- | --- |
| UE identifier | AF | IP address of the UE at the time the measurements was made. |
| UE location | AF | The location of the UE when the performance measurement was made. |
| Application ID | AF | To identify the service and support analytics per type of service (the desired level of service). |
| IP filter information | AF | Identify a service flow of the UE for the application. |
| Locations of Application | AF/NEF | Locations of application represented by a list of DNAI(s). The NEF may map the AF-Service-Identifier information to a list of DNAI(s) when the DNAI(s) being used by the application are statically defined. |
| Application Server Instance address | AF/NEF | The IP address/FQDN of the Application Server that the UE had a communication session when the measurement was made. |
| Performance Data | AF | The performance associated with the communication session of the UE with an Application Server that includes: Average Packet Delay, Average Loss Rate and Throughput. |
| Timestamp | AF | A time stamp associated to the Performance Data provided by the AF. |

NWDAF subscribes to the performance data from AF in the Table 6.4.2-1a either directly for trusted AFs by invoking Naf_EventExposure_Subscribe service (Event ID = Performance Data, Event Filter information = Area of Interest, Application ID) as defined in TS 23.502 [3], or indirectly for untrusted AFs via NEF by invoking Nnef_EventExposure_Subscribe service (Event ID = Performance Data, Event Filter information = Area of Interest, Application ID) where NEF translates the Area of Interest into geographic zone identifier(s).

Table 6.4.2-2: QoS flow level Network Data from 5GC NF related to the QoS profile assigned for a particular service (identified by an Application Id or IP filter information)

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| Location | AMF | The UE location information, e.g. cell ID or TAI. |
| (list of)SUPI(s) | AMF | If UE IDs are not provided as target of analytics reporting for slice service experience, AMF returns the UE IDs matching the AMF event filters. |
| DNN | SMF | DNN for the PDU Session which contains the QoS flow. |
| S-NSSAI | SMF | S-NSSAI for the PDU Session which contains the QoS flow. |
| Application ID | SMF | Used by NWDAF to identify the application service provider and application for the QoS flow. |
| UPF info | SMF | UPF ID/address/FQDN information for the UPF serving the UE. |
| DNAI | SMF | Identifies the access to DN to which the PDN session connects. |
| IP filter information | SMF | Provided by the SMF, which is used by NWDAF to identify the service data flow for policy control and/or differentiated charging for the QoS flow. |
| QFI | SMF | QoS Flow Identifier. |
| QoS flow Bit Rate | UPF | The observed bit rate for UL direction; and The observed bit rate for DL direction. |
| QoS flow Packet Delay | UPF | The observed Packet delay for UL direction; and The observed Packet delay for the DL direction. |
| Packet transmission | UPF | The observed number of packet transmission. |
| Packet retransmission | UPF | The observed number of packet retransmission. |

NOTE 1:	How NWDAF collects QoS flow Bit Rate, QoS flow Packet Delay, Packet transmission and Packet retransmission information from UPF is not defined in this Release of the specification.

NOTE 2:	Care shall be taken with regards to load and major signalling caused when requesting Any UE. This could be achieved via utilization of some event filters (e.g. Area of Interest for AMF), Analytics Reporting Information (e.g. SUPImax), or sampling ratio as part of Event Reporting Information.

NWDAF subscribes to the network data from 5GC NF(s) in the Table 6.4.2-2 by invoking Nnf_EventExposure_Subscribe service operation with the following Event IDs as input parameters:

-	AMF Source: Namf_EventExposure_Subscribe (Event IDs = Location Changes, Area of Interest).

-	SMF Source: Nsmf_EventExposure_Subscribe (Event ID = QFI allocation).

Table 6.4.2-3: UE level Network Data from OAM related to the QoS profile

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | OAM | A time stamp associated with the collected information. |
| Reference Signal Received Power | OAM | The per UE measurement of the received power level in a network cell, including SS-RSRP, CSI-RSRP as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRP as specified in clause 5.5.5 of TS 36.331 [15] |
| Reference Signal Received Quality | OAM | The per UE measurement of the received quality in a network cell, including SS-RSRQ, CSI-RSRQ as specified in clause 5.5 of TS 38.331 [14] and E-UTRA RSRQ as specified in clause 5.5.5 of TS 36.331 [15] |
| Signal-to-noise and interference ratio | OAM | The per UE measurement of the received signal to noise and interference ratio in a network cell, including SS-SINR, CSI-SINR, E-UTRA RS-SINR, as specified in clause 5.1 of TS 38.215 [12] |
| The mapping information between cell ID and frequency | OAM | The mapping information between cell ID and frequency, as specified in clause 4.3.5 of TS 28.541 [22]. |
| Cell Energy Saving State | OAM | List of the cells which are within the area of interest and are in energy saving state, as specified in clauses 3.1 and 6.2 of TS 28.310 [24]. |

Editor's note:	How to get the mapping information between cell ID and frequency is FFS and depends on OAM further feedback, i.e. whether the mapping information can be statically configured in NWDAF by operator or dynamically retrieved from OAM.

NWDAF subscribes the network data from OAM in the Table 6.4.2-3 by using the services provided by OAM as described in clause 6.2.3.

Table 6.4.2-4: UE level Network Data from 5G NF related to the Service Experience

| Information | Source | Description |
| --- | --- | --- |
| Timestamp | 5GC NF | A time stamp associated with the collected information. |
| Location | AMF | The UE location information, e.g. cell ID or TAI. |
| UE ID | AMF | (list of) SUPI(s). |
| RAT Type |  | The RAT type the UE camps on. |

Editor's note:	How to retrieve RAT Type per UE from 5GC is FFS.

The Event Filters for the service data collection from SMF, AMF and AF are defined in TS 23.502 [3].

The timestamps are provided by each NF to allow correlation of QoS and traffic KPIs. The clock reference is able to know the accuracy of the time and correlate the time series of the data retrieved from each NF.

------------------------------------------------------------END OF CHANGES--------------------------------------------------------