SA WG2 Meeting #143e	S2-2102870r02

# Feb 24th – March 9th, 2021 ; Elbonia              		 	(revision of S2-210)

Source: 	Nokia, Nokia Shanghai Bell

Title: 	 ECS Address Configuration

Document for: 	Agreement (P-CR)

Agenda Item: 	8.3

Work Item / Release:	eEDGE_5GC / Rel-17

Abstract of the contribution: ECS Address Configuration in UDM

# 1 Discussion

There is a companion 23.502 CR on this

# 2 Proposal

It is proposed to update TS 23.548 as follows

FIRST CHANGE

### 6.5.2	ECS Address Provisioning

#### 6.5.2.1	ECS Address Provisioning by SMF to the UE

If the UE hosts an EEC and supports transferring the ECS address received from the 5GC to the EEC, the UE indicates in the PCO at PDU Session establishment or modification that it supports the ability to receive ECS address(es) via NAS and to transfer the ECS Address(es) to the EEC(s).

The ECS Address Information consists of one or more FQDN(s) and/or IP Address(es) of Edge Configuration Server(s). As described in clause 4.3.2 in TS 23.502 [3], if the UE supports the ability to receive ECS address(es) via NAS and to transfer the ECS Address(es) to the EEC(s), the UE may receive ECS Address Information from the SMF via PCO during PDU Session Establishment and/or during PDU Session modification procedures. The SMF may derive the ECS Address Information based on local configuration, the UE's location, and/or UE subscription information. The SMF may decide to send updated ECS Address Information to the UE based on locally configured policy, updated UE subscription information, or a change of UE location. The PDU Session Modification procedure is used to send updated ECS Address Information to the UE as described in clause 4.3.3 in TS 23.502 [3].

NOTE 1:	In home routed sessions, the ECS Address Information comes from the H-SMF.

NOTE 2:	The SMF does not derive the ECS Address Configuration Information based on the UE's location if the ECS Address Information comes from subscription information.

#### 6.5.2.2	ECS Address Provisioning by a 3rd Party AF

As described in TS 23.558 [5], the Edge Configuration Server can be deployed in the MNO domain or can be deployed in a 3rd party domain by a service provider. If the ECS is deployed in a 3rd party domain by a service provider, a 3rd party AF can use Nnef_ParameterProvision to provide, update, or delete ECS Address Information .

When the AF uses Nnef_ParameterProvision to send a new ECS Address Information to the UDM for a group of UE(s) or for all UE(s), the UDM may notify the SMF of the updated ECS Address Information and the new ECS Address Information will be sent to the UE in a PDU Session Modification procedure. The mechanisms to propagate a change of ECS Address Information from the AF to the UE are further documented in clause 6.5.2.1 and in TS 23.502 [3], clause 4.15.6.2 and 4.15.6.3d.

.

NEXT CHANGE (2)

End of changes