3GPP TSG-SA2 Meeting # 144E (e-meeting)	S2-2102897

# Elbonia, April 12 – 16, 2021									  (based on some update of S2-2102169?)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.401 | CR | 3638 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Function Description for Multi-USIM devices -TAU rate limit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone (based on tdoc from Ericsson, LG Electronics, Huawei, China Telecom, Charter, MediaTek, Intel, Vodafone, Spreadtrum, Sony, Apple, Samsung, Nokia, Nokia Shanghai Bell) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | MUSIM |  |  |  |  |  | Date: |  |  | 2021-03-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | With a non-synchronised PLMN (with regard to 1-10 ms level of time sync) it is possible that a MUSIM UE can determine that its “Alternative IMSI” needs to be changed after cell reselection. However, to limit load on the network (and also to save UE battery), it is useful to ensure that frequent TAUs are not performed just to avoid paging collisions. It is proposed that stage 3 specifies the actual rate limit, but, allowing one TAU per “pre-Release 10 maximum Periodic TAU time period” seems like a reasonable value. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Limit on too frequent Paging Occasion collision avoidance TAUs is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | High MME processing load, excessive radio signalling, and less UE battery life. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Yellow highlighted text in 4.3.x.5(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR …CR … |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

****************** START CHANGE – this tdoc ONLY proposes the yellow highlighted text ***************

### 4.3.x	Support for Multi-USIM UE

#### 4.3.x.1	General

A network and a UE may support one or more of the following enhancements for Multi-USIM UE operation:

-	Connection release, as described in clause 4.3.x.2

-	Paging cause indication for voice service, as described in clause 4.3.x.3

-	Reject paging request, as described in clause 4.3.x.4

-	Paging timing control, as described in clause 4.3.x.5.

In the Attach procedure (as specified in clause 5.3.2.1), or in the Tracking Area Update procedure (as specified in clause 5.3.3), a Multi-USIM UE may include the Multi-USIM Mode Indication to the MME if it has more than one USIM active and intends to use Multi-USIM specific features. Based on the received Multi-USIM Mode Indication from the UE, the MME shall indicate to the UE the Multi-USIM features supported by network. The MME shall also indicate whether the network supports Paging Restriction as described in clause 4.3.x.6 .

#### 4.3.x.2	Connection release

A Multi-USIM UE may request to be released to CM-IDLE state for a USIM due to activity on another USIM.

The UE indicates that it requests to be released to CM-IDLE state for the USIM by initiating the Service Request procedure (using an Extended Service Request message) , or, a Tracking Area Update procedure ( if the UE needs to perform Tracking Area Update at the same time with this network), including a Release Indication.  If supported by the UE, and supported by the network, the UE may also provide, only together with the Release Indication, a Paging Restriction Information, as specified in clause 4.3.x.6, which requests the network to restrict paging. The Paging Restriction Information from the UE is stored in the UE context in the MME. If no Paging Restriction Information is provided in the Service Request or the Tracking Area Update procedure, any stored Paging Restriction Information in the UE context is removed.

When the UE initiates for the USIM a Service Request procedure or Tracking Area Update procedure without providing a Release Indication, the network removes any stored Paging Restriction Information.

#### 4.3.x.3	Paging cause indication for voice service

Besides the CN Domain indication in the Paging message, the UE and the network may support an additional indication of voice service (MMTel voice) in the Paging message when CN Domain is set to PS. The MME determines whether the paging relates to a voice service based on the Paging Policy Indicator as specified in clause 4.9 for MMTel voice.

Editor's note:	It is FFS if further parameter is needed for voice service determination.

#### 4.3.x.4	Rejection of paging request

Based on activity ongoing on another USIM. a Multi-USIM UE may respond to a page for a USIM with an indication to that USIM’s MME that the UE does not accept the paging and requests to be released to CM-IDLE state,

The Multi-USIM UE sends a Service Request to the paging network including the above indication as the response to the paging unless it is unable to do so, e.g. due to UE implementation constraints. In addition to the Rejection of paging indication, the UE may include Paging Restriction Information as specified in 4.3.x.6 in the Service Request, if supported by UE and if supported by the network.

#### 4.3.x.5	Paging timing control

To avoidrepetitive paging occasion collisions and to guarantee the paging reception for different USIMs, the UE may request to change, for at least one USIM, the IMSI value that is used for determination of paging occasions (as specified in TS 36.304[34]) to be the value of an Alternative IMSI. This is achieved by the UE providing a Requested Alternative IMSI Offset value during the Attach procedure or Tracking Area Update procedure for this USIM.  Upon reception of a Requested Alternative IMSI Offset value from UE in Attach Request or Tracking Area Update Request, a supporting MME provides an Accepted Alternative IMSI offset value to the UE in the Attach Accept or Tracking Area Update Accept message to provide the accepted value. The Accepted Alternative IMSI offset value is stored in the UE context in the MME, and it may differ from the one initially provided by the UE.  If the UE does not provide any Requested Alternative IMSI Offset value in Attach Request or Tracking Area Request, the MME removes any stored value in the UE context. The UE and MME use the Accepted IMSI Offset value to calculate the Alternative IMSI value that is determined based on UE’s IMSI as follows:

Alternative IMSI = [MCC] [MNC] [(MSIN value + Accepted Alternative IMSI Offset) mod (MSIN address space)]

where: the MCC, MNC and MSIN value are as per the UE’s IMSI.

The Alternative IMSI is used instead of the IMSI for:

-	determination of paging occasions as specified in TS 36.304 [34] in the UE, and

-	to compute the the UE Identity Index information the MME sends to the RAN (see TS 36.413 [36]) for the RAN to derive the paging occasions as per TS 36.304[34].

NOTE:	Stage-3 groups will define, if required, the handling to avoid excessive signalling load from UE due to this procedure.

#### 4.3.x.6	Paging Restriction

The UE and the network may support Paging Restriction. The UE, if the MME indicates that the network supports Paging Restriction, may indicate Paging Restriction Information in the Service Request or the Tracking Area Update procedures as specified in clause 4.3.x.2 and 4.3.x.4. The Paging Restriction Information may indicate any of the following:

a) 	all paging is restricted, or

b)	all paging is restricted, except paging for voice service (MMTel voice or CS domain voice), or

c)	all paging is restricted, except for certain PDN Connection(s), or

d)	all paging is restricted, except for certain PDN Connection(s) and voice service (MMTel voice or CS domain voice).

NOTE:	The UE expects not to be paged for any purpose in case a). The UE expects to be paged only for voice service in case b). The UE expects to be paged only for certain PDN Connection(s) in case c). The UE expects be paged for voice service and certain PDN Connection(s) in case d). When the UE requests Paging restrictions, the Extended Service Request message is used in the Service Request procedure.

****************** END CHANGES ***************