3GPP TSG-WG SA2 Meeting #145 e-meeting 	S2-2103787

Online, May 17 – 28, 2021

Source:	Panasonic Corporation, DENSO CORPORATION

Title:	Discussion of UE Authorization of 5MBS Session

Document for:	Agreement

Agenda Item:	8.9

Work Item / Release:	5MBS / Rel-17

Abstract: This discussion paper analyses the alignment of UE authorization for 5MBS between TS23.247 clause 7.4 and evaluation conclusion of key Issue 3 in TR23.757.

# 1. Introduction/Discussion

In Sect 7.4 of TR23.757, the solutions for the authorization of UE to join an MBS session were evaluated and some recommendations were made.  Particularly, the following level of authorization were identified:

A)	No authorization in 5GC;

B)	Network authorization on 5MBS feature level;

C)	MB service level authorization.

These were attempted to be captured in clause 6.1.2 of TS23.247.  However, it is the opinion of the co-signed companies that the current text was inadequate.

Firstly, the level of “no authorization” can be implicitly deduced from the other two levels.

Secondly, “authorization at a feature level” is currently captured as “Whether a UE is authorized to receive any Multicast MBS session data at all”.  Comparing this with the extracted text from Sect 7.4 of TR23.757:

If the authorization is performed in 5GS on 5MBS feature level, this can be achieved by including a new authorization parameter for 5MBS in the subscription data, so that the UE joining to any MBS service can be rejected if the UE is not authorized to use 5MBS feature

We understand that the feature level authorization has already been technically endorsed as S2-2103074 in SA2 #144e.

Thirdly, “MB service level authorization” is sufficiently captured in the current text of 6.1.2.  However, we further note the following two issues:

- the Editor’s Note uses the phrase “ … multicast services performed by a Content Provider is FFS.” Which inappropriately introduced an undefined term “Content Provider”.  We propose that it be changed to the more appropriate “AF of the content provider”.

- The ending clause in section 6.1.2 refers to clause 7.2.4, which we believe should instead be 7.2.1.

Furthermore, we propose to standardize the use of the term “Multicast MBS Session” in 6.1.2.

# 2. Proposal

It is proposed to approve the changes as outlined in S2-2103786.