3GPP TSG-SA2 Meeting #145E	S2-2103813

# Electronic,  May 17 – May 28, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.288 | CR | 0334 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on output parameters in OSE |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNA |  |  |  |  |  | Date: |  |  | 2021-05-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current description of the putput of the Observed Service Experience has been extended, now it is not clear how the information is provided by NWDAF |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarification in the output statistics and prediction such as: - S-NSSAI is provided as part of either the application service experience or the slice service experience so it belongs to both. - The list of SUPIs and its related context is also provided by NWDAF in a list of parameters appliable for a list of SUPIs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Unclear and misleading output of OSE |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * 1st Change * * *

### 6.4.3	Output Analytics

The NWDAF services as defined in the clause 7.2 and 7.3 are used to expose the analytics.

-	Service Experience statistics information is defined in Table 6.4.3-1.

-	Service Experience predictions information is defined in Table 6.4.3-2.

Table 6.4.3-1: Service Experience statistics

| Information | Description |
| --- | --- |
| Slice instance service experiences (0…max) | observed service experience information for each Network Slice instance (if applicable). |
| > S-NSSAI | Identifies the Newtork Slice |
| > NSI ID | Identifies the Network Slice instance within the Network Slice (optional). |
| > Network Slice instance service experience | Service experience across Applications on a Network Slice instance (NOTE 1) |
| > SUPI list (0..SUPImax) | List of SUPI(s) registered in the sliceand for which the slice instance service experience applies. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area where the Network Slice service experience analytics applies. |
| > Validity period | Validity period for the Network Slice service experience analytics as defined in clause 6.1.3. |
| Application service experiences (0..max) | observed service experience information for each Application. |
|  | . |
| > Application ID | Application Identifer. |
| > ServiceType | Type of Service, e.g. voice, video, other. |
| > SUPI list (0..SUPImax) | List of SUPI(s) consuming these applications and for which the application service experience applies.(optional) |
| >>For each SUPI | UE context information for each SUPI. |
| >>UE location | the UE location information (e.g. TAI , gNB ID, etc) when the application is delivered to the UE. |
| >>UPF Info | Indicating UPF serving the UE when accessing the application via DNN,S-NSSAI. |
| >>DNAI | Indicating which DNAI is used to access this application |
| >>DNN | DNN used to access this application |
| >> S-NSSAI | S-NSSAI used to access this application |
| >>Application Server Instance Address | Identifies the Application Server Instance (IP address of the Application Server) or FQDN of Application Server. |
| >> Service Experience | Service Experience. |
| >> Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs).(optional) |
| > Spatial validity | Area where the Application service experience analytics applies. |
| > Validity period | Validity period for the Application service experience analytics as defined in clause 6.1.3. |
| NOTE 1: The Service experience is measured over the Analytics target period. |  |

Table 6.4.3-2: Service Experience predictions

| Information | Description |
| --- | --- |
| Network Slice instance service experiences (0…max) | List of observed service experience information for each Network Slice instance. |
| > S-NSSAI | Identifies the Network Slice |
| > NSI ID | Identifies the Network Slice instance within the Network Slice (optional). |
| > Network Slice instance service experience | Service experience across Applications on a Network Slice instance |
| > SUPI list (0..SUPImax) | List of SUPI(s) registered in the sliceand for which the slice instance service experience applies. |
| > Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area where the Network Slice service experience analytics applies. |
| > Validity period | Validity period for the Network Slice service experience analytics as defined in clause 6.1.3. |
| > Probability assertion | Confidence of this prediction. |
| Application service experiences (0..max) | List of predicted service experience information for each Application. |
| > Application ID | Application Identifier. |
| > Service Type | Type of Service, e.g. on voice, video, other. |
| > SUPI list (0..SUPImax) | List of SUPI(s) consuming these applications and for which the application service experience applies.(optional) |
| >>For each SUPI | UE context information for each SUPI. |
| >>UE location | the UE location information (e.g. TAI , gNB ID, etc) when the application is delivered to the UE. |
| >>UPF Info | Indicating UPF serving the UE when accessing the application via DNN,S-NSSAI. |
| >>DNAI | Indicating which DNAI is used to access this application |
| >>DNN | DNN used to access this application |
| >< S-NSSAI | S-NSSAI used to access this application |
| >>Application Server Instance Address | Identifies the Application Server Instance (IP address of the Application Server) or FQDN of Application Server. |
| >> Service Experience | Service Experience |
| >> Ratio | Estimated percentage of UEs with similar service experience (in the group, or among all UEs). |
| > Spatial validity | Area where the Application service experience analytics applies. |
| > Validity period | Validity period for the Application service experience analytics as defined in clause 6.1.3. |
| > Probability assertion | Confidence of this prediction. |
| NOTE 1: The Service experience is measured over the Analytics target period. |  |

NOTE 1:	The NSI ID is an optional parameter, if not provided the Network Slice instance service experience indicates the service experience for the S-NSSAI.

NOTE 3:	If the consumer NF is an AF, the item "DNN" and "UPF info" shall not be included, and the NEF is responsible for translation of SUPI to GPSI, internal group identifiers to external ones, and UE location to geographical area, by querying UDM, prior to contacting the AF.

The number of Service Experiences and SUPIs are limited respectively by the maximum number of objects and the Maximum number of SUPIs provided as part of Analytics Reporting Information by the NWDAF Service Consumer.

* * *End of Changes * * *