3GPP TSG-SA WG2 Meeting #145E 	S2-2103861

Elbonia, 17 - 28 May, 2021

Source:	Ericsson, Huawei

Title:	ECS Address provision clarification

Document for:	Agreement

Agenda Item: 	8.3

Work Item / Release:	eEDGE_5GC /Rel-17

Abstract of the contribution: This contribution simplifies and clarifies the ECS address provision.

# Discussion

It is proposed to reduce options and simplify the ECS address configuration information provisioning while retaining enough flexibility. The option to configure the ECS address configuration information data in the SMF and the option to Provision ECS address configuration data as UE subscription data are essentially the same and the SMF configuration option can be removed without loss of functionality.

# Proposal

It is proposed to clarify how the ECS address configuration is provisioned and remove the SMF configuration option.

**************************** 1st Change ***************************

### 6.5.2	ECS Address Provisioning

#### 6.5.2.1	General

Editor's note:	This clause describes here the procedure agreed in conclusions in 9.1.3.

If the UE hosts an EEC and supports transferring the ECS address received from the 5GC to the EEC, the UE indicates in the PCO at PDU Session establishment or modification that it supports the ability to receive ECS address(es) via NAS and to transfer the ECS Address(es) to the EEC(s).

The ECS Address Configuration Information consists of one or more FQDN(s) and/or IP Address(es) of Edge Configuration Server(s), ECS Provider ID and ECS ID. As described in clause 4.3.2 in TS 23.502 [3], if the UE supports the ability to receive ECS Address Configuration Information via NAS and to transfer the ECS Address(es) to the EEC(s), the UE may receive ECS Address Configuration Information from the SMF via PCO during PDU Session Establishment and/or during PDU Session Modification procedures. The SMF derives the ECS Address Configuration Information based on UE subscription information from UDM, the UE's location, ECS Provider ID.. The SMF may decide to send updated ECS Address Configuration Information to the UE based on locally configured policy, updated UE subscription information, or a change of UE location. The PDU Session Modification procedure is used to send updated ECS Address Configuration Information to the UE as described in clause 4.3.3 in TS 23.502 [3].

NOTE 1:	In home routed sessions, the ECS Address Configuration Information comes from the H-SMF. The traffic to the indicated Edge Configuration Server(s) may be transmitted via a PDU session with local breakout.

NOTE 2:	Although the Service Provisioning procedure with the ECS may take place over a HR session, the UE needs to establish an LBO PDU Session to access the EES(s) and EAS(s) in VPLMN. The Service Provisioning procedure is described in TS 23.558 [5].

#### 6.5.2.2	ECS Address Provisioning by a 3rd Party AF

As described in TS 23.558 [5], the Edge Configuration Server can be deployed in a 3rd party domain by a service provider. If the ECS is deployed in a 3rd party domain by a service provider, a 3rd party AF can use Nnef_ParameterProvision to provide, update, or delete ECS Address Configuration Information for a UE (See TS 23.502 [3], clause 4.15.6.2).

When the AF uses Nnef_ParameterProvision to send a new ECS Address Configuration Information to the UDM for a UE (e.g. because on Application layer activity, change of UE location, etc.), the UDM may notify the SMF of the updated ECS Address Configuration Information and the new ECS Address Configuration Information will be sent to the UE in a PDU Session Modification procedure.

Editor's note:	It is FFS whether the 3rd party AF would know the GPSI; the possibility to configure the ECS address for all UE in one operation should be considered.

Editor's note:	Potential signalling overload is FFS When the AF uses Nnef_ParameterProvision to send a new ECS Address Information to the UDM for many UE.

#### 6.5.2.3	ECS Address Provisioning by MNO

The ECS Address Configuration Information can be provisioned by the MNO subscription provisioning in UDM.

********************************* End Change ****************************