SA WG2 Meeting #145E (e-meeting)	S2-2104072

May 17 – May 28, 2021, Elbonia	(revision of S2-200xxxx)

Source:	LG Electronics

Title:	Discussion on how to send a PMF message over a target QoS Flow

Document for:	Approval

Agenda Item:	8.6 Access Traffic Steering, Switch and Splitting support in the 5G system architecture Phase 2

Work Item / Release:	ATSSS_Ph2 / Rel-17

Abstract of the contribution: This paper discusses how to send a PMF message over a target QoS Flow.

Discussion

Flowing table summarizes two options in the table and compares impacts. The red text shows additional impacts while blue text is aligned current specification.

|  | Multiple PMF address per each QoS Flows | QFI in PMF message |
| --- | --- | --- |
| 1 | The UPF allocates one UDP port per access type and per QoS Flow | The UPF allocated one UDP port per access (as in Rel-16). |
| 2 | The UE receives the association between UDP ports and QoS Flows, e.g.: UDP port for non-3GPP access + QoS Flow 1 = A1 UDP port for non-3GPP access + QoS Flow 2 = A2 | The UE receives QoS Flow lists |
| 3 | The UE also receives QoS rules for PMF messages, which map the dest IP address + dest UDP port to a QoS Flow (QFI). | No need to provide additional QoS rules for PMF message |
| 4 | The UE-PMF sends a PMF message to dest. UDP A1, if this PMF message should be sent over QoS Flow 1. NOTE: No changes in PMF message itself (See #7) | The UE-PMF sends a PMF message which contains the target QFI in the header of the PMF message. NOTE: Need to add new IE in PMF message (See #7) |
| 5 | PMF message is sent over target QoS Flow based on QoS rules. | PMF message is sent over target QoS Flow based on QFI in the PMF message (and ignores QoS rules) |
| 6 | For determining the QoS Flow of a received PMF message or of a data packet, the UE checks the QoS rules. | For determining the QoS flow of a PMF message, the UE inspects the header of the PMF message. For determining the QoS flow of a data packet (anything that is not PMF), the UE checks the QoS rules. Different handling for PMF message and data packet to detect received QoS flow. |
| 7 | For Rel-16 UE, the UPF behaviour is the same. No need to differentiate whether a UE is Rel-16 or Rel-17 UE. | For Rel-16 UE, the UE does not indicate QFI in the PMF message. So the UPF needs to know whether a UE is Rel-16 UE or Rel-17 UE to differentiate whether missing QFI is caused by protocol error or not. UPF need to know whether a UE is Rel-16 or Rel-17 UE and operate differently. |
| 8 | For MA PDU over 3GPP access in EPC and non-3GPP access in 5GC, Need to provide EPS bearer contexts for PMF messages (This is normal EPS interworking behaviour) | For MA PDU over 3GPP access in EPC and non-3GPP access in 5GC, EPC modem should be updated to support sending a PMF message based on QFI in the PMF message while ignoring EPS bearer contexts. PMF message needs to include EPS bearer ID. |

Proposal

Based on comparison in the table in discussion part, it is proposed to use multiple PMF addresses to send a PMF message over a target QoS Flow.