3GPP SA2 Meeting #145e	S2-2104155

# Elbonia, May 17th – 28th, 2021                                                              (revision of S2-10xxxx)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.502 | CR | 2780 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update on ECS Address Configuration Information Parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eEDGE_5GC |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | C |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In TS 23.502, clause 4.15.6.3.d describes that “The ECS Address Configuration Information is associated with a DNN and S-NSSAI included in the message from UDM”. It is however not clarified how the ECS Address Configuration Information can be associated with a DNN and S-NSSAI in UDM. Additionally, since it was agreed to include ECS ID and ECS Provider ID in ECS Address Configuration information as per TS 23.548, clause 4.15.6.3.d in TS 23.502 needs to be revised accordingly. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This CR proposes that an AF includes ECS address along with ECS ID, ECS provider ID, DNN, and S-NSSAI in ECS Address Configuration Information. If DNN and S-NSSAI are included in the message from an ECS acting as an AF, the UDM stores the ECS Address Configuration information in association with them. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | It is not clarified how the ECS Address Configuration Information is associated with DNN(s) and S-NSSAI(s). Clause 4.15.6.3.d of TS 23.502 is not aligned with clause 6.5.2 for ECS Address Provisioning in TS 23.548. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.15.6.3.d |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 4.15.6.3d	ECS Address Configuration Information Parameters

ECS Address Configuration Information parameters that an AF may provide are described in Table 4.15.6.3d-1. An AF includes in ECS Address Configuration Information ECS Address, ECS ID, and ECS Provider ID optionally along with DNN(s) and S-NSSAI(s). ECS Address Configuration Information is stored as Session Management Subscription data. If DNN(s) and S-NSSAI(s) are included, the ECS Address Configuration Information is stored by UDM in association with the DNN and the S-NSSAI. The ECS Address Configuration Information is associated with a DNN and S-NSSAI included in the message from UDM.

Table 4.15.6.3d-1: Description of ECS Address Configuration Information parameters

| Parameters | Description |
| --- | --- |
| ECS Address | Consists of one or more FQDN(s) and/or IP Address(es) of Edge Configuration Server(s). |
| ECS Provider ID | Identifier of edge computing service provider that provides the Edge Configuration Server |
| ECS ID | Identifier of Edge Configuration Server |
| DNN(s) | DNN(s) for the Edge Configuration Server |
| S-NSSAI(s) | S-NSSAI(s) for the Edge Configuration Server |

Editor's note:	It is FFS for multiple ECS address configuration information.

* * * End of Change * * * *