SA WG2 Meeting #S2-145E	S2-2104261

# 17 – 28 May, 2021, Electronic

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.501 | CR | 2909 | rev | - | Current version: | 17.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Support multiple NSACFs for one S-NSSAI during UE mobility |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | China Mobile |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA WG2 |  |  |  |  |  |  |  |  |  |
| Work item code: | eNS_Ph2 |  |  |  |  |  | Date: |  |  | 2021-05-10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Currently the existing specification does not support multiple NSACFs for one S-NSSAI during registration or handover, which is not possible for the real network deployment because at least for load balance or service backup, there will be at least two or more NSACFs which can supporting one S-NSSAI, and the network deployment may not guarantee any AMF can access to one NSACF during UE mobility. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Support multiple NSACFs for one S-NSSAI during UE mobility |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 5GC cannot support the distributed NSACFs feature during UE mobility |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.15.11.1; 6.3.22 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<<< First change >>>

5.15.11.1	Maximum number of UEs per network slice admission control

The NSACF controls (i.e. increase or decrease) the current number of UEs registered for a network slice so that it does not exceed the maximum number of UEs allowed to register with that network slice. The NSACF also maintains a list of UE IDs registered with a network slice that is subject to NSAC. When the current number of UEs registered with a network slice is to be increased, the NSACF first checks whether the UE Identity is already in the list of UEs registered with that network slice and if not, it checks whether the maximum number of UEs per network slice for that network slice has already been reached.

The AMF triggers a request to NSACF for maximum number of UEs per network slice admission control when the UE's registration status for a network slice subject to NSAC may change, i.e. during the UE Registration procedure in clause 4.2.2.2.2 in TS 23.502 [3], UE Deregistration procedure in clause 4.2.2.3 in TS 23.502 [3], Network Slice-Specific Authentication and Authorisation procedure in clause 4.2.9.2 in TS 23.502 [3], AAA Server triggered Network Slice-Specific Re-authentication and Re-authorization procedure in clause 4.2.9.3 in TS 23.502 [3], and AAA Server triggered Slice-Specific Authorization Revocation in clause 4.2.9.4 in TS 23.502 [3].

During UE mobility, the Mobility Registration or Handover can trigger the serving AMF change. When there are multiple NSACF instances for one S-NSSAI, the source AMF provides NSACF instance(s) ID corresponding to Allowed NSSAI to target AMF, and target AMF decides whether previous NSACF instance(s) can be used and may select new NSACF instance(s) if necessary, as described in clause 6.3.22. If target AMF reselect a new NSACF instance for the S-NSSAI of the Allowed NSSAI, the old AMF should be notified about this change and triggers an update procedure towards the old NSACF instance serving the S-NSSAI to indicate that the UE should be removed, and the target AMF triggers an update procedure towards the new NSACF instance to indicate the UE should be added. If target AMF decides to use the same NSACF instance, the update procedure is not needed.

<<< Second change >>>

6.3.22	NSACF discovery and selection

Multiple instances of NSACF may be deployed in a network for one S-NSSAI. The maximum number of the UE/PDU session are configured by OAM.

The NF consumers shall utilise the NRF to discover NSACF instance(s) unless NSACF information is available by other means, e.g. locally configured in NF consumers.

If the NSACF NF consumer is the AMF, the NSACF selection function in the AMF selects an NSACF instance based on the available NSACF instances, which are obtained from the NRF or locally configured in the AMF. The AMF can decide whether the NSACF instance which received from other AMFs can be used based on NRF query or local configuration.

The following factors may be considered by the NF consumer for NSACF selection:

-	S-NSSAI(s).

-	NSACF Serving Area information

Editor's note:	The definition of NSACF Serving Area information is FFS.

-	NSACF service capabilities:

-	Support monitoring and controlling the number of registered UEs per network slice for the network slice that is subject to NSAC.

-	Support monitoring and controlling the number of established PDU Sessions per network slice for the network slice that is subject to NSAC.

In the case of delegated discovery and selection in SCP, the NSACF NF consumer shall send all available factors to the SCP.

<<< End of changes >>>