3GPP TSG-WG SA2 Meeting #145E e-meeting	S2-2104263

Elbonia, May 17 – 28, 2021	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon, OPPO

Title:	Additions and updates to UE-UE protocol stacks

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	5G_ProSe / Rel-17

Abstract: This contribution proposes to add the user plane protocol stack for NR PC5.

# 1. Introduction/Discussion

This contribution proposes to add the user plane protocol stack for NR PC5 based on the TR conclusion, and update the wrong clause reference for ProSe Communication via Layer-3/Layer-2 UE-to-Network Relay.

# 2. Text Proposal

It is proposed to capture the following changes vs. TS 23.304.

# * * * * First change * * * *

##### 6.1.1.2.2	PC5 Signalling Protocol

The PC5 Signalling Protocol stack specified in clause 6.1.2 of TS 23.287 [2] is used. The protocol used for the control plane signalling over the PC5 reference point for the secure layer-2 link is specified in clauses 6.4.3, 6.5.1 and 6.5.2.

# * * * * Second change * * * *

#### 6.1.2.1	UE - UE

Figure 6.1.2.1-1 depicts a user plane for NR PC5 reference point, i.e. PC5 User Plane Protocol stack.

Legend:

-	PC5-U: The SDAP/PDCP/RLC/MAC/PHY functionality is specified in TS 38.300 [11].

# Figure 6.1.2.1-1: User Plane for NR PC5 reference point

IP, Ethernet and Unstructured PDCP SDU types are supported. For IP PDCP SDU type, both IPv4 and IPv6 are supported.

The packets from ProSe application layer are handled by the ProSe layer before transmitting them to the AS layer, e.g. ProSe layer maps the IP, Ethernet and Unstructured packets to PC5 QoS Flow and marks the corresponding PFI.

# * * * * End of changes * * * *