3GPP TSG-WG SA2 Meeting #145E e-meeting	S2-2104265

Elbonia, May 17 – 28, 2021	(revision of S2-200xxxx)

Source:	Huawei, HiSilicon

Title:	Update on ProSe Subscription

Document for:	Approval

Agenda Item:	8.8

Work Item / Release:	5G_ProSe / Rel-17

Abstract: This contribution proposes to add the PLMNs list authorised to relay into ProSe Subscription.

# 1. Introduction/Discussion

This contribution proposes to add the PLMNs list authorised to relay into ProSe Subscription.

# 2. Text Proposal

It is proposed to capture the following changes vs. TS 23.304.

# * * * * First change * * * *

## 5.8	Subscription to ProSe

The subscription information in the UDM contains information to give the user permission to use 5G ProSe.

At any time, the operator can amend or remove the ProSe UE subscription rights from subscription information in the UDM, or to revoke the user's permission to use 5G ProSe.

The following subscription information is defined for 5G ProSe:

-	subscription for open ProSe Direct Discovery for NR PC5:

-	open ProSe Direct Discovery Model A.

-	subscription for restricted ProSe Direct Discovery for NR PC5:

-	restricted ProSe Direct Discovery Model A;

-	restricted ProSe Direct Discovery Model A with application-controlled extension;

-	restricted ProSe Direct Discovery Model A with "on demand" announcing;

-	restricted ProSe Direct Discovery Model B.

-	subscription for Broadcast, Groupcast and Unicast mode ProSe Direct Communication for NR PC5.

-	subscription for ProSe UE acting as Layer-2 UE-to-Network Relay.

-	subscription for ProSe UE acting as Layer-3 UE-to-Network Relay.

-	subscription for Layer-2 Remote UE access via Layer-2 5G ProSe UE-to-Network Relay

-	subscription for Layer-3 Remote UE access via Layer-3 ProSe UE-to-Network Relay

-	UE-PC5-AMBR for NR PC5.

-	PC5 QoS parameters as defined in clause 5.7.1 used by NG-RAN.

-	the list of the PLMNs authorized for 5G ProSe services, including:

-	the list of the PLMNs where the UE is authorised for open Direct Discovery Model A, i.e. to announce or monitor or both.

-	the list of the PLMNs where the UE is authorised for restricted ProSe Direct Discovery Model A, i.e. to announce or monitor or both.

-	the list of the PLMNs where the UE is authorised for restricted ProSe Direct Discovery Model B, i.e. to perform Discoverer operation or Discoveree operation or both.

-	the list of the PLMNs where the UE is authorised to perform Broadcast, Groupcast and Unicast mode ProSe Direct Communication for NR PC5.

-	the list of the PLMNs where the UE is authorised to relay traffic for a Remote UE.

# * * * * End of changes * * * *